| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0248r1 |
| --- | --- |

| Title: | Support of Field Coding for Signalling of Chroma Phase for Upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa, Tokyo 141-8610 Japan Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com +1-408-352-4086 Cheung.Auyeung@am.sony.com |
| Source: | Sony Corp. / Sony Electronics Inc. |  |  |

_____________________________

# Abstract

It is proposed by JCTVC-M0465 that chroma sampling position be transmitted for the purpose of up-sampling for spatial scalability. The proposed syntax is in sps_extension() and only one phase can be specified. However if field structure is applied, chroma phase for the top filed picture is different from the one for the bottom field picture within a sequence.

This contribution claims to modify the syntax to allow up to 2 phases for up-sampling to support field coding.

# Introduction / Problem Statement

As shown in Table 1.1 and 1.2, it is proposed by JCTVC-M0465 that signal sampling grid shift information and chroma sampling location be transmitted in SPS so an up-sampling filter with correct phase. With this syntax information on only one phase per sequence can be transmitted.

Table 1.1

| sps_extension ( ) { | Descriptor |
| --- | --- |
| sampling_grid_information() |  |
| } |  |

Table 1.2

| sampling_grid_information( ) { | Descriptor |
| --- | --- |
| phase_offset_present_flag | u(1) |
| if ( sampling_grid_info_present_flag ) { |  |
| horizontal_phase_offset16 | ue(v) |
| vertical_phase_offset16 | ue(v) |
| chroma_phase_x_flag | u(1) |
| chroma_phase_y | u(2) |
| } |  |

However if field coding is applied for the sequence, chroma phase for the top filed pictures is different from the one for the bottom field pictures within a sequence as shown in Fig 1.1.

Fig 1.1

# Solution

To solve the problem stated above, the following solutions are proposed to allow transmission of 2 phases within a sequence.

## Solution 1

In this solution sampling_grid_information() as proposed by [1] is transmitted not in sps_extension() but in pps_extension() so that it is allowed that different offsets are applied picture-by-picture.

Table 2.1.1

| pps_extension ( ) { | Descriptor |
| --- | --- |
| sampling_grid_information() |  |
| } |  |

## Solution 2

In this solution it is allowed to transmit more than one phase in sps_extension(), and in pps_extension() an index is transmitted to indicate which phase is applied for the current picture.

Table 2.2.1

| sampling_grid_information( ) { | Descriptor |
| --- | --- |
| num_phase_offset_minus1 | u(1) |
| for( phase_offset_idx=0; phase_offset_idx < =num_phase_offset_minus1; phase_offset_idx++ ) { |  |
| horizontal_phase_offset16[phase_offset_idx] | ue(v) |
| vertical_phase_offset16[phase_offset_idx] | ue(v) |
| chroma_phase_x_flag[phase_offset_idx] | u(1) |
| chroma_phase_y[phase_offset_idx] | u(2) |
| } |  |

num_phase_offset_minus1 plus 1 specifies the number of offsets signalled in sampling_grid_information() syntax. Its value shall be in the range of 0 to 1, inclusive. When its value is not specified, it is inferred to be 0.

Table 2.2.2

| pps_extension ( ) { | Descriptor |
| --- | --- |
| … |  |
| phase_offset_idx | u(1) |
| … |  |
| } |  |

phase_offset_idx indicates which phase specified in sampling_grid_information() is applied for the picture associated with active PPS. Its value shall be in the range of 0 to num_phase_offset_minus1 inclusive. When its value is not specified, it is inferred to be 0.

# Conclusion

It is proposed by JCTVC-M0465 that chroma sampling position be transmitted for the purpose of up-sampling for spatial scalability. The proposed syntax is in sps_extension() and only one phase can be specified per sequence. This proposal claims to modify syntax so that information on 2 phases can be transmitted within a sequence to support field coding. Solution 1 proposes that sampling_grid_information() as proposed by JCTVC-M0465 is transmitted not in sps_extension() but in pps_extension() so that it is allowed that different offsets are applied picture-by-picture. Solution 2 proposes that it is allowed to transmit more than one phase in sps_extension(), and in pps_extension() an index is transmitted to indicate which phase is applied for the current picture.

It is recommended that one of the proposed solutions be adopted.

# References

[1] “Signaling of Phase Offset in Up-sampling Process and Chroma Sampling Location”, JCTVC-M0465, April 2013

# Patent rights declaration(s)

Sony Corp. and Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).