| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0249 |
| --- | --- |

| Title: | Non-RCE3: Modified Palette Mode for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Joel Sole Rajan Joshi 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes a modified palette-based coding method. The modifications compared to the palette-based coding described in RCE3 Test 3.1 are: (1) “pixel mode” is removed and all the pixel values are converted to palette indices for encoding; (2) the possible error (from pixel values to palette indices) is encoded using the HEVC residue coding method; and (3) the palette index and the “run” are shared by all the 3 color components.

# Introduction

The coding of screen content plays an important role in applications such as remote desktop control and on-line collaboration. Compared to natural video content, screen content tends to have fewer distinct pixel values (discrete tone) and sharper object edges. A palette-based coding method (JCTVC-M0323) [1] was presented at the Incheon meeting and was included as test 3.1 in RCE3 for the Vienna meeting [2].

This contribution describes a modified method for palette-based coding. The modified method removes the “pixel mode” to simplify the design, so that all pixels are coded using their palette indices. Since the palette size is limited, residue coding of HEVC is used to code the possible error resulting from converting a pixel value to a palette index. Furthermore, the palette index and the “run” values are shared by all the color components to reduce overhead.

# Proposed method

The proposed palette-based coding is designed as a CU mode. The encoding process is as follows:

- Transmission of the palette: the palette size is first transmitted followed by the palette elements. The maximum palette size is set to 24.

- Transmission of pixel values: the pixels in the CU are encoded in a raster scan order. For each position, a flag is first transmitted to indicate whether the “run mode” or “copy above mode” is being used.

  - “Run mode”: In “run mode”, a palette index is first signaled followed by “palette_run” (e.g., M). No further information needs to be transmitted for the current position and the following M positions as they have the same palette index as signaled. The palette index (e.g., i) is shared by all three color components, which means that the reconstructed pixel values are (Y, U, V) = (paletteY[i], paletteU[i], paletteV[i]) (assuming the color space is YUV)

  - “Copy above mode”: In “copy above mode”, a value “copy_run” (e.g., N) is transmitted to indicate that for the following N positions (including the current one), the palette index is equal to the palette index of the one that is at the same location in the row above.

- Transmission of residue: the palette indices transmitted in Stage 2 are converted back to pixel values and used as the prediction. Residue information is transmitted using HEVC residue coding and is added to the prediction for the reconstruction.

# Experimental results

The proposed method is implemented on HM-10.1+RExt-3.0. Test conditions specified in [2] are used. Encoding time and decoding time is measured on a homogenous cluster.

## Lossless results

Table 1  Results of proposed method for lossless coding

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.1% | 31.7 | 31.7 | 0.0% | 49.8 | 49.7 | 0.0% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 21.1 | -38.2% | 100.4 | 203.9 | -31.7% | 381.6 | 763.6 | -28.0% |
| SC YUV 444 | 11.4 | 22.5 | -27.7% | 128.9 | 252.3 | -24.6% | 325.6 | 673.4 | -22.6% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 134% |  |  | 110% |  |  | 106% |  |  |
| Dec Time[%] | 82% |  |  | 99% |  |  | 105% |  |  |

## Lossy results

Table 2 Results of proposed method for lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -1.9% | -1.4% | -0.9% | -1.9% | -1.4% | -1.0% | -1.7% | -1.3% |
| Class B | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -41.1% | -40.1% | -40.4% | -41.4% | -40.8% | -41.1% | -41.6% | -41.5% | -41.6% |
| SC YUV 444 | -21.6% | -24.8% | -25.9% | -25.9% | -28.0% | -29.0% | -27.6% | -29.2% | -30.0% |
| RangeExt | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 123% |  |  | 123% |  |  | 124% |  |  |
| Dec Time[%] | 79% |  |  | 79% |  |  | 79% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -2.6% | -1.4% | -0.4% | -1.9% | -1.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -35.7% | -34.6% | -35.1% | -36.4% | -35.7% | -36.1% |  |  |  |
| SC YUV 444 | -20.3% | -24.1% | -25.2% | -23.0% | -25.7% | -26.8% |  |  |  |
| RangeExt | 0.2% | 0.4% | 0.4% | 0.1% | 0.2% | 0.2% |  |  |  |
| Enc Time[%] | 118% |  |  | 117% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -2.9% | -1.4% | -0.1% | -1.8% | -0.8% |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -30.9% | -30.1% | -30.4% | -33.1% | -32.4% | -32.8% |  |  |  |
| SC YUV 444 | -15.4% | -17.8% | -18.8% | -18.3% | -20.0% | -20.8% |  |  |  |
| RangeExt | 0.2% | 0.3% | 0.5% | 0.2% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 113% |  |  | 113% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

# Complexity Analysis

## Throughput

At the prediction stage, conversion between pixel values and palette index does not need adjacent samples. At the reconstruction stage, adding residue block and the prediction block is also position-wise operation. So in lossless coding, all the samples in the CU can be encoded or decoded in parallel.

In lossy coding, inverse transform may be performed for the residue block. In this case, the throughput is the same as regular HEVC mode.

## Number of operations

The prediction value is derived from the parsed index with a look-up table operation. The prediction is added with the residue for the decoding. So in the lossless case, the operation is one LUT and one addition for each position. In lossy coding, inverse transform and inverse quantization may be needed as in regular HEVC.

## Line buffers

The reconstruction of the pixel values does not depend on values of previous CU, so no line buffers of the reconstructed picture are necessary.

# Conclusions

This contribution presents a modified palette coding method. The modifications include removing pixel mode, using the HEVC residue coding module and sharing the received palette indices and run values for all 3 components.

# Draft texts

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if (palette_mode_flag) { |  |
| palette_size_minus1[x0][y0] | ae(v) |
| for (cIdx = 0; cIdx < 3; cIdx ++) |  |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][ cIdx][i] | ae(v) |
| scanPos = 0 |  |
| while (scanPos < nCbS * nCbS) { |  |
| run_mode_flag | ae(v) |
| if (run_mode_flag) |  |
| { |  |
| palette_index | ae(v) |
| } |  |
| run | ae(v) |
| runPos= 0 |  |
| while (runPos <= run){ |  |
| xC = scanPos% nCbS |  |
| yC = scanPos/ nCbS |  |
| if (run_mode_flag){ |  |
| paletteMap[ x0 ][ y0 ] [ xC ][ yC ] = palette_index |  |
| } |  |
| else{ |  |
| paletteMap[ x0 ][ y0 ] [ xC ][ yC ] = paletteMap[ x0 ][ y0 ] [ xC ][ yC -1 ] |  |
| } |  |
| runPos ++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| ….. |  |
| } |  |

## Semantics

palette_mode_flag[x0][ y0] equal to 1 specifies that the current coding unit is encoded using palette mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than palette mode.

When palette_mode_flag is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_size_minus1[x0][y0] plus 1 specifies the number of elements in the palette. palette_size_minus1[x0][y0] shall be the range of 0 to 23.

When palette_size_minus1 [x0][y0] is not present, it is inferred to be equal to 0.

palette[x0][y0][cIdx][i] specifies the i-th element in the palette for the color component cIdx.

When palette[x0][y0][cIdx][i] is not present, it is inferred to be equal to 0.

run_mode_flag equal to 1 specifies that the decoding is swtiched to “run mode” where one or more following locations have the same palette index. The run_mode_flag equal to 0 specifies that the decoding is switched to “copy above mode” where  one or more following locations have the same palette index as their corresponding location in the above row.

When run_mode_flag is not present, it is inferred to be equal to 0.

copy_above_flag equal to 1 specifies that the decoding is switched to “copy from above mode” where the decoded pixel value is equal to the pixel at the same location in the above row. copy_above_flag equal to 0 specifies that the decoding is switched to either “run mode” or “pixel mode”.

When copy_above_flag is not present, it is inferred to be equal to 0.

palette_index represents an index in the palette.

When palette_index is not present, it is inferred to be equal to 0.

run represents the number of consecutive locations with the same palette index ( “run_mode_flag = 1”) or the number of consecurive locations with the same palette index as the position in the above row.

##### Palette mode prediction process

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

–	the width and height of this coding block, nCbS and nCbS,

–	one (nCbS)x(nCbS) array paletteMap

–	a variable cIdx specifying colour component index.

–	an array palette[xB][yB][cIdx] of size (palette_size_minus1+1)

Outputs of this process are:

–	the (nPbW)x(nPbH) array predSamples of prediction sample values.

The variable nPbW and nPbH are derived as follows.

–	If cIdx is equal to 0, nPbW is set equal to nCbS and nPbH is set equal to nCbS.

–	Otherwise, nPbW is set equal to nCbS/SubWidthC and nPbH is set equal to nCbS/SubHeightC.

For each sample location ( x = 0..nPbW−1, y = 0..nPbH−1 ) inside the prediction sample array predSamples, the corresponding prediction sample value predSamples[ x, y ] is derived as follows:

- predSamples[ x, y ] = palette[xB][yB][cIdx][paletteMap[xP][yP]]

- The variables xP and yP are derived by

xP = x/ SubWidthC

yP = y/ SubHeightC

# References

- L. Guo, M. Karczewicz, and J. Sole. “Palette Mode for Screen Content Coding”, JCTVC-M0323, Incheon, Korea, Apr. 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, Korea, Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).