| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0250 |
| --- | --- |

| Title: | Non-SCE3.3: Modified Interlayer SAO with highpass processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, chihming.fu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal attempts to further reduce the complexity for inter-layer SAO processing studied in SCE3.3. It is achieved by reusing the current method employed in the base-layer HEVC for sample classification for the two diagonal EO classes. The method proposed in SCE3.3 is only applied to the horizontal and vertical classes. The experimental results reportedly show that the proposed algorithm slightly improves the previous method for the spatial scalable ratio 2x (0.1% luma BD-rate saving) and is comparable for other scalable configurations in average BD-rates under the common test conditions.

# Introduction

The inter-layer (IL) sample adaptive offset (SAO) utilized the highpass filtered picture for sample classification is first proposed in JCTVC-L0234 [1] and demonstrated substantial BD-rate savings. However, this method needed to generate the highpass filtered picture from the difference of the reconstructed base-layer picture and the low-pass filtered reconstructed base-layer picture, leading to a significant increase in computational complexity.

The complexity of the IL-SAO algorithm was significantly reduced without showing performance loss by JCTVC-M0114 [2] and was further studied in SCE3.3. In their proposed method, the category index, cat_idx, for edge offset (EO) was determined by

cat_idx = sign(((2*c-c-1-c1+2)>>2)-((2*c1-c-c2+2)>>2))  +

sign(((2*c-c-1-c1+2)>>2)-((2*c-1-c-2-c+2)>>2))+ 2

This sample classification method for EO was equivalent to the sample classification method currently employed by the HEVC SAO but with the reconstructed sample values replaced by the highpass output of the reconstructed pixels with filter coefficients [-1, 2, -1]/4. Due to the highpass filtering operation, four neighboring samples are effectively employed for sample classification, increased from two neighboring samples in the current HEVC. Fig. 1 shows the neighboring samples employed for sample classification using the highpass-filtered reconstructed picture for sample classification in JCTVC-M0114 [2].

Fig. 1.  Sample classification for the four EO classes for IL-SAO with highpass processing

# Proposed method

This proposal attempts to further reduce the complexity for IL-SAO processing. It is achieved by reusing the current method employed in the base-layer HEVC for sample classification for the two diagonal EO classes. The same method with highpass filtering in JCTVC-M0114 [2] is employed for processing the horizontal and vertical classes, as illustrated in Fig. 2. In this way, additional memory access and processing for highpass filtering can be saved when the two diagonal EO classes are selected. The implementation cost can also be saved by reusing the existing base-layer SAO module for processing the two diagonal EO classes.

Fig. 2.  Sample classification for the four EO classes for the proposed IL-SAO

# Experimental Results

The proposed algorithm has been evaluated according to the common test conditions detailed in [3]. The BD-rate results are summarized in Table 1.

Table 1.  BD-rate results of the proposed algorithm (short-length for now, will be updated)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.1% | -1.4% | -1.4% |  |  |  |  |  |  |
| Class B | -0.2% | -0.4% | -0.4% | -0.2% | -0.4% | -0.5% |  |  |  |
| Overall (Test vs Ref) | -0.4% | -0.7% | -0.7% | -0.2% | -0.4% | -0.5% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.0% | -0.8% | -0.9% |  |  |  | -2.3% | -1.7% | -1.9% |
| Class B | -0.4% | -0.5% | -0.3% | -0.3% | -0.6% | -0.4% | -1.0% | -1.0% | -0.6% |
| Overall (Test vs Ref) | -0.5% | -0.6% | -0.5% | -0.3% | -0.6% | -0.4% | -1.4% | -1.2% | -1.0% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.0% | -1.1% | -1.4% |  |  |  | -3.0% | -2.2% | -2.1% |
| Class B | -0.6% | -1.1% | -1.0% | -0.7% | -1.3% | -1.1% | -1.7% | -1.6% | -1.2% |
| Overall (Test vs Ref) | -0.7% | -1.1% | -1.1% | -0.7% | -1.3% | -1.1% | -2.0% | -1.8% | -1.5% |
| Overall (Test vs single layer) | 45.3% | 64.6% | 69.8% | 34.1% | 54.7% | 64.5% | 34.3% | 56.7% | 71.6% |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.0% | -1.0% | -1.1% |  |  |  | -2.1% | -1.7% | -1.9% |
| Class B | -0.4% | -0.9% | -0.7% | -0.3% | -0.9% | -0.7% | -0.9% | -1.3% | -1.0% |
| Overall (Test vs Ref) | -0.6% | -0.9% | -0.9% | -0.3% | -0.9% | -0.7% | -1.3% | -1.4% | -1.3% |

# References

- J. Chen, K. Rapaka, M. Karczewicz, “High frequency pass inter layer sample adaptive offset filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0234, January, 2013.

- G. Laroche, P. Onno, J. Taquet, C. Gisquet, E. Francois, “Non-SCE4: On interlayer SAO in SHVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0114, April, 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-M1009, April, 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).