| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0251 |
| --- | --- |

| Title: | SCE3 : cross-check of Test 3.4.1 on chroma enhancement for inter layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification results for SCE3 Test 3.4.1, JCTVC-N0184 on chroma enhancement for inter layer prediction. Cross-check simulation confirms that BD-rate results match with those from proponents. Study of software reports that implementation matches the description in the contribution JCTVC-N0184.

# Introduction

Chroma enhancement for inter layer prediction was proposed in the 13th JCTVC meeting in JCTVC-M0183 and further studied in SCE3 as Test 3.4.1. In Test 3.4.1, chroma is enhanced by applying a high-pass filter to 3x4 corresponding luma pixels. The filters are different for Cb and Cr. An on/off control flag is signaled at slice header. Filter coefficients are also signaled at slice header.

We study the software implementation and confirm that the implementation matches the description in JCTVC-N0184.

# Simulation and results

The code is compiled using MS Visual Studio 2010. The released source code is compiled under macro settings as follows.

#define CHROMA_ENHANCE                   1

## BD-rate performance

BD-rate numbers match with results from proponents. Encoding and decoding time is not accurate because a heterogeneous cluster is used for simulation.  The results are summarized in Table 1.

Table 1 Summary of crosschecking results

## Complexity measurement

Complexity modules report the same numbers from log files as proponents. As explained in Section 3 of 4.3.1 report, memory access pattern for Luma is (Width+1)×(Height+2). Cell J20, K20 and J25 and K25 in "WorstCaseMemory" sheet have been updated to this pattern in complexity results excel file from proponents. Cross-check confirms that results match with those from proponents when such changes are applied to complexity template.