# Working Draft Text

Highlighted in yellow are the changes with respect to SHVC Working Draft 2 (version 3):

Syntax and semantics modifications:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_temporal_mvp_enabled_flag && nuh_layer_id>0 ) { |  |
| sps_override_mfm_flag | u(1) |
| } |  |
| … |  |
| } |  |

sps_override_mfm_flag equal to 1 indicates that a slice level flag el_collocated_enabled_flag used to control the overwrite process is present in the slice header, while sps_override_mfm_flag equal to 0 indicates that a slice level flag el_collocated_enabled_flag used to control the overwrite process is not present in the slice header. When not present sps_override_mfm_flag is inferred to be zero.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_override_mfm_flag ) { |  |
| el_collocated_enabled_flag | u(1) |
| if( el_collocated_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| el_collocated_from_l0_flag | u(1) |
| if( ( el_collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !el_collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| el_collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| } |  |

el_collocated_enabled_flag equal to 1 specifies that the reference layer mapped motion information used for temporal motion vector predictors may be overwritten in the current slice, el_collocated_enabled_flag equal to 0 specifies that the reference layer mapped motion information used for temporal motion vector predictors will not be overwritten in the current slice, When not present el_collocated_enabled_flag is inferred to be zero.

el_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for overwriting is derived from reference picture list 0. el_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for overwriting is derived from reference picture list 1. When el_collocated_from_l0_flag is not present, it is inferred to be equal to 1.

el_collocated_ref_idx specifies the reference index of the collocated picture used for overwriting.

When slice_type is equal to P or when slice_type is equal to B and el_collocated_from_l0 is equal to 1, el_collocated_ref_idx refers to a picture in list 0, and the value of el_collocated_ref_idx shall be in the range of 0 to num_ref_idx_l0_active_minus1, inclusive.

When slice_type is equal to B and el_collocated_from_l0 is equal to 0, el_collocated_ref_idx refers to a picture in list 1, and the value of el_collocated_ref_idx shall be in the range of 0 to num_ref_idx_l1_active_minus1, inclusive.

- G.8.1.4.2 Resampling process of  picture motion field

Input to this process is rlPicMotion specifying the motion field of the picture rlPic.

Output of this process is rsPicMotion specifying the resampled motion field, and modified reference picture lists, of the resampled picture.

The motion field of rlPic specified by rlPicMotion consists of:

- a ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) array predModeRL specifies the prediction modes of the reference layer picture rlPic,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays refIdxLXRL specify the reference indices of the reference layer picture rlPic, with X = 0,1,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays mvLXRL specify the luma motion vectors of the reference layer picture rlPic, with X = 0,1,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays refPicOrderCntLXRL specify the reference picture order counts of the reference layer picture rlPic, with X = 0, 1,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays predFlagLXRL specify the prediction list utilization flags of the reference layer picture rlPic, with X = 0,1.

The resampled motion field specified by rsPicMotion consists of:

–	a ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) array predMode specifies the prediction modes of the resampled picture,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays refIdxLX specify the reference indexes of the resampled picture, with X = 0,1,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays mvLX  specify the luma motion vectors of the resampled picture, with X = 0,1,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays refPicOrderCntLX specify the reference picture order counts of the resampled picture, with X = 0, 1

- two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays predFlagLX specify the prediction list utilization flags of the resampled picture, with X = 0,1.

If el_collocated_enabled_flag is equal to 1, collocated reference picture in current layer clPic is derived as follows:

- If slice_type is equal to B and el_collocated_from_l0_flag is equal to 0, clPic is set equal to RefPicList1[ el_collocated_ref_idx ].

- Otherwise (slice_type is equal to B and el_collocated_from_l0_flag is equal to 1 or slice_type is equal to P), clPic is set equal to RefPicList0[ el_collocated_ref_idx ].

The motion field of clPic specified by clPicMotion consists of:

–	a ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) array predModeCL specifies the prediction modes of the current layer picture clPic,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays refIdxLXCL specify the reference indexes of the current layer picture clPic, with X = 0,1,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays mvLXCL  specify the luma motion vectors of the current layer picture clPic, with X = 0,1,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays refPicOrderCntLXCL specify the reference picture order counts of the current layer picture clPic, with X = 0, 1

- two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays predFlagLXCL specify the prediction list utilization flags of the current layer picture clPic, with X = 0,1.

The list refPicListXRS specifies the reference picture list LX of the resampled picture, with X = 0,1.

For each luma sample location xPb = 0 ... ( ( PicWidthInSamplesL + 15 ) >> 4 ) − 1 and yPb = 0 … ( ( PicHeightInSamplesL + 15 ) >> 4) − 1,

–	The variables xP and yP are set to ( xPb  << 4 ) and ( yPb  << 4 ), respectively,

–	If el_collocated_enabled_flag is equal to 0, The variables predMode[xP][yP], refIdxLX[xP][yP], mvLX[xP][yP] and refPicOrderCntLX[xP][yP], and predFlagLX[xP][yP], with X = 0,1, of the resampled picture are derived by invoking inter layer motion derivation process specified in subclause G.8.1.4.2.1 with the luma location ( xP, yP ), predModeRL, refIdxLXRL, mvLXRL,   refPicOrderCntLXRL, and predFlagLXRL, with X = 0,1, given as input.

- Otherwise, The variables predMode[xP][yP], refIdxLX[xP][yP], mvLX[xP][yP] and refPicOrderCntLX[xP][yP], predFlagLX[xP][yP], refPicListXRS with X = 0,1, of the resampled picture are updated by invoking inter layer motion update process specified in subclause G.8.1.4.2.1 with the luma location ( xP, yP ), predModeRL, refIdxLXRL, mvLXRL, refPicOrderCntLXRL, predFlagLXCL, refPicListXRS, clPic, predModeCL, refIdxLXCL, mvLXCL,   refPicOrderCntLXCL, and predFlagLXCL, with X = 0,1, given as input.

- G.8.1.4.2.1 Derivation process for inter layer motion

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- the reference layer prediction modearray predModeRL,

- the reference layer reference index arrays refIdxL0RL and refIdxL1RL

- the reference layer motion vector arrays mvL0RL and mvL1RL

- the reference layer reference picture order counts arrays refPicOrderCntL0RL and refPicOrderCntL1RL

- the reference layer prediction list utilization flag arrays predFlagL0RL and predFlagL1RL.

- the reference picture lists refPicList0RS and refPicList1RS of the resampled picture

- the current layer collocated reference picture clPic

- the current layer prediction mode array predModeCL

- the current layer reference index arrays refIdxL0CL and refIdxL1CL

- the current layer motion vector arrays mvL0CL and mvL1CL

- the current layer reference picture order counts arrays refPicOrderCntL0CL and refPicOrderCntL1CL

- the current layer prediction list utilization flag arrays predFlagL0CL and predFlagL1CL.

Outputs of this process are

- a derived prediction mode predMode,

- two derived motion vectors mvL0 and mvL1

- two derived reference indices refIdxL0 and refIdxL1

- two derived reference picture order counts refPicOrderCntL0 and refPicOrderCntL1

- two derived prediction list utilization flags predFlagL0 and predFlagL1

- two modified reference picture lists refPicList0RS and refPicList1RS of the resampled picture

The variables predMode, mvLX, refIdxLX, refPicOrderCntLX, and predFlagLX are derived as follows.

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xP + 8		(G-1)

yPCtr = yP + 8		(G-2)

- The derivation process for reference layer luma sample location specified in subclause G.6.1Error: Reference source not found is invoked with luma location ( xPCtr , yPCtr ) given as the inputs and ( xRef , yRef ) as output.

- The collocated position (xRL, yRL) is derived as follows

xRL = ( xRef >> 4 ) << 4		(G-3)

yRL = ( yRef >> 4 ) << 4		(G-4)

- The reference layer motion vector is derived as follows

- Set the variables usedL0CL to 0 and usedL1CL to 0

- If el_collocated_enabled_flag is equal to 1, for each X = 0, 1,  the following applies:

The motion vector mvCL, the reference index refIdxCL, and the reference list identifier LId are derived as follows:

- predMode[ xP ][ yP ] is set equal to MODE_INTRA and both components of mvL0[ xP ][ yP ] and mvL1[ xP ][ yP ] are set to 0, refIdxL0[ xP ][ yP ] and refIdxL1[ xP ][ yP ] are set to –1, refPicOrderCntL0[ xP ][ yP ] and refPicOrderCntL1[ xP ][ yP ] are set to –1, predFlagL0[ xP ][ yP ] and predFlagL1[ xP ][ yP ] are set to 0

        - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice, mvCL, refIdxCL, and LId are set equal to mvLXCL[ xP ][ yP ], refIdxLXCL[ xP ][ yP ] and LX, respectively.

        - Otherwise, mvCL, refIdxCL, and LId are set equal to mvLNCL[ xP ][ yP ], refIdxLNCL[ xP ][ yP ] and LN, respectively, with N being the value of collocated_from_l0_flag.

        - Set numRefIdxLId equal to the length of resampled picture’s reference picture list refPicListLIdRS

        - When predFlagLIdCL[ xP ][ yP ] is equal to 1, and DiffPicOrderCnt(clPic, refPicListLIdCL[ refIdxCL ]) is not equal to 0, the following applies:

Set usedLXCL equal to 1.

predMode[ xP ][ yP ], predFlagLId[ xP ][ yP ] are derived as follows

predMode[ xP ][ yP ] = MODE_INTER	(xx)

predFlagLId[ xP ][ yP ] = predFlagLIdCL[ xP ][ yP ]	(xx)

refIdxLId[ xP ][ yP ], refPicOrderCntLId[ xP ][ yP ] and mvLId[ xP ][ yP ]  are derived as follows

mvLId[ xP ][ yP ] = mvCL		(xx)

refPicOrderCntLId[xP][yP]=DiffPicOrderCnt(currPic,clPic)+refPicOrderCntLIdCL[ xP ][ yP ]	(xx)

- If DiffPicOrderCnt( currPic, aPic ) is equal to DiffPicOrderCnt( clPic, refPicListLIdCL[ refIdxCL ] ) and aPic and refPicListLIdCL[ refIdxCL ] have the same long term status for a picture aPic in the reference picture list refPicListLIdRS of the resampled picture then refIdxLId[ xP ][ yP ] is set to be the reference index of aPic

- Otherwise, if numRefIdxLId equals to 15, the following applies:

refIdxLId[ xP ][ yP ]  = 0

clPocDiff = DiffPicOrderCnt( clPic, refPicListLIdCL [refIdxCL])	(xx)

currPocDiff = DiffPicOrderCnt( currPic, refPicListLIdRS[ refIdxLId[xP][yP] ] )	(xx)

If clPocDiff is equal to currPocDiff, mvLId[ xP ][ yP ]   is derived as follows:

mvLId[ xP ][ yP ]  = mvCL	(xx)

Otherwise, mvLId is derived as a scaled version of the motion vector mvCL as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(xx)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(xx)

mvLId[ xP ][ yP ]  =  Clip3( −32768, 32767, Sign( distScaleFactor * mvCL ) * ( ( Abs( distScaleFactor * mvCol ) + 127 )  >>  8 ) )	(xx)

where td and tb are derived as follows:

td = Clip3( −128, 127, clPocDiff )	(xx)

tb = Clip3( −128, 127, currPocDiff )	(xx)

- Otherwise, the following applies:

refIdxLId[ xP ][ yP ] is set to numRefIdxLId

Set PicOrderCntVal of refPicListLIdRS[ numRefIdxLId ] to refPicOrderCntLId[ xP ][ yP ]

Set length of refPicListLIdRS equal to numRefIdxLId + 1

- If  ( ( xRL < 0 ) or ( xRL >= RefLayerPicWidthInSamplesL ) or ( yRL < 0 ) or ( yRL >= RefLayerPicHeightInSamplesL ) and ( usedL0CL = = 0 && usedL1CL = = 0 ), predMode[ xP ][ yP ] is set to MODE_INTRA.

- Otherwise, predMode[ xP ][ yP ] is derived as follows

predMode[ xP ][ yP ] = predModeRL[ xRL ][ yRL ]		(G-5)

- If predMode[ xP ][ yP ] is equal to MODE_INTER, for each X = 0, 1, and usedLXCL equal to 0, the following applies

refIdxLX[ xP ][ yP ] = refIdxLXRL[ xRL ][ yRL ]		(G-6)

refPicOrderCntLX[ xP ][ yP ] = refPicOrderCntLXRL[ xRL ][ yRL ]	(G-7)

predFlagLX[ xP ][ yP ] = predFlagLXRL[ xRL ][ yRL ]	(G-8)

        - If ScaledRefLayerPicWidthInSamplesL is not equal to RefLayerPicWidthInSamplesL, mvLX[ xP ][ yP ][ 0 ] is derived as follows:

scaleFactorMVX = Clip3( −4096, 4095, ( ( ScaledRefLayerPicWidthInSamplesL << 8 ) + ( RefLayerPicWidthInSamplesL >> 1 ) ) / RefLayerPicWidthInSamplesL)	(G-9)

mvLX[ xP ][ yP ][0] = Clip3( −32768, 32767, Sign(scaleFactorMVX * 
mvLXRL[ xRL ][ yRL ][ 0 ] ) * 	( ( Abs ( scaleFactorMVX * mvLXRL[ xRL ][ yRL ][ 0 ] )
 + 127 ) >> 8 ) ) 			(G-10)

        - Otherwise,

mvLX[ xP ][ yP ][ 0 ] = mvLXRL[ xRL ][ yRL ][ 0 ] 	(G-11)

        - If ScaledRefLayerPicHeightInSamplesL is not equal to RefLayerPicHeightInSamplesL, mvLX[ xP ][ yP ][ 1 ] is derived as follows:

scaleFactorMVY = Clip3( −4096, 4095, ( ( ScaledRefLayerPicHeightInSamplesL << 8 ) + ( RefLayerPicHeightInSamplesL >> 1 ) ) / RefLayerPicHeightInSamplesL)	(G-12)

mvLX[ xP ][ yP ][ 1 ] = Clip3( −32768, 32767, Sign(scaleFactorMVY * 
mvLXRL[ xRL ][ yRL ][ 1 ] ) * 	( ( Abs  ( scaleFactorMVY * mvLXRL[ xRL ][ yRL ][ 1 ] )
 + 127 ) >> 8 ) )			(G-13)

        - Otherwise,

mvLX[ xP ][ yP ][ 1 ] = mvLXRL[ xRL ][ yRL ][ 1 ] 	(G-14)

- Otherwise, if predMode[ xP ][ yP ] is equal to MODE_INTRA

        - both components of mvL0[ xP ][ yP ] and mvL1[ xP ][ yP ] are set to 0, refIdxL0[ xP ][ yP ] and refIdxL1[ xP ][ yP ] are set to –1, refPicOrderCntL0[ xP ][ yP ] and refPicOrderCntL1[ xP ][ yP ] are set to –1, predFlagL0[ xP ][ yP ] and predFlagL1[ xP ][ yP ] are set to 0.