| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0254 |
| --- | --- |

| Title: | Non-RCE3: Pipeline Friendly Intra Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Liwei Guo, Marta Karczewicz, Rajan Joshi 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document addresses pipelining issues for the intra motion compensation method. It is asserted that restricting the search region to the left CTU or to the rightmost 4 columns of the left CTU makes intra motion compensation more suitable for pipelining. It is proposed to remove the interpolation filter in intra motion compensation to reportedly improve memory bandwidth and pipelining. Finally, intra vectors are binarized with an exponential-Golomb code to allow the signaling of any position within the search area. The BD-rate of each of these modifications is reported. The average BD-rate impact of removing the intra interpolation filter is 0.1% for lossless and 0.3% for lossy, respectively.

# Introduction

Intra motion compensation (MC) is illustrated in Fig. 1. For a coding unit (CU) using Intra MC, the prediction signal is obtained from the reconstructed region in the current picture pointed to by a motion vector (MV). Then, the motion vector and residual are coded. Issues identified with the Intra MC presented in [1] are the following:

- Need to buffer 2 left CTUs of reconstructed non-filtered data. This is a large amount of data for many practical implementations.

- Need to apply interpolation filter in intra frames. This increases complexity and may increase memory bandwidth requirements.

- The fixed length binarization (0..63) of MV is not flexible enough to allow for the signaling of all the prediction positions in the search area.

Methods proposed in this contribution alleviate the first issue and eliminate the second and third ones.  Additional constraints on the MVs [1] such as restriction to 1-D are discussed in the accompanying contribution JCTVC-N0256.

Fig. 1 Illustration of intra MC as in [1]

# Search Area Restriction

In [1], the search region covers the current CTU and the two left CTUs. Therefore, deblocking and SAO of these two CTUs have to be delayed until the search and motion compensation is completed. This exerts additional burden on the pipelining and a large extra buffer is required. Motivated by this, it is proposed to limit the search region for Intra MC to make it better suited for pipelining:

- Restriction 1: Search restricted to the current CTU and the left CTU.

- Restriction 2: Search restricted to the current CTU and the rightmost 4 columns of the left CTU.

## Restriction 1

The horizontal MV is restricted to be in the region of the current CTU and the left CTU. Vertical MVs are restricted to point inside the current CTU (as in [1])

With such a restriction, the pipeline delay is only one CTU, since only the left reconstructed CTU needs to be stored.

## Restriction 2

The horizontal MV is restricted to be in the region of the current CTU and the rightmost 4 columns of the left CTU. Vertical MVs are restricted to point inside the current CTU.

Typically, loop filtering of the left boundary of a CTU is performed only after finishing the encoding or decoding of the current CTU. The rightmost 4 columns of the left CTU have to be stored to perform loop filtering. Thus, the current CTU and the rightmost 4 columns of the left CTU are available and can be used for Intra MC. In this case, no extra pipeline delay is involved and no extra storage is needed.

# Removal of the Interpolation filter

In [1], the MV resolution for luma component is integer-pixel. However, the MV for chroma components may point to a half-pixel position for the 4:2:0 and 4:2:2 color formats. This increases the memory bandwidth requirements and introduces filtering step in the intra coding pipeline.

In this proposal, integer-pixel resolution is used for the MV for chroma components as well. This is accomplished by rounding the MV. By doing this, the interpolation filtering of the chroma components is avoided.

# MV binarization

In [1], the MVs are binarized using fixed length codes (6 bits). This binarization doesn’t allow signaling of all the positions within the search area, since the maximum value is bounded by 63. This restriction is unnecessary, since the entire search area is available at the encoder as well as the decoder.

An exponential-Golomb code with parameter 3 is proposed to code the MVs.

# Experimental Results

The proposed methods are implemented in HM10.1_Rext3.0 and simulated under the RCE3 common test conditions Error: Reference source not found. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. The performance is compared to the anchor for lossless as well as lossy configuration in terms of BD-rates.

Results are provided step-by-step to assess the impact of each change:

- Interpolation filter on/off

  - Setting: with fixed length MV and search area restricted to the left CTU

- Exponential-Golomb MV vs. Fixed length MV

  - Setting: without the interpolation filter and search area restricted to the left CTU

  - For exponential-Golomb all the positions can be searched

- Restriction of the search area: 2 left CTUs, 1 left CTU or 4 columns in the left CTU

  - Setting: without the interpolation filter and with exponential-Golomb binarization

## Performance of the Interpolation filter

This section provides BD-rate for lossless (Table 1 & Table 2) and lossy (Table 3 & Table 4) for restricting the search to the left CTU with and without interpolation filter.

Table 1 Lossless results-Interpolation filter OFF vs HM10.1_Rext3.0.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.2% | 31.7 | 32.7 | -1.3% | 49.8 | 50.5 | -0.6% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 14.1 | -21.3% | 100.4 | 142.8 | -16.9% | 381.6 | 555.2 | -14.8% |
| SC YUV 444 | 11.4 | 16.7 | -18.3% | 128.9 | 187.0 | -16.5% | 325.6 | 508.8 | -16.0% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 126% |  |  | 108% |  |  | 100% |  |  |
| Dec Time[%] | 94% |  |  | 108% |  |  | 107% |  |  |

Table 2 Lossless results - Interpolation filter OFF vs ON.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.4 | 5.4 | 0.1% | 32.7 | 32.7 | 0.0% | 50.6 | 50.5 | 0.0% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 14.1 | 14.1 | 0.0% | 142.8 | 142.8 | 0.0% | 555.2 | 555.2 | 0.0% |
| SC YUV 444 | 16.7 | 16.7 | 0.0% | 187.0 | 187.0 | 0.0% | 508.8 | 508.8 | 0.0% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 98% |  |  |
| Dec Time[%] | 94% |  |  | 99% |  |  | 100% |  |  |

Table 3  Lossy results - Interpolation filter OFF vs HM10.1_Rext3.0.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -7.6% | -7.5% | -7.6% | -6.5% | -6.6% | -6.6% | -5.6% | -5.7% | -5.7% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -27.3% | -27.3% | -27.6% | -26.8% | -26.9% | -27.1% | -26.4% | -26.4% | -26.4% |
| SC YUV 444 | -23.9% | -23.8% | -24.1% | -23.3% | -23.4% | -23.5% | -22.8% | -22.8% | -22.8% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  | 127% |  |  | 127% |  |  |
| Dec Time[%] | 87% |  |  | 86% |  |  | 86% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -5.6% | -5.9% | -6.0% | -4.8% | -5.2% | -5.2% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -22.0% | -21.8% | -22.0% | -21.4% | -21.3% | -21.4% |  |  |  |
| SC YUV 444 | -20.2% | -20.4% | -20.7% | -19.8% | -19.9% | -20.0% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 108% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.4% | -3.9% | -3.5% | -3.0% | -3.2% | -3.1% |  |  |  |
| Class B | 0.0% | -0.1% | -0.3% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -16.9% | -16.1% | -16.2% | -16.8% | -15.9% | -15.9% |  |  |  |
| SC YUV 444 | -15.9% | -15.4% | -15.7% | -16.0% | -15.4% | -15.5% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 108% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |

Table 4  Lossy results - Interpolation filter OFF vs ON.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.3% | 0.3% | 0.1% | 0.3% | 0.3% | 0.1% | 0.3% | 0.2% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% | 0.2% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | 0.2% | 0.0% | 0.1% | 0.2% | 0.1% |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | -0.2% |  |  |  |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |  |  |  |

## Results for Exponential-Golomb

This section provides BD-rate for lossless (Table 5 & Table 6) and lossy (Table 7 & Table 8) for exponential-Golomb when comparing to fixed length MV.

Table 5 Lossless results - Exponential-Golomb MV vs HM10.1_Rext3.0

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.5% | 31.7 | 32.8 | -1.4% | 49.8 | 50.6 | -0.7% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 14.2 | -22.1% | 100.4 | 144.5 | -17.5% | 381.6 | 565.1 | -15.3% |
| SC YUV 444 | 11.4 | 17.0 | -19.3% | 128.9 | 190.8 | -17.4% | 325.6 | 520.8 | -16.8% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 134% |  |  | 110% |  |  | 106% |  |  |
| Dec Time[%] | 90% |  |  | 106% |  |  | 106% |  |  |

Table 6 Lossless results - Exponential-Golomb MV vs Fixed length MV

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.4 | 5.4 | -0.3% | 32.7 | 32.8 | -0.1% | 50.5 | 50.6 | -0.1% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 14.1 | 14.2 | -1.1% | 142.8 | 144.5 | -0.7% | 555.2 | 565.1 | -0.8% |
| SC YUV 444 | 16.7 | 17.0 | -1.4% | 187.0 | 190.8 | -1.3% | 508.8 | 520.8 | -1.2% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 103% |  |  | 101% |  |  |

Table 7 Lossy results - Exponential-Golomb MV vs HM10.1_Rext3.0

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -8.2% | -8.1% | -8.2% | -7.1% | -7.1% | -7.2% | -6.0% | -6.2% | -6.2% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -28.5% | -28.4% | -28.7% | -28.0% | -28.0% | -28.1% | -27.5% | -27.4% | -27.4% |
| SC YUV 444 | -25.2% | -25.1% | -25.4% | -24.6% | -24.6% | -24.8% | -24.1% | -24.0% | -24.0% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 128% |  |  | 129% |  |  | 130% |  |  |
| Dec Time[%] | 87% |  |  | 87% |  |  | 87% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6.0% | -6.2% | -6.5% | -5.2% | -5.6% | -5.6% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -22.9% | -22.7% | -22.9% | -22.3% | -22.1% | -22.3% |  |  |  |
| SC YUV 444 | -21.4% | -21.6% | -21.9% | -21.2% | -21.3% | -21.4% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 114% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 109% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.7% | -4.0% | -3.9% | -3.2% | -3.4% | -3.3% |  |  |  |
| Class B | 0.0% | -0.2% | -0.4% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -18.1% | -17.2% | -17.2% | -17.6% | -16.7% | -16.6% |  |  |  |
| SC YUV 444 | -16.6% | -16.2% | -16.3% | -16.8% | -16.1% | -16.2% |  |  |  |
| RangeExt | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 110% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |  |  |  |

Table 8 Lossy results-Exponential-Golomb MV vs Fixed length MV

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -0.7% | -0.7% | -0.6% | -0.6% | -0.6% | -0.5% | -0.6% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.7% | -1.6% | -1.6% | -1.6% | -1.5% | -1.4% | -1.5% | -1.4% | -1.4% |
| SC YUV 444 | -2.0% | -1.9% | -2.0% | -1.9% | -1.9% | -1.9% | -1.9% | -1.8% | -1.8% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  | 87% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.4% | -0.6% | -0.4% | -0.4% | -0.5% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.2% | -1.2% | -1.2% | -1.2% | -1.2% | -1.2% |  |  |  |
| SC YUV 444 | -1.8% | -1.7% | -1.8% | -1.9% | -1.9% | -1.9% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 103% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.1% | -0.3% | -0.2% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.5% | -1.4% | -1.4% | -1.0% | -1.0% | -0.9% |  |  |  |
| SC YUV 444 | -1.0% | -1.0% | -0.9% | -1.0% | -0.9% | -0.9% |  |  |  |
| RangeExt | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |  |  |  |

## Results for Search Areas

This section provides BD-rate for lossless (Table 9 & Table 11) and lossy (Table 12 & Table 14) for restricting the search to the two left CTU, left CTU and only 4 columns, using the Exponential-Golomb and no interpolation filter.

Table 9 Lossless results-Exponential-Golomb MVwith 2 CTUs vs HM10.1_Rext3.0

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -3.0% | 31.7 | 33.1 | -1.8% | 49.8 | 50.8 | -0.8% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 14.7 | -24.0% | 100.4 | 148.9 | -19.0% | 381.6 | 590.7 | -16.8% |
| SC YUV 444 | 11.4 | 17.7 | -21.7% | 128.9 | 199.5 | -19.6% | 325.6 | 555.8 | -19.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 135% |  |  | 110% |  |  | 103% |  |  |
| Dec Time[%] | 89% |  |  | 105% |  |  | 104% |  |  |

Table 10 Lossless results-Exponential-Golomb MVwith 1 CTU vs HM10.1_Rext3.0

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.5% | 31.7 | 32.8 | -1.4% | 49.8 | 50.6 | -0.7% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 14.2 | -22.1% | 100.4 | 144.5 | -17.5% | 381.6 | 565.1 | -15.3% |
| SC YUV 444 | 11.4 | 17.0 | -19.3% | 128.9 | 190.8 | -17.4% | 325.6 | 520.8 | -16.8% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 134% |  |  | 110% |  |  | 106% |  |  |
| Dec Time[%] | 90% |  |  | 106% |  |  | 106% |  |  |

Table 11 Lossless results - Exponential-Golomb MVwith 4 columns vs HM10.1_Rext3.0

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.2% | 31.7 | 32.2 | -0.7% | 49.8 | 50.1 | -0.3% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 12.6 | -15.3% | 100.4 | 127.7 | -12.1% | 381.6 | 492.9 | -10.7% |
| SC YUV 444 | 11.4 | 14.3 | -12.1% | 128.9 | 162.2 | -11.0% | 325.6 | 431.7 | -10.8% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 122% |  |  | 104% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 109% |  |  | 103% |  |  |

Table 12 Lossy results - Exponential-Golomb MVwith 2 CTUs vs HM10.1_Rext3.0

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -9.5% | -9.3% | -9.5% | -8.3% | -8.3% | -8.3% | -7.1% | -7.2% | -7.2% |
| Class B | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -30.9% | -30.9% | -31.2% | -30.3% | -30.4% | -30.5% | -29.7% | -29.6% | -29.7% |
| SC YUV 444 | -28.1% | -28.0% | -28.3% | -27.5% | -27.5% | -27.6% | -26.8% | -26.8% | -26.8% |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 126% |  |  | 128% |  |  | 129% |  |  |
| Dec Time[%] | 88% |  |  | 87% |  |  | 87% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6.9% | -7.2% | -7.4% | -6.0% | -6.4% | -6.4% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -25.0% | -24.8% | -25.0% | -24.2% | -24.0% | -24.1% |  |  |  |
| SC YUV 444 | -24.0% | -24.1% | -24.4% | -23.6% | -23.7% | -23.8% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 110% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.2% | -5.1% | -4.3% | -3.7% | -4.1% | -3.9% |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -19.7% | -18.9% | -18.9% | -19.5% | -18.5% | -18.4% |  |  |  |
| SC YUV 444 | -19.0% | -18.7% | -18.8% | -19.1% | -18.6% | -18.7% |  |  |  |
| RangeExt | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 107% |  |  | 107% |  |  |  |  |  |
| Dec Time[%] | 109% |  |  | 109% |  |  |  |  |  |

Table 13 Lossy results - Exponential-Golomb MVwith 1 CTU vs HM10.1_Rext3.0

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -8.2% | -8.1% | -8.2% | -7.1% | -7.1% | -7.2% | -6.0% | -6.2% | -6.2% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -28.5% | -28.4% | -28.7% | -28.0% | -28.0% | -28.1% | -27.5% | -27.4% | -27.4% |
| SC YUV 444 | -25.2% | -25.1% | -25.4% | -24.6% | -24.6% | -24.8% | -24.1% | -24.0% | -24.0% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 128% |  |  | 129% |  |  | 130% |  |  |
| Dec Time[%] | 87% |  |  | 87% |  |  | 87% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6.0% | -6.2% | -6.5% | -5.2% | -5.6% | -5.6% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -22.9% | -22.7% | -22.9% | -22.3% | -22.1% | -22.3% |  |  |  |
| SC YUV 444 | -21.4% | -21.6% | -21.9% | -21.2% | -21.3% | -21.4% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 114% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 109% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.7% | -4.0% | -3.9% | -3.2% | -3.4% | -3.3% |  |  |  |
| Class B | 0.0% | -0.2% | -0.4% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -18.1% | -17.2% | -17.2% | -17.6% | -16.7% | -16.6% |  |  |  |
| SC YUV 444 | -16.6% | -16.2% | -16.3% | -16.8% | -16.1% | -16.2% |  |  |  |
| RangeExt | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 110% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |  |  |  |

Table 14 Lossy results - Exponential-Golomb MVwith 4 columns vs HM10.1_Rext3.0

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -4.4% | -4.3% | -4.4% | -3.7% | -3.8% | -3.8% | -3.2% | -3.3% | -3.3% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -18.9% | -19.0% | -19.2% | -18.9% | -19.1% | -19.1% | -18.8% | -18.8% | -18.8% |
| SC YUV 444 | -15.3% | -15.3% | -15.6% | -15.1% | -15.3% | -15.4% | -15.1% | -15.1% | -15.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  | 123% |  |  | 123% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  | 104% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.2% | -3.4% | -3.5% | -2.8% | -3.0% | -3.1% |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -15.3% | -15.3% | -15.5% | -15.1% | -15.1% | -15.2% |  |  |  |
| SC YUV 444 | -12.9% | -13.1% | -13.3% | -13.0% | -13.1% | -13.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 111% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 106% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.1% | -2.3% | -2.1% | -1.7% | -1.7% | -1.8% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -11.7% | -11.2% | -11.2% | -11.8% | -11.1% | -11.1% |  |  |  |
| SC YUV 444 | -10.1% | -10.0% | -9.9% | -10.5% | -10.2% | -10.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 108% |  |  | 108% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |  |  |  |

# Conclusions

The Intra MC method as presented in [1] has large implications on the pipeline for HEVC. This proposal tackles two critical issues: the large memory requirements / delays in the pipelining and the interpolation filter for intra coding. As a result, the proposed Intra MC becomes better suited for practical implementations. Also, the binarization is made more flexible to allow exploitation of all the available data in the search area.

# References

- M. Budagavi, D.-K. Kwon, “AHG8: Video coding using Intra motion compensation”, JCTVC-M0350, Incheon, KR, April 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, KR, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).