| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0255 |
| --- | --- |

| Title: | SCE2 test 1.3 (combined motion buffer and motion compression modifications) results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com +81 50 3750 2741 Kazushi.Sato@jp.sony.com +1-408-352-4086 jun.xu@am.sony.com |
| Source: | SHARP, Sony Corp./Sony Electronics |  |  |

_____________________________

# Abstract

This contribution reports the results for SCE2 test 1.3 on combined motion buffer and motion compression modifications.

The proposed motion buffer modification (first proposed in JCTVC-M0258) replaces for 16x16 luma pixel blocks motion field mapped from the base layer (BL) with motion information from a candidate enhancement layer (EL) motion field, if the candidate enhancement layer motion information is valid and does not reference its own base layer.

The proposed motion compression modification (first proposed in JCTVC-M0141) performs a 2:1 BL motion data compression after base layer coding and another 2:1 BL motion data compression after enhancement layer coding.

For the test the average luma EL+BL BD rate with SHM-2.0 anchors is determined to be:

Test 1.3 (combined motion buffer and motion compression modifications):
0.0%,	0.0% 		(AI 	2x/1.5x)
-0.8%,	-0.3%,	-0.2%	(RA 	2x/1.5x/SNR)
-0.4%,	0.0%,	0.1%	(LD-B 	2x/1.5x/SNR)
-0.3%,	0.0%,	0.1%	(LD-P 	2x/1.5x/SNR)

# Introduction

In the past JCTVC-M0258 [1] proposed that within the inter-layer picture for a 16x16 luma pixel block the mapped base layer motion information is replaced by motion field from a candidate enhancement layer (EL) motion field, if the candidate EL motion information is valid and does not reference its own base layer.

In JCTVC-M0141 [2] a 2-stage motion data compression scheme was proposed. First BL motion data is compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

The SCE2 test description document [4] on “Combination of inter-layer syntax prediction and motion data compression” includes the combined tool tests. This document reports results on:

- Combined tool corresponding to JCTVC-M0258 + JCTVC-M0141 and listed as test 1.3 in [4].

# Algorithm description

Motion buffer modification: As in JCTVC-M0258 the motion buffer modification process is controlled at the sequence and slice level using syntax elements sps_override_mfm_flag and el_collocated_enabled_flag respectively. The candidate enhancement layer motion field is identified using slice level syntax elements el_collocated_from_flag and el_collocated_ref_idx.

Motion buffer compression modification: As in JCTVC-M0141 the BL motion data is first compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

# Memory requirement for storing motion data

The motion data storage requirement [3] for each modification is listed below:

Motion buffer modification: 			Same as in SHM-2.0.

# Experimental Setup

Below is a summary BD rate results for test 1.3 (combined tool) with SHM-2.0 anchors. The experimental setup corresponds to the common test condition [5] provided by AhG10 with additionally included SNR test case. The detailed results can be found in attached excel sheet.

## Summary of results for test 1.3 (Combined tools - Motion buffer and motion compression modifications)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Dec Time[%] | 93.2% |  |  | 99.5% |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -1.1% | -1.1% |  |  |  | -0.1% | 0.2% | 0.2% |
| Class B | -0.7% | -0.5% | -0.6% | -0.3% | 0.0% | -0.1% | -0.2% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.8% | -0.7% | -0.7% | -0.3% | 0.0% | -0.1% | -0.2% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 18.3% | 32.4% | 31.0% | 15.9% | 28.8% | 29.0% | 14.2% | 32.2% | 34.3% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.9% | -0.8% | -0.9% | -0.3% | -0.1% | -0.1% | -0.2% | 0.1% | 0.2% |
| Dec Time[%] | 101.4% |  |  | 99.5% |  |  | 98.8% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.0% | -1.0% |  |  |  | 0.1% | 0.2% | 0.2% |
| Class B | -0.3% | -0.3% | -0.3% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.4% | -0.5% | -0.5% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 28.0% | 38.3% | 39.0% | 24.7% | 33.1% | 36.0% | 24.4% | 34.9% | 39.6% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.6% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
| Dec Time[%] | 100.6% |  |  | 98.0% |  |  | 99.1% |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.9% | -0.9% |  |  |  | 0.1% | 0.2% | 0.1% |
| Class B | -0.3% | -0.3% | -0.5% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.3% | -0.4% | -0.6% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 26.1% | 37.4% | 38.3% | 22.8% | 32.7% | 35.5% | 23.5% | 34.8% | 39.5% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.7% | -0.1% | -0.1% | -0.1% | 0.1% | 0.2% | 0.1% |
| Dec Time[%] | 98.3% |  |  | 99.3% |  |  | 100.3% |  |  |

# Conclusion

This document reports experimental result of test 1.3. Simulation results show that for the combined tool test 1.3 (combined motion buffer and motion compression modifications) the average luma BD rate is:

0.0%,	0.0% 		(AI 	2x/1.5x)
-0.8%,	-0.3%,	-0.2%	(RA 	2x/1.5x/SNR)
-0.4%,	0.0%,	0.1%	(LD-B 	2x/1.5x/SNR)
-0.3%,	0.0%,	0.1%	(LD-P 	2x/1.5x/SNR)

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. / Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] K. Misra, J. Zhao, A. Segall, "Modified Motion Vector Signalling and Prediction Under Reference Index Based SHVC," JCTVC-M0258, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.

[2] K. Sato, J. Xu, J. Park, B. Jeon, “SCE5: Result of SCE5.2.2,” JCTVC-M0141, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.

[3] K. Sato, J. Xu, “SCE5: Buffer Size Estimation of SCE5.2.x,” JCTVC-M0142, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.

[4] K. Sato, C. Gisquet, “Description of Core Experiment SCE2: Combination of inter-layer syntax prediction and motion data compression,” JCTVC-M1102, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.

[5] X. Li, J. Boyce, P. Onno, and Y.Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model,” JCTVC-M1009, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.