| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0256 |
| --- | --- |

| Title: | Non-RCE3: Intra Motion Compensation with 2-D MVs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Liwei Guo, Marta Karczewicz, Rajan Joshi 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The intra motion compensation method in HEVC range extension core experiment 3 uses 1-D motion vectors binarized with a fixed length code. In this proposal, the intra motion compensation method is extended to support 2-D motion vectors. It is asserted that 2-D motion vectors provide better alignment with the HEVC inter method. The 2-D motion vectors are binarized with an exponential-Golomb code. In a tested variant, motion vectors are predicted prior to coding. Results are also reported for the combination of 2-D intra motion compensation method with the method of JCTVC-N0254, which uses a reduced prediction area and no interpolation filter.

# Introduction

Intra motion compensation (MC) is illustrated in Fig. 1. For the coding units (CUs) using intra MC, the prediction block is obtained from the reconstructed region. Then, the motion vectors (MVs) and residual are coded.

In [1], the Intra MC diverges from the inter coding case in, at least, the following:

- MVs are restricted to be 1-D (i.e., either horizontal or vertical) instead of being 2-D

  - This removes flexibility from the encoder without providing benefits in the pipelining

- Binarization is fixed length instead of exponential-Golomb

- A new syntax element is introduced to signal whether the MV is horizontal or vertical

This proposal proposes better alignment of the Intra MC method with the HEVC inter method.

Fig. 1 Illustration of intra MC in [1]

# Technical description

## Intra MC with 2-D MV

In this proposal, intra MC is extended to support 2-D MVs, so that both MV components can be non-zero at the same time (as in the HEVC inter case).

This provides more flexibility to Intra MC than [1] where the MV is restricted to be strictly horizontal or vertical (and a new flag is introduced to indicate whether the MV is horizontal or vertical). Repeated patterns in screen content are not necessarily in the same row or column: large gains are obtained with the 2-D MVs when the content correlation is not strictly horizontal or vertical.

## MV Coding

Two MV coding methods are tested:

- Method 1: The left or above MV is selected as the MV predictor and the resulting MVD is coded. First, one flag indicates whether the MVD is zero. When MVD is not zero, an exponential-Golomb of 3rd order codes the remaining absolute level of the MVD and a flag codes its sign.

- Method 2: No predictor is used and the MV is coded using the exponential-Golomb codes used for MVD coding in HEVC.

## Pipeline Friendly Intra MC

The 2-D Intra MC is combined with the pipeline friendly approach proposed in [2]:

- No interpolation filters are used

- MV search area is restricted. Two cases are analyzed:

  - Search area is the current CTU and the left CTU or

  - Search area is the current CTU and the rightmost 4 columns of the left CTU.

## Encoder Aspects

The following aspects of the encoder are also discussed in the experimental results section:

- Fast encoder for the 2-D MV (instead of full search)

- Restricting the MV to be 1-D (but use 2-D signaling)

# Experimental Results

In this section, experimental results are presented to analyze the proposed 2-D Intra MC. The proposed methods are implemented in HM10.1_Rext3.0 and simulated under the RCE3 common test conditions [3]. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. The performance is compared to the anchor for lossless as well as lossy configuration in terms of BD-rates.

Results are provided to assess the impact of each aspect:

- Performance of 2-D MV vs 1-D MV

- Fast search vs full search

- Performance of the proposed 2-D signaling

  - 1-D search using 2-D signaling vs 1-D signaling

  - MV coding: method 1 vs method 2

The results are provided for the two cases of restricting the search area. Note that the tested methods do not include interpolation filter for chroma (refer to [2] for the impact of the interpolation filter).

## Performance of 2-D MV

Results of this section are obtained with MV coding method 1.

Results for lossless (Table 1) and lossy (Table 2) of the 2-D MV for

- Search area is the current CTU and the left CTU

- Full search

Table 1

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.5 | -4.4% | 31.7 | 34.4 | -3.2% | 49.8 | 53.4 | -2.4% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 15.1 | -26.3% | 100.4 | 152.6 | -21.0% | 381.6 | 581.4 | -18.1% |
| SC YUV 444 | 11.4 | 18.5 | -23.1% | 128.9 | 207.0 | -20.8% | 325.6 | 558.6 | -19.7% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 282% |  |  | 154% |  |  | 127% |  |  |
| Dec Time[%] | 92% |  |  | 104% |  |  | 103% |  |  |

Table 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -11.2% | -11.1% | -11.4% | -9.7% | -9.9% | -10.0% | -8.3% | -8.5% | -8.6% |
| Class B | -0.4% | -0.6% | -0.5% | -0.3% | -0.4% | -0.4% | -0.1% | -0.2% | -0.2% |
| SC RGB 444 | -35.5% | -35.5% | -35.8% | -34.7% | -34.8% | -35.0% | -33.9% | -33.9% | -33.9% |
| SC YUV 444 | -29.8% | -29.8% | -30.1% | -29.3% | -29.4% | -29.6% | -28.7% | -28.7% | -28.7% |
| RangeExt | -0.4% | -0.4% | -0.5% | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 221% |  |  | 213% |  |  | 206% |  |  |
| Dec Time[%] | 81% |  |  | 81% |  |  | 81% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -8.5% | -9.0% | -9.1% | -7.4% | -7.9% | -8.0% |  |  |  |
| Class B | -0.1% | -0.3% | -0.4% | -0.1% | -0.2% | -0.2% |  |  |  |
| SC RGB 444 | -29.3% | -29.1% | -29.3% | -28.3% | -28.1% | -28.2% |  |  |  |
| SC YUV 444 | -25.9% | -26.1% | -26.4% | -25.4% | -25.6% | -25.7% |  |  |  |
| RangeExt | -0.2% | -0.4% | -0.4% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 151% |  |  | 150% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -5.2% | -5.6% | -5.2% | -4.6% | -4.7% | -4.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.4% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -23.5% | -22.6% | -22.7% | -23.2% | -22.2% | -22.2% |  |  |  |
| SC YUV 444 | -20.1% | -19.7% | -19.8% | -20.6% | -19.9% | -20.0% |  |  |  |
| RangeExt | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 142% |  |  | 141% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |

Results for lossless (Table 3) and lossy (Table 4) of the 2-D MV for

- Search area is the current CTU and the left CTU

- Fast search

Table 3

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.5 | -4.4% | 31.7 | 34.4 | -3.2% | 49.8 | 53.3 | -2.4% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 15.0 | -26.1% | 100.4 | 152.2 | -20.8% | 381.6 | 581.0 | -18.0% |
| SC YUV 444 | 11.4 | 18.4 | -23.1% | 128.9 | 207.5 | -20.9% | 325.6 | 552.6 | -19.6% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 149% |  |  | 102% |  |  | 100% |  |  |
| Dec Time[%] | 94% |  |  | 106% |  |  | 105% |  |  |

Table 4

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -10.9% | -10.8% | -11.1% | -9.5% | -9.8% | -9.8% | -8.2% | -8.5% | -8.5% |
| Class B | -0.4% | -0.5% | -0.4% | -0.3% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% |
| SC RGB 444 | -35.2% | -35.2% | -35.5% | -34.5% | -34.6% | -34.7% | -33.6% | -33.5% | -33.6% |
| SC YUV 444 | -29.6% | -29.6% | -29.9% | -29.2% | -29.2% | -29.4% | -28.6% | -28.6% | -28.6% |
| RangeExt | -0.3% | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 133% |  |  | 129% |  |  | 127% |  |  |
| Dec Time[%] | 85% |  |  | 85% |  |  | 85% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -8.2% | -8.6% | -8.6% | -7.2% | -7.7% | -7.6% |  |  |  |
| Class B | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% |  |  |  |
| SC RGB 444 | -29.1% | -28.9% | -29.1% | -28.1% | -28.0% | -28.0% |  |  |  |
| SC YUV 444 | -25.9% | -26.1% | -26.4% | -25.5% | -25.6% | -25.8% |  |  |  |
| RangeExt | -0.1% | -0.3% | -0.2% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 114% |  |  | 113% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -5.0% | -5.1% | -5.3% | -4.5% | -4.4% | -4.8% |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -23.8% | -22.9% | -22.9% | -23.2% | -22.2% | -22.2% |  |  |  |
| SC YUV 444 | -19.8% | -19.5% | -19.5% | -20.3% | -19.7% | -19.8% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 110% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |

Results for lossless (Table 5) and lossy (Table 6) of the 2-D MV for

- Search area is the current CTU and the rightmost 4 columns of the left CTU

- Full search

Table 5

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.7% | 31.7 | 33.3 | -2.0% | 49.8 | 52.2 | -1.7% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 13.0 | -18.2% | 100.4 | 131.6 | -14.5% | 381.6 | 497.5 | -12.4% |
| SC YUV 444 | 11.4 | 15.0 | -14.8% | 128.9 | 169.0 | -13.1% | 325.6 | 443.8 | -12.4% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 172% |  |  | 117% |  |  | 113% |  |  |
| Dec Time[%] | 97% |  |  | 109% |  |  | 108% |  |  |

Table 6

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -6.5% | -6.6% | -6.8% | -5.7% | -5.9% | -6.0% | -4.9% | -5.1% | -5.1% |
| Class B | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | -24.0% | -24.3% | -24.5% | -23.8% | -24.1% | -24.2% | -23.5% | -23.6% | -23.6% |
| SC YUV 444 | -18.4% | -18.6% | -18.8% | -18.4% | -18.6% | -18.7% | -18.3% | -18.4% | -18.4% |
| RangeExt | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 153% |  |  | 150% |  |  | 148% |  |  |
| Dec Time[%] | 91% |  |  | 91% |  |  | 91% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -5.0% | -5.4% | -5.6% | -4.4% | -4.8% | -4.8% |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -19.6% | -19.6% | -19.8% | -19.1% | -19.1% | -19.2% |  |  |  |
| SC YUV 444 | -15.9% | -16.2% | -16.4% | -15.8% | -16.1% | -16.1% |  |  |  |
| RangeExt | -0.1% | -0.3% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 120% |  |  | 120% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.1% | -3.5% | -3.4% | -2.7% | -2.9% | -2.9% |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -15.6% | -14.9% | -15.0% | -15.4% | -14.7% | -14.7% |  |  |  |
| SC YUV 444 | -12.0% | -11.8% | -11.8% | -12.6% | -12.2% | -12.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 117% |  |  | 117% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |  |  |  |

Results for lossless () and lossy (Table 8) of the 2-D MV for

- Search area is the current CTU and the rightmost 4 columns of the left CTU.

- Fast search

Table 7

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.6% | 31.7 | 33.3 | -2.0% | 49.8 | 52.2 | -1.7% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 13.0 | -18.2% | 100.4 | 131.9 | -14.6% | 381.6 | 495.0 | -12.0% |
| SC YUV 444 | 11.4 | 14.9 | -14.8% | 128.9 | 170.2 | -13.3% | 325.6 | 442.2 | -12.4% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 122% |  |  | 99% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 106% |  |  | 106% |  |  |

Table 8

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -6.4% | -6.3% | -6.6% | -5.6% | -5.7% | -5.8% | -4.8% | -5.0% | -5.0% |
| Class B | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | -23.9% | -24.1% | -24.4% | -23.8% | -24.0% | -24.1% | -23.4% | -23.5% | -23.5% |
| SC YUV 444 | -18.3% | -18.5% | -18.7% | -18.4% | -18.6% | -18.7% | -18.3% | -18.4% | -18.4% |
| RangeExt | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 119% |  |  | 117% |  |  | 116% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  | 92% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.8% | -5.3% | -5.5% | -4.3% | -4.7% | -4.8% |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -19.6% | -19.6% | -19.8% | -19.1% | -19.1% | -19.2% |  |  |  |
| SC YUV 444 | -15.9% | -16.2% | -16.3% | -15.9% | -16.1% | -16.1% |  |  |  |
| RangeExt | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 106% |  |  | 107% |  |  |  |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.8% | -3.4% | -3.4% | -2.6% | -2.7% | -3.0% |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -15.4% | -14.9% | -14.9% | -15.3% | -14.6% | -14.6% |  |  |  |
| SC YUV 444 | -12.1% | -11.9% | -11.9% | -12.8% | -12.4% | -12.5% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 109% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 107% |  |  | 108% |  |  |  |  |  |

## Performance of 2-D MV vs 1-D MV

Gains of the 2-D with MV coding method 1 with respect to 1-D MV method using Exponential-Golomb of 3rd order.

Results for lossless () and lossy (Table 10) for

- Search area is the current CTU and the left CTU

Table 9

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.4 | 5.5 | -2.1% | 32.8 | 34.4 | -1.8% | 50.6 | 53.4 | -1.8% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 14.2 | 15.1 | -5.4% | 144.5 | 152.6 | -4.5% | 565.1 | 581.4 | -3.3% |
| SC YUV 444 | 17.0 | 18.5 | -5.7% | 190.8 | 207.0 | -4.9% | 520.8 | 558.6 | -4.2% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 222% |  |  | 142% |  |  | 126% |  |  |
| Dec Time[%] | 101% |  |  | 93% |  |  | 96% |  |  |

Table 10

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -3.5% | -3.5% | -3.7% | -3.0% | -3.2% | -3.3% | -2.6% | -2.7% | -2.8% |
| Class B | -0.4% | -0.4% | -0.4% | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |
| SC RGB 444 | -9.7% | -9.8% | -9.9% | -9.3% | -9.4% | -9.5% | -8.9% | -8.9% | -9.0% |
| SC YUV 444 | -7.1% | -7.2% | -7.2% | -7.2% | -7.3% | -7.4% | -7.2% | -7.2% | -7.3% |
| RangeExt | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 177% |  |  | 170% |  |  | 164% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.9% | -3.1% | -3.0% | -2.5% | -2.7% | -2.7% |  |  |  |
| Class B | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% |  |  |  |
| SC RGB 444 | -8.3% | -8.3% | -8.3% | -7.8% | -7.8% | -7.8% |  |  |  |
| SC YUV 444 | -6.5% | -6.5% | -6.5% | -6.3% | -6.4% | -6.4% |  |  |  |
| RangeExt | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 136% |  |  | 133% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.7% | -1.7% | -1.5% | -1.5% | -1.4% | -1.3% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -6.6% | -6.6% | -6.6% | -6.9% | -6.7% | -6.7% |  |  |  |
| SC YUV 444 | -4.7% | -4.8% | -4.7% | -5.3% | -5.3% | -5.3% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 129% |  |  | 128% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |

Results for lossless (Table 11) and lossy (Table 12) for

- Search area is the current CTU and the rightmost 4 columns of the left CTU

Table 11

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.3 | 5.4 | -1.6% | 32.2 | 33.3 | -1.4% | 50.1 | 52.2 | -1.4% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 12.6 | 13.0 | -3.4% | 127.7 | 131.6 | -2.8% | 492.9 | 497.5 | -1.8% |
| SC YUV 444 | 14.3 | 15.0 | -3.3% | 162.2 | 169.0 | -2.7% | 431.7 | 443.8 | -2.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 148% |  |  | 114% |  |  | 117% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  | 104% |  |  |

Table 12

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.4% | -2.5% | -2.7% | -2.1% | -2.3% | -2.4% | -1.8% | -1.9% | -2.0% |
| Class B | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | -6.2% | -6.4% | -6.4% | -6.0% | -6.1% | -6.1% | -5.7% | -5.7% | -5.8% |
| SC YUV 444 | -4.0% | -4.1% | -4.0% | -4.1% | -4.2% | -4.2% | -4.2% | -4.2% | -4.3% |
| RangeExt | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 128% |  |  | 126% |  |  | 124% |  |  |
| Dec Time[%] | 90% |  |  | 91% |  |  | 92% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.0% | -2.1% | -2.2% | -1.7% | -1.9% | -1.9% |  |  |  |
| Class B | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -4.9% | -5.0% | -5.0% | -4.6% | -4.6% | -4.6% |  |  |  |
| SC YUV 444 | -3.6% | -3.8% | -3.7% | -3.5% | -3.7% | -3.6% |  |  |  |
| RangeExt | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 110% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.1% | -1.3% | -1.4% | -1.0% | -1.3% | -1.2% |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -4.3% | -4.2% | -4.3% | -4.1% | -4.0% | -4.1% |  |  |  |
| SC YUV 444 | -2.3% | -2.3% | -2.3% | -2.5% | -2.5% | -2.5% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 108% |  |  | 108% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |  |  |  |

Comparison of 2-D MV with 1 CTU search area versus 1-D MV with 2 CTU search area. Results for lossless (Table 13) and lossy (Table 14).

Table 13

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | \ | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.4 | 5.5 | -1.6% | 33.1 | 34.4 | -1.5% | 50.8 | 53.4 | -1.7% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 14.7 | 15.1 | -2.8% | 148.9 | 152.6 | -2.5% | 590.7 | 581.4 | -1.2% |
| SC YUV 444 | 17.7 | 18.5 | -2.4% | 199.5 | 207.0 | -1.9% | 555.8 | 558.6 | -0.9% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 219% |  |  | 142% |  |  | 128% |  |  |
| Dec Time[%] | 101% |  |  | 94% |  |  | 98% |  |  |

Table 14

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.0% | -2.1% | -2.2% | -1.7% | -1.9% | -1.9% | -1.4% | -1.6% | -1.6% |
| Class B | -0.4% | -0.4% | -0.4% | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |
| SC RGB 444 | -6.3% | -6.4% | -6.4% | -6.1% | -6.1% | -6.2% | -5.8% | -5.8% | -5.8% |
| SC YUV 444 | -2.8% | -2.8% | -2.9% | -2.9% | -3.0% | -3.1% | -3.0% | -3.0% | -3.2% |
| RangeExt | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 179% |  |  | 171% |  |  | 165% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 97% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.8% | -2.0% | -2.0% | -1.5% | -1.8% | -1.8% |  |  |  |
| Class B | -0.1% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% |  |  |  |
| SC RGB 444 | -5.4% | -5.5% | -5.5% | -5.2% | -5.3% | -5.3% |  |  |  |
| SC YUV 444 | -2.9% | -3.0% | -3.0% | -2.8% | -2.9% | -3.0% |  |  |  |
| RangeExt | -0.1% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 140% |  |  | 137% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -0.6% | -0.9% | -0.9% | -0.7% | -0.8% |  |  |  |
| Class B | 0.0% | 0.2% | -0.3% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -4.5% | -4.4% | -4.4% | -4.5% | -4.4% | -4.4% |  |  |  |
| SC YUV 444 | -1.6% | -1.5% | -1.6% | -2.2% | -2.0% | -2.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% |  |  |  |
| Enc Time[%] | 132% |  |  | 131% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |

## Performance of 1-D search using 2-D signaling

These results show the impact of having the 2-D signaling when the MV search is restricted to be 1-D. Results for lossless (Table 15) and lossy (Table 16).

- Search area is the current CTU and the left CTU

- Full search

Table 15

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.4 | 5.4 | 0.0% | 32.8 | 32.8 | -0.1% | 50.6 | 50.6 | 0.0% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 14.2 | 14.3 | 0.0% | 144.5 | 145.0 | -0.2% | 565.1 | 565.2 | 0.0% |
| SC YUV 444 | 17.0 | 17.0 | -0.1% | 190.8 | 191.3 | -0.2% | 520.8 | 521.9 | -0.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 107% |  |  | 97% |  |  | 100% |  |  |

Table 16

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.1% | -1.3% | -1.4% | -1.1% | -1.3% | -1.4% | -1.1% | -1.3% | -1.3% |
| SC YUV 444 | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -1.1% | -1.2% | -1.2% | -1.0% | -1.2% | -1.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | 0.4% | -0.1% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | -0.1% | 0.1% |  |  |  |
| SC RGB 444 | -0.6% | -0.8% | -0.8% | -0.9% | -1.0% | -1.1% |  |  |  |
| SC YUV 444 | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

## Performance of Motion Vector Predictor

Performance of the MV coding method 2 versus MV coding method 1. Results for lossless (Table 17) and lossy (Table 18).

- Search area is the current CTU and the left CTU

- Full search

Table 17

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.5 | 5.5 | -0.3% | 34.0 | 34.4 | -0.4% | 52.8 | 53.4 | -0.4% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 15.0 | 15.1 | -0.5% | 151.4 | 152.6 | -0.4% | 577.2 | 581.4 | -0.3% |
| SC YUV 444 | 18.2 | 18.5 | -0.7% | 205.3 | 207.0 | -0.4% | 545.1 | 558.6 | -0.9% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 92% |  |  | 106% |  |  | 97% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  | 99% |  |  |

Table 18

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.1% | -1.0% | -1.1% | -0.8% | -0.8% | -0.8% | -0.6% | -0.6% | -0.7% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.7% | -2.8% | -2.8% | -2.5% | -2.5% | -2.5% | -2.3% | -2.3% | -2.3% |
| SC YUV 444 | -2.2% | -2.3% | -2.3% | -1.8% | -1.9% | -1.8% | -1.5% | -1.5% | -1.4% |
| RangeExt | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  | 92% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.9% | -0.9% | -1.0% | -0.7% | -0.8% | -0.8% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -2.2% | -2.2% | -2.2% | -1.9% | -1.9% | -1.9% |  |  |  |
| SC YUV 444 | -2.0% | -2.1% | -2.0% | -1.6% | -1.6% | -1.5% |  |  |  |
| RangeExt | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.6% | -0.8% | 0.0% | -0.4% | -0.4% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.7% | -1.6% | -1.6% | -1.8% | -1.6% | -1.6% |  |  |  |
| SC YUV 444 | -1.6% | -1.6% | -1.5% | -1.2% | -1.1% | -1.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |  |  |  |

# Conclusions

A 2-D Intra MC method is proposed. The method achieves better performance than the 1-D method, while it is better aligned with the HEVC inter method, uses no interpolation filter and reduces the additional storage to be at most one CTU.

# References

- M. Budagavi, D.-K. Kwon, “AHG8: Video coding using Intra motion compensation”, JCTVC-M0350, Incheon, KR, April 2013.

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Pipeline Friendly Intra Motion Compensation”, JCTVC-N0254, Vienna, AT, August 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, KR, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).