| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0258 |
| --- | --- |

| Title: | Non-RCE2: Extension of TU-Based Inter RDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document an extension of TU-based inter RDPCM method is proposed. In the method, when the residual at the first row or column in each TU is not at the boundary of the corresponding CU, it is predicted using the residual in the neighboring TU. Comparing with [2], bit-rate reduction of up to 0.4% and 0.7% is obtained for random access and low delay settings for lossless case.

# Introduction

In [1], an inter RDPCM method is proposed for lossless coding. In inter RDPCM, three coding modes are used: no RDPCM, vertical RDPCM and horizontal RDPCM.

In [2], the coding mode is explicitly signalled for each transform unit. For vertical RDPCM, prediction is applied to the residuals except for those in the first row. Similarly, for horizontal RDPCM prediction is applied to the residuals except for those in the first column. In this contribution, the residual prediction is extended to the first row/column provided the first row/column is not at the boundary of a CU.

# Proposed Technique

Consider a block of size N × N. Let ri,j and ri,j, 0≤i≤N−1, 0≤j≤N−1 denote the motion compensated residuals and residuals after RDPCM, respectively. In the proposed method, when the residuals in the first row (or column) in each TU is not at the boundary of the corresponding CU, they are predicted using the residuals in the neighboring TU.

Let (i, j) and (i', j') be the residual position in the TU and CU respectively. Then for vertical RDPCM, ri,jis obtained as

ri,j=ri,j,i'= 0, i=0, 0≤j≤(N−1)ri,j−r(i−1),j,i'≠ 0, 0≤i≤(M−1), 0≤j≤(N−1).

For horizontal RDPCM,

ri,j=ri,j,j'=0, 0≤i≤(M−1), j=0ri,j−ri,(j−1)j'≠0, 0≤i≤(M−1), 0≤j≤(N−1).

# Experimental Results

The proposed method is implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions Error: Reference source not found. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. The performance is compared to the anchor (Table 1) and [2] (Table 2) for lossless configuration in terms of BD-rate savings.

Table 1 Proposed method vs HM10.1_Rext3.0 for lossless

|  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  |
| Class F | 31.7 | 33.4 | -3.8% | 49.8 | 54.7 | -6.1% |
| Class B | 2.6 | 2.6 | -0.5% | 2.6 | 2.6 | -0.6% |
| SC RGB 444 | 100.4 | 102.6 | -3.4% | 381.6 | 404.3 | -6.1% |
| SC YUV 444 | 128.9 | 131.1 | -1.9% | 325.6 | 342.1 | -4.4% |
| RangeExt | 2.5 | 2.5 | -0.2% | 2.5 | 2.5 | -0.3% |

Table 2 Proposed method vs [2] for lossless

|  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  |
| Class F | 33.2 | 33.4 | -0.4% | 54.3 | 54.7 | -0.7% |
| Class B | 2.6 | 2.6 | -0.1% | 2.6 | 2.6 | -0.1% |
| SC RGB 444 | 102.6 | 102.6 | -0.1% | 403.5 | 404.3 | -0.4% |
| SC YUV 444 | 131.3 | 131.1 | 0.1% | 342.3 | 342.1 | -0.2% |
| RangeExt | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |

The performance is compared to the anchor (Table 3) and [2] (Table 4) for lossy configuration in terms of BD-rate savings.

Table 3 Proposed method vs HM10.1_Rext3.0 for lossy

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.1% | -1.1% | -1.2% | -1.2% | -1.3% | -1.3% |
| SC YUV 444 | 0.0% | -0.2% | -0.2% | 0.1% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Class F | -0.3% | -0.2% | -0.7% | -0.4% | -0.4% | -0.7% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| SC RGB 444 | -3.0% | -3.1% | -3.2% | -3.3% | -3.3% | -3.2% |
| SC YUV 444 | -1.1% | -1.1% | -1.3% | -1.2% | -1.3% | -1.5% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 4 Proposed method vs [2] for lossy

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Class F | -0.1% | 0.0% | -0.4% | -0.2% | -0.1% | -0.4% |
| Class B | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusions

In this document an extension of the TU-based inter RDPCM method is proposed. The RDPCM is extended to the first column/row of each TU if the first column/row is not at the boundary of the CU.

# References

- M. Naccari, A. Gabriellini, M. Mrak, S. Blasi, E. Izquierdo, “Inter-Prediction Residual DPCM”, JCTVC-M0442, Incheon, KR, April 2013.

- M. Naccari, M. Mrak, “RCE2: Experimental results for Test C.1”, JCTVC-N0074, Vienna, AT, August 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).