| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0259 |
| --- | --- |

| Title: | SCE2 Cross check report of test 2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution reports the cross check results for SCE2 test 2.2. The proposed test evaluates the performance of modifying motion data compression and memory reduction (based on JCTVC-M0141). The proponents integrated the tools within SHM-2.0 and the source code modifications were found to be consistent with the tool description provided in SCE2 description document JCTVC-M1102. The average luma BD rate change measured for SHM-2.0 anchors is: -0.4%, 0.0%, 0.0% -0.4%, 0.0%, 0.1%, -0.3%, 0.0% and 0.1% for encoder configurations RA_2x, RA_1.5x, RA_SNR, LB_2x, LB_1.5x, LB_SNR, LD_2x, LD_1.5x and LD_SNR respectively.

# Introduction

SCE2 test 2.2 corresponds to the following tool in [2]:

## Motion data compression and memory reduction (JCTVC-M0141)

2-stage motion data compression as follows: first BL motion data is compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

The following macro settings were used in the software distributed amongst the SCE2 participants:

#define M0141_MOTION_COMP                1

#if M0141_MOTION_COMP

#define HALF_COMP                        1

#endif

# Experimental results

Below is a summary BD rate results for test 2.2 with SHM-2.0 anchors. The experimental setup corresponds to the common test condition [3] provided by AhG10 with additionally included SNR test case. Tests corresponding to all-intra were not carried out. The detailed results can be found in the attached excel sheet.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.1% | -1.1% |  |  |  | 0.1% | 0.1% | 0.2% |
| Class B | -0.3% | -0.4% | -0.4% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall (Test vs Ref) | -0.4% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 18.7% | 32.4% | 31.1% | 16.2% | 28.8% | 29.1% | 14.5% | 32.2% | 34.3% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.5% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Dec Time[%] | 98.6% |  |  | 98.8% |  |  | 98.8% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.0% | -1.0% |  |  |  | 0.1% | 0.2% | 0.2% |
| Class B | -0.3% | -0.3% | -0.3% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.4% | -0.5% | -0.5% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 28.0% | 38.3% | 39.0% | 24.7% | 33.1% | 36.0% | 24.4% | 34.9% | 39.6% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.6% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
| Dec Time[%] | 99.1% |  |  | 98.9% |  |  | 98.0% |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.9% | -0.9% |  |  |  | 0.1% | 0.2% | 0.1% |
| Class B | -0.3% | -0.3% | -0.5% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.3% | -0.4% | -0.6% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 26.1% | 37.4% | 38.3% | 22.8% | 32.7% | 35.5% | 23.5% | 34.8% | 39.5% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.7% | -0.1% | -0.1% | -0.1% | 0.1% | 0.2% | 0.1% |
| Dec Time[%] | 98.3% |  |  | 98.9% |  |  | 99.7% |  |  |

# Conclusion

The source code modifications corresponding to test 1.1 were studied and found to be consistent with the tool description provided in SCE2 description document JCTVC-M1102. The average luma BD rate change measured for SHM-2.0 anchors is: -0.4%, 0.0%, 0.0% -0.4%, 0.0%, 0.1%, -0.3%, 0.0% and 0.1% for encoder configurations RA_2x, RA_1.5x, RA_SNR, LB_2x, LB_1.5x, LB_SNR, LD_2x, LD_1.5x and LD_SNR respectively.

# References

[1] K. Sato, J. Xu, J. Park, B Jeon, “SCE5: Result of SCE5.2.2”, JCTVC-M0141, 13th JCTVC Meeting: Incheon, KR, 18–26 April 2013

[2] K. Sato, C. Gisquet, “Description of Core Experiment SCE2: Combination of inter-layer syntax prediction and motion data compression,” JCTVC-M1102, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.

[3] X. Li, J. Boyce, P. Onno, and Y.Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model,” JCTVC-M1009, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.