| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0260 |
| --- | --- |

| Title: | Cross check report of JCTVC-N0185 on low-delay flag checking process of SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution reports the cross check results for JCTVC-N0185 which proposes a modification to the low-delay flag checking process of SHVC. The proponents integrated their proposed change within SHM-2.0. The source code modifications were found to be consistent with the algorithm description provided in JCTVC-N0185. The average luma BD rate change measured for SHM-2.0 anchors is: -0.4%, -0.4%, -0.5% for encoder configurations RA_2x, RA_1.5x, RA_SNR, respectively.

# Introduction

JCTVC-N0185 [1] proposes that the low-delay flag in SHM-2.0 be set to true if the inter-layer picture is selected as the collocated picture for the temporal vector prediction process. The asserted reason for this change is that stronger correlation exists for motion information within the same reference picture list between current picture and the reference layer picture.

# Experimental results

Below is a summary BD rate results for the proposed change with SHM-2.0 anchors. The experimental setup corresponds to the random access test condition in [2] provided by AhG10. The detailed results can be found in the attached excel sheet.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.5% | -0.5% |  |  |  | -0.4% | -0.8% | -0.9% |
| Class B | -0.4% | -0.7% | -0.7% | -0.4% | -0.7% | -0.7% | -0.5% | -1.0% | -1.1% |
| Overall (Test vs Ref) | -0.4% | -0.6% | -0.7% | -0.4% | -0.7% | -0.7% | -0.5% | -0.9% | -1.0% |
| Overall (Test vs single layer) | 18.7% | 32.4% | 31.1% | 15.7% | 28.0% | 28.2% | 13.9% | 30.9% | 32.8% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.6% | -0.9% | -0.9% | -0.6% | -0.9% | -1.0% | -0.7% | -1.2% | -1.3% |

# Conclusion

The source code modifications were studied and found to be consistent with the algorithm description provided in the contribution document JCTVC-N0185. The average luma BD rate change measured for SHM-2.0 anchors is: -0.4%, -0.4%, -0.5% for encoder configurations RA_2x, RA_1.5x, RA_SNR, respectively.

# References

[1] X. Xiu, Y. Ye, Y. He, Y. He, Y. Lin, X. Zheng, X. Chen, “On low-delay flag checking process of SHVC,” JCTVC-M0185, 14th JCTVC Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[2] X. Li, J. Boyce, P. Onno, and Y.Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model,” JCTVC-M1009, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.