| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0261 M29907 |
| --- | --- |

| Title: | AhG5: Memory Bandwidth Reduction for HEVC RExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Vadim Seregin, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

HEVC Range Extensions 4:4:4 potential applications include consumer applications, so the complexity increase due to the 4:4:4 processing has to be taken into account. The scope of this contribution is the memory bandwidth increase for 4:4:4 motion estimation and compensation, which is around 37% more than the 4:2:0 case for 4×2 memory blocks. Specifically, in this contribution, restriction of bi-directional prediction for 8×8 chroma components is studied, while luma component still can have bi-directional prediction. This modification increases 4:4:4 bandwidth by 8%, as opposed to the current 37% with respect to 4:2:0. The impact on coding performance is 0.2% and 0.4% luma BD-rate loss for RA and LB, respectively, in YUV 4:4:4. In addition, restriction of motion interpolation is also studied. When the vertical motion interpolation is disallowed for chroma 8×8 PU, the bandwidth is increased by 13% with respect to 4:2:0 with the coding efficiency loss of 0.1% and 0.2% for RA and LB, respectively, in YUV 4:4:4.

# Introduction

High Efficiency Video Coding (HEVC) version 1 has been developed mainly targeting video in the 4:2:0 chroma format, while the HEVC Range Extensions (RExt) considers video in 4:2:2 and 4:4:4 chroma formats as well. As the block size of chroma components has been increased, the memory bandwidth to access reference pixels for inter-prediction is increased as well. Accordingly, the implementation complexity of HEVC RExt is increased, which can be a problem, especially for consumer applications.

This problem has been analysed in detail in the previous contribution, JCTVC-M0298 [1]. The same idea is implemented and further analysed on the top of HM-10.1_RExt3.0 and the results are reported in this contribution.

In addition, we propose a new method to achieve bandwidth reduction by restriction of motion interpolation.

Table 1 compares memory bandwidth in 4:2:0 and 4:4:4 formats in the current HM, where the analysis is performed based on the method in [2]. JCTVC-M0298 includes further details.

Table 1. Comparison of memory bandwidth in 4:2:0 and 4:4:4 formats.

| Memory configuration | 4:2:0 | 4:4:4 | Ratio to 4:2:0 |
| --- | --- | --- | --- |
| 4×2 | 15.0 | 20.5 | 137% |
| 8×2 | 18.0 | 24.0 | 133% |

# Proposed Methods

To reduce memory bandwidth for inter-prediction two approaches are proposed. In the first approach bi-directional prediction is restricted. In the second approach motion interpolation is restricted. Each method is described in detail in below.

## Approach 1: Bandwidth reduction with uni-directional prediction

As analyzed in JCTVC-M0298, the bandwidth worst case is 8×8 PU. By disallowing bi-directional prediction for 8×8 PUs, the bandwidth is reduced by 21% in case of 4×2 memory configuration with respect to the current HM. Note that this method can be applied to all color components (as in Test 1) or only for chroma components (as in Test 2).

While both Test 1 and Test 2 achieve the same amount of worst-case bandwidth reduction, the coding efficiency is better in Test 2.

Table 2 compares memory bandwidth in the current HM and Test 1 & 2.

Table 2. Memory bandwidth of Test 1 & 2 compared to HM.

| Memory configuration | HM | Test 1 & 2 | Ratio to HM | Ratio to 4:2:0 |
| --- | --- | --- | --- | --- |
| 4×2 | 20.5 | 16.3 | 79% | 108% |
| 8×2 | 24.0 | 21.0 | 88% | 117% |

When restriction of bi-directional prediction is applied, the motion vector associated with the reference picture list RefPicList0 is always selected. However, when weighted prediction is enabled, reference picture list selection can be done according to the magnitude of the weight applied to each direction. Specifically, when the weight applied to RefPicList1 is larger than that applied to RefPicList0, the motion vector associated with RefPicList1 is used. Otherwise, RefPicList0 is used. This particular scheme to deal with weighted prediction has been tested and the results are included as Test 2a.

## Approach 2: Bandwidth reduction with restriction in motion interpolation

Another way to reduce bandwidth is to restrict motion interpolation. As the motion is interpolated in the horizontal and vertical directions, it is possible to use the integer motion only in one direction. Also, this restriction can be applied to all color components or only to chroma components:

- In Test 3-1, when the PU size is 8×8, the motion interpolation is applied only in the horizontal direction for all color components.

- In Test 3-2, when the PU size is 8×8, for chroma components the motion interpolation is applied only in the horizontal direction.

- In Test 3-3, when the PU size is 8×8, the motion interpolation is not applied in either direction, and for the other PUs except 16×16, the motion interpolation is performed only in the horizontal direction. When PU is 16×16, the motion interpolation is performed in both directions.

Table 3, 4, and 5 show the bandwidth reduction in Test 3-1, 3-2, and 3-3, respectively. Note that Test 3-3 results in almost the same bandwidth as in the 4:2:0 format.

No change in the entropy coding part is proposed. Therefore, the motion vectors coded and stored in the current block can have all the possible precisions such as integer, half-pel, and quarter-pel. However, when the motion interpolation is not applied, the motion vector component is rounded to the closest integer at motion compensation stage for the restricted color components.

Table 3. Memory bandwidth of Test 3-1 compared to HM.

| Memory configuration | HM | Test 3-1 | Ratio to HM | Ratio to 4:2:0 |
| --- | --- | --- | --- | --- |
| 4×2 | 20.5 | 16.3 | 79% | 108% |
| 8×2 | 24.0 | 21.0 | 88% | 117% |

Table 4. Memory bandwidth of Test 3-2 compared to HM.

| Memory configuration | HM | Test 3-2 | Ratio to HM | Ratio to 4:2:0 |
| --- | --- | --- | --- | --- |
| 4×2 | 20.5 | 17.0 | 83% | 113% |
| 8×2 | 24.0 | 21.0 | 88% | 117% |

Table 5. Memory bandwidth of Test 3-3 compared to HM.

| Memory configuration | HM | Test 3-3 | Ratio to HM | Ratio to 4:2:0 |
| --- | --- | --- | --- | --- |
| 4×2 | 20.5 | 15.0 | 73% | 100% |
| 8×2 | 24.0 | 18.5 | 77% | 103% |

# Experimental Results

To evaluate the coding efficiency of the proposed methods, they are implemented on top of HM-10.1_RExt3.0, and experiments are performed using the common test conditions [3].

Experiments are performed except for all intra coding case, where no change is proposed. The tables below are the summary results of YUV 4:4:4 and 4:2:2 cases. Full results can be found in the accompanying spread-sheets.

For the first approach, additional experiments are performed, where the fading sequences are coded with the weighted prediction enabled to verify the impact of the proposed scheme. The fading sequences are generated as described in [4]. Note that both fading to black and white cases have been tested. Both cases show similar performance, and only fading to black is summarized in the tables below. Full results can be found in the spreadsheets.

Test 2a is also tested, and the summary results are given in Tables 16 and 17. Full results can be found in the spreadsheets.

Tables 10-20 present summary pages of all spreadsheets. Note that the weighted prediction is not tested with YUV 4:2:2 as it is not currently supported in HM10.1_RExt3.0.

Table 6. Summary results of Random Access configuration (YUV 4:4:4).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Test 1 | 0.7% | 0.8% | 0.8% | 1.0% | 0.9% | 1.2% |
| Test 2 | 0.2% | 0.7% | 0.8% | 0.4% | 0.8% | 1.1% |
| Test 1 WP | 0.6% | 0.6% | 0.6% | 0.8% | 0.8% | 0.9% |
| Test 2 WP | 0.1% | 0.5% | 0.5% | 0.2% | 0.6% | 0.9% |
| Test 3-1 | 0.5% | 0.4% | 0.5% | 0.7% | 0.5% | 0.7% |
| Test 3-2 | 0.1% | 0.2% | 0.3% | 0.2% | 0.2% | 0.5% |
| Test 3-3 | 1.2% | 2.8% | 3.3% | 1.7% | 2.3% | 3.4% |

Table 7. Summary results of Low delay B configuration (YUV 4:4:4).

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Test 1 | 0.6% | 0.6% | 0.5% | 0.9% | 0.7% | 0.8% |
| Test 2 | 0.2% | 0.3% | 0.3% | 0.4% | 0.4% | 0.5% |
| Test 1 WP | 0.3% | 0.3% | 0.4% | 0.6% | 0.6% | 0.7% |
| Test 2 WP | 0.1% | 0.4% | 0.3% | 0.3% | 0.5% | 0.5% |
| Test 3-1 | 0.5% | 0.4% | 0.4% | 0.7% | 0.4% | 0.6% |
| Test 3-2 | 0.2% | 0.2% | 0.4% | 0.3% | 0.2% | 0.5% |
| Test 3-3 | 2.0% | 4.3% | 5.8% | 2.1% | 3.1% | 4.8% |

Table 8. Summary results of Random Access configuration (YUV 4:2:2).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Test 1 | 0.6% | 0.6% | 0.6% | 0.9% | 0.7% | 1.0% |
| Test 2 | 0.2% | 0.5% | 0.6% | 0.4% | 0.5% | 0.8% |
| Test 3-1 | 0.6% | 0.4% | 0.5% | 0.7% | 0.4% | 0.7% |
| Test 3-2 | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.3% |
| Test 3-3 | 0.5% | 1.5% | 1.8% | 0.7% | 1.1% | 1.9% |

Table 9. Summary results of Low delay B configuration (YUV 4:2:2).

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Test 1 | 0.7% | 0.4% | 0.6% | 0.9% | 0.6% | 1.0% |
| Test 2 | 0.2% | 0.1% | 0.1% | 0.3% | 0.2% | 0.5% |
| Test 3-1 | 0.7% | 0.5% | 0.6% | 0.7% | 0.5% | 0.7% |
| Test 3-2 | 0.1% | 0.2% | 0.3% | 0.2% | 0.1% | 0.4% |
| Test 3-3 | 0.8% | 3.4% | 3.8% | 1.0% | 2.0% | 3.0% |

Table 10. Results of Test 1.

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.4% | 0.9% | 1.3% | 1.6% | 0.8% | 1.4% |
| YCbCr 4:4:4 | 0.7% | 0.8% | 0.8% | 1.0% | 0.9% | 1.2% |
| YCbCr 4:2:2 | 0.6% | 0.6% | 0.6% | 0.9% | 0.7% | 1.0% |
| Overall | 1.0% | 0.8% | 0.9% | 1.2% | 0.8% | 1.2% |
|  | 0.9% | 0.6% | 0.9% | 1.2% | 0.6% | 1.2% |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.4% | 0.9% | 1.2% | 1.4% | 0.8% | 1.3% |
| YCbCr 4:4:4 | 0.6% | 0.6% | 0.5% | 0.9% | 0.7% | 0.8% |
| YCbCr 4:2:2 | 0.7% | 0.4% | 0.6% | 0.9% | 0.6% | 1.0% |
| Overall | 0.9% | 0.7% | 0.8% | 1.1% | 0.7% | 1.0% |
|  | 0.9% | 1.0% | 0.8% | 1.1% | 0.6% | 1.1% |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |

Table 11. Results of Test 1 WP (fading to black).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.0% | 0.8% | 1.0% | 1.4% | 0.9% | 1.3% |
| YCbCr 4:4:4 | 0.6% | 0.6% | 0.6% | 0.8% | 0.8% | 0.9% |
| Overall | 0.8% | 0.7% | 0.8% | 1.1% | 0.8% | 1.1% |
|  | 0.8% | 0.7% | 0.8% | 1.1% | 0.9% | 1.1% |
| Enc Time[%] | 94% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.9% | 0.8% | 1.0% | 1.1% | 0.8% | 1.2% |
| YCbCr 4:4:4 | 0.3% | 0.3% | 0.4% | 0.6% | 0.6% | 0.7% |
| Overall | 0.6% | 0.6% | 0.7% | 0.9% | 0.7% | 1.0% |
|  | 0.6% | 0.6% | 0.7% | 0.9% | 0.7% | 1.0% |
| Enc Time[%] | 90% |  |  | 90% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 12. Results of Test 1 WP (fading to white).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.1% | 0.9% | 1.1% | 1.4% | 0.9% | 1.3% |
| YCbCr 4:4:4 | 0.6% | 0.5% | 0.5% | 0.8% | 0.7% | 0.8% |
| Overall | 0.8% | 0.7% | 0.8% | 1.1% | 0.8% | 1.1% |
|  | 0.8% | 0.7% | 0.8% | 1.1% | 0.8% | 1.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.9% | 0.8% | 1.0% | 1.1% | 0.8% | 1.1% |
| YCbCr 4:4:4 | 0.4% | 0.4% | 0.6% | 0.6% | 0.5% | 0.8% |
| Overall | 0.7% | 0.6% | 0.8% | 0.9% | 0.7% | 1.0% |
|  | 0.7% | 0.6% | 0.8% | 0.9% | 0.7% | 1.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 13. Results of Test 2.

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.2% | 1.0% | 1.4% | 1.3% | 0.9% | 1.5% |
| YCbCr 4:4:4 | 0.2% | 0.7% | 0.8% | 0.4% | 0.8% | 1.1% |
| YCbCr 4:2:2 | 0.2% | 0.5% | 0.6% | 0.4% | 0.5% | 0.8% |
| Overall | 0.5% | 0.8% | 1.0% | 0.7% | 0.7% | 1.1% |
|  | 0.5% | 0.7% | 1.0% | 0.7% | 0.5% | 1.1% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.3% | 1.0% | 1.3% | 1.3% | 0.8% | 1.3% |
| YCbCr 4:4:4 | 0.2% | 0.3% | 0.3% | 0.4% | 0.4% | 0.5% |
| YCbCr 4:2:2 | 0.2% | 0.1% | 0.1% | 0.3% | 0.2% | 0.5% |
| Overall | 0.6% | 0.5% | 0.6% | 0.7% | 0.5% | 0.8% |
|  | 0.6% | 0.7% | 0.6% | 0.7% | 0.4% | 0.8% |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |

Table 14. Results of Test 2 WP (fading to black).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.9% | 1.0% | 1.2% | 1.2% | 0.9% | 1.4% |
| YCbCr 4:4:4 | 0.1% | 0.5% | 0.5% | 0.2% | 0.6% | 0.9% |
| Overall | 0.5% | 0.8% | 0.9% | 0.8% | 0.8% | 1.2% |
|  | 0.5% | 0.7% | 0.9% | 0.8% | 0.8% | 1.2% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.1% | 1.0% | 1.1% | 1.2% | 0.9% | 1.3% |
| YCbCr 4:4:4 | 0.1% | 0.4% | 0.3% | 0.3% | 0.5% | 0.5% |
| Overall | 0.6% | 0.7% | 0.8% | 0.8% | 0.7% | 0.9% |
|  | 0.6% | 0.7% | 0.8% | 0.8% | 0.7% | 1.0% |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

Table 15. Results of Test 2 WP (fading to white).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.0% | 1.1% | 1.3% | 1.3% | 1.0% | 1.5% |
| YCbCr 4:4:4 | 0.1% | 0.6% | 0.5% | 0.3% | 0.7% | 0.8% |
| Overall | 0.6% | 0.9% | 0.9% | 0.8% | 0.9% | 1.1% |
|  | 0.6% | 0.9% | 0.9% | 0.8% | 1.0% | 1.1% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.1% | 1.0% | 1.1% | 1.2% | 0.9% | 1.3% |
| YCbCr 4:4:4 | 0.2% | 0.4% | 0.3% | 0.3% | 0.5% | 0.6% |
| Overall | 0.7% | 0.7% | 0.7% | 0.8% | 0.7% | 1.0% |
|  | 0.7% | 0.7% | 0.7% | 0.8% | 0.7% | 1.0% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 16. Results of Test 2a WP (fading to black).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.9% | 0.9% | 1.2% | 1.2% | 0.9% | 1.4% |
| YCbCr 4:4:4 | 0.1% | 0.6% | 0.5% | 0.3% | 0.7% | 0.8% |
| Overall | 0.5% | 0.8% | 0.8% | 0.8% | 0.8% | 1.1% |
|  | 0.5% | 0.8% | 0.8% | 0.8% | 0.9% | 1.1% |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 110% |  |  | 110% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.0% | 0.9% | 1.1% | 1.2% | 0.9% | 1.3% |
| YCbCr 4:4:4 | 0.1% | 0.6% | 0.3% | 0.2% | 0.7% | 0.5% |
| Overall | 0.6% | 0.8% | 0.7% | 0.7% | 0.8% | 0.9% |
|  | 0.6% | 0.7% | 0.7% | 0.8% | 0.7% | 1.0% |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 17. Results of Test 2a WP (fading to white).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.0% | 1.1% | 1.3% | 1.3% | 1.0% | 1.5% |
| YCbCr 4:4:4 | 0.1% | 0.5% | 0.5% | 0.3% | 0.7% | 0.8% |
| Overall | 0.6% | 0.8% | 0.9% | 0.8% | 0.8% | 1.1% |
|  | 0.6% | 0.8% | 0.9% | 0.8% | 0.9% | 1.1% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.1% | 1.0% | 1.2% | 1.2% | 0.9% | 1.3% |
| YCbCr 4:4:4 | 0.1% | 0.6% | 0.4% | 0.2% | 0.6% | 0.6% |
| Overall | 0.6% | 0.8% | 0.8% | 0.7% | 0.7% | 1.0% |
|  | 0.6% | 0.8% | 0.8% | 0.8% | 0.7% | 1.0% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 18. Results of Test 3-1.

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.0% | 0.5% | 0.8% | 1.0% | 0.5% | 0.8% |
| YCbCr 4:4:4 | 0.5% | 0.4% | 0.5% | 0.7% | 0.5% | 0.7% |
| YCbCr 4:2:2 | 0.3% | 0.3% | 0.3% | 0.4% | 0.3% | 0.5% |
| Overall | 0.6% | 0.4% | 0.5% | 0.7% | 0.4% | 0.7% |
|  | 0.6% | 0.4% | 0.6% | 0.7% | 0.3% | 0.7% |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.1% | 0.7% | 1.0% | 1.0% | 0.6% | 0.9% |
| YCbCr 4:4:4 | 0.5% | 0.4% | 0.4% | 0.7% | 0.4% | 0.6% |
| YCbCr 4:2:2 | 0.5% | 0.3% | 0.4% | 0.5% | 0.4% | 0.6% |
| Overall | 0.7% | 0.5% | 0.6% | 0.7% | 0.5% | 0.7% |
|  | 0.7% | 0.7% | 0.6% | 0.8% | 0.4% | 0.7% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 109% |  |  | 109% |  |  |

Table 19. Results of Test 3-2.

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.9% | 0.5% | 0.9% | 0.9% | 0.4% | 0.8% |
| YCbCr 4:4:4 | 0.1% | 0.2% | 0.3% | 0.2% | 0.2% | 0.5% |
| YCbCr 4:2:2 | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.3% |
| Overall | 0.4% | 0.3% | 0.5% | 0.4% | 0.2% | 0.6% |
|  | 0.4% | 0.3% | 0.5% | 0.5% | 0.1% | 0.5% |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.5% | 0.9% | 1.3% | 1.2% | 0.7% | 1.1% |
| YCbCr 4:4:4 | 0.2% | 0.2% | 0.4% | 0.3% | 0.2% | 0.5% |
| YCbCr 4:2:2 | 0.1% | 0.2% | 0.3% | 0.2% | 0.1% | 0.4% |
| Overall | 0.7% | 0.5% | 0.7% | 0.6% | 0.4% | 0.7% |
|  | 0.6% | 0.7% | 0.7% | 0.6% | 0.3% | 0.7% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |

Table 20. Results of Test 3-3.

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 5.8% | 3.8% | 5.3% | 4.4% | 3.1% | 4.1% |
| YCbCr 4:4:4 | 1.2% | 2.8% | 3.3% | 1.7% | 2.3% | 3.4% |
| YCbCr 4:2:2 | 0.5% | 1.5% | 1.8% | 0.7% | 1.1% | 1.9% |
| Overall | 2.7% | 2.8% | 3.5% | 2.3% | 2.2% | 3.2% |
|  | 2.7% | 3.1% | 3.2% | 2.4% | 0.9% | 3.4% |
| Enc Time[%] | 109% |  |  | 109% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 7.4% | 5.0% | 6.2% | 5.4% | 4.0% | 5.0% |
| YCbCr 4:4:4 | 2.0% | 4.3% | 5.8% | 2.1% | 3.1% | 4.8% |
| YCbCr 4:2:2 | 0.8% | 3.4% | 3.8% | 1.0% | 2.0% | 3.0% |
| Overall | 3.6% | 4.3% | 5.3% | 3.0% | 3.1% | 4.3% |
|  | 3.6% | 4.4% | 5.3% | 2.9% | 3.2% | 4.3% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 109% |  |  | 108% |  |  |

# Conclusion

In this contribution, methods to reduce memory bandwidth for inter-prediction are proposed. In the first approach, the bi-prediction is restricted in 8×8 PU, which can be applied to all color components (Test 1) or only for chroma components (Test 2). In the second approach (Test 3), the motion interpolation is restricted, which can achieve essentially the same bandwidth as in 4:2:0 format.

Experimental results show that the reduced bandwidth is 79% of that of HM at the cost of a coding efficiency loss of about 0.2% for Luma in YUV 4:4:4 case for Test 2. In case of Test 3-2, it is 83% at the cost of 0.1% loss.

# References

- V. Seregin, X. Wang, J. Sole, and M. Karczewicz, “Bandwidth reduction for range extension,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M0298, Apr. 2013.

- E. François, A. Tabatabai, and E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0440, Jan. 2013.

- D. Flynn and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1006, Jan. 2013.

- P. Bordes, “Weighted prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-E041, Mar. 2011.

# Proposed Text Changes

## Approach 1: Bandwidth reduction with uni-directional prediction

### Test 1

8.5.3.2.1	Derivation process for luma motion vectors for merge mode

…

10. When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12 or equal to 16 and ChromaArrayType !=0, the following applies:

refIdxL1 = −1		(8-1)

predFlagL1 = 0		(8-2)

##### Table 7-1 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( nPbW + nPbH ) != 12 ¦¦ ( ( nPbW + nPbH ) > 16 && ChromaArrayType !=0 ) | ( nPbW + nPbH ) == 12 ¦¦ (( nPbW + nPbH ) == 16 && ChromaArrayType !=0 ) |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | na |

### Test 2

8.5.3.3.1	General

…

For X being each of 0 and 1, when predFlagLX is equal to 1, the following applies:

–	The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and when ChromaArrayType is not equal to 0, two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause Error: Reference source not found with refIdxLX as input.

–	For X being equal to 1, and ChromaArrayType !=0, when predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nPbW + nPbH ) is equal to 16, the following applies:

refPicL1Cb = refPicL0Cb

refPicL1Cr = refPicL0Cr

mvCL1 = mvCL0

–	The array predSamplesLXL, and when ChromaArrayType is not equal to 0, the arrays predSamplesLXCb, and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xCb, yCb ) and ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH, the motion vectors mvLX and, when ChromaArrayType is not equal to 0, mvCLX, and the reference arrays refPicLXL, refPicLXCb, and refPicLXCr as inputs.

…

## Approach 2: Bandwidth reduction with restriction in motion interpolation

### Test 3-2

8.5.3.3.1	General

…

xFracC = mvLX[ 0 ] & 7		(8-3)

If ChromaArrayType != 0 and ( nPbW + nPbH ) is equal to 16,

If xFracC == 3,

xIntC ++

xFracC = 0

- Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).