| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0262 |
| --- | --- |

| Title: | Cross check report of JCTVC-N0233 scaled MV in motion field buffer update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution reports the cross check results for JCTVC-N0233 which proposes a modification to motion field buffer update mechanism tested in SCE2. Specifically, JCTVC-N0233 proposes to re-direct references to non-existing reference pictures to existing reference pictures in current reference picture list. The asserted benefit of this change is that it prevents the appending of entries to the reference picture list of the resampled reference picture. The proponents integrated their proposed change within the SHM-2.0 based software distributed to SCE2 participants. The proposed modifications were tested with various tests defined in SCE2 which included the motion field buffer update tool. The source code modifications were found to be consistent with the algorithm description provided in JCTVC-N0233. The R-D results matched those provided by the proponents. The average luma BD rate impact of the proposed changes is found to be small for the various SCE2 tests.

# Introduction

JCTVC-N0233 [1] proposes to re-direct references to non-existing reference pictures to existing reference pictures in current reference picture list. The motion vector corresponding to the re-directed reference is scaled appropriately (based on POC values). The existing reference picture chosen for the re-direction corresponds to the first picture in the reference picture list. The asserted benefit of this change is that it prevents the appending of entries to the reference picture list of the resampled picture.

The changes were integrated within the SHM-2.0 based software distributed to SCE2 participants.

# Experimental results

Below is a summary BD rate results for the proposed change with SHM-2.0 anchors. The experimental setup corresponds to the test conditions specified in [2][3]. The detailed results can be found in the attached excel sheets.

## SCE2 test 1.3 + proposed changes

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -1.1% | -1.1% |  |  |  | -0.1% | 0.2% | 0.2% |
| Class B | -0.7% | -0.5% | -0.6% | -0.3% | -0.1% | -0.1% | -0.2% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.8% | -0.7% | -0.7% | -0.3% | -0.1% | -0.1% | -0.2% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 18.3% | 32.3% | 31.0% | 15.9% | 28.7% | 29.1% | 14.2% | 32.3% | 34.3% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.9% | -0.8% | -0.9% | -0.3% | -0.1% | -0.1% | -0.2% | 0.1% | 0.2% |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.0% | -1.0% |  |  |  | 0.1% | 0.2% | 0.2% |
| Class B | -0.3% | -0.3% | -0.3% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.4% | -0.5% | -0.5% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 28.0% | 38.3% | 39.0% | 24.7% | 33.1% | 36.0% | 24.4% | 34.9% | 39.6% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.6% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.9% | -0.9% |  |  |  | 0.1% | 0.2% | 0.1% |
| Class B | -0.3% | -0.3% | -0.5% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | -0.3% | -0.4% | -0.6% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 26.1% | 37.4% | 38.3% | 22.8% | 32.7% | 35.5% | 23.5% | 34.8% | 39.5% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.7% | -0.1% | -0.1% | -0.1% | 0.1% | 0.2% | 0.1% |

## SCE2 test 1.4 + proposed changes

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -1.1% | -1.1% |  |  |  | -0.1% | 0.0% | 0.1% |
| Class B | -0.7% | -0.5% | -0.6% | -0.3% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.8% | -0.7% | -0.7% | -0.3% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 18.3% | 32.3% | 31.0% | 15.9% | 28.7% | 28.9% | 14.2% | 32.2% | 34.2% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.9% | -0.8% | -0.9% | -0.3% | -0.1% | -0.2% | -0.2% | 0.0% | 0.1% |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.0% | -1.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | -0.3% | -0.3% | -0.3% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.4% | -0.5% | -0.5% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 28.0% | 38.3% | 39.0% | 24.7% | 33.1% | 35.8% | 24.3% | 34.7% | 39.5% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.6% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.9% | -0.9% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | -0.3% | -0.3% | -0.5% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.3% | -0.4% | -0.6% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 26.1% | 37.4% | 38.3% | 22.7% | 32.7% | 35.4% | 23.4% | 34.6% | 39.4% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -0.5% | -0.6% | -0.7% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |

## SCE2 test 2.3 + proposed changes

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.2% | -0.2% |  |  |  | -0.1% | 0.0% | 0.1% |
| Class B | -0.5% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.4% | -0.2% | -0.2% | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 18.7% | 33.1% | 31.7% | 15.9% | 28.8% | 29.1% | 14.2% | 32.2% | 34.2% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.5% | -0.2% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

## Comparison of average luma BD rate for SCE2 tests with and without the proposed changes

The summary results are listed below:

|  | With proposed changes | Without proposed changes |
| --- | --- | --- |
| SCE2 test 1.3 [Motion buffer update + Motion compression modification] | 0.0%, 0.0%, −0.8%, −0.3%, −0.2%, −0.4%, 0.0%, 0.1%, −0.3%, 0.0%, 0.1% For AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LDB_2x, LDB_1.5x, LDB_SNR, LDP_2x, LDP_1.5x, LDP_SNR | 0.0%, 0.0%, −0.8%, −0.3%, −0.2%, −0.4%, 0.0%, 0.1%, −0.3%, 0.0%, 0.1% For AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LDB_2x, LDB_1.5x, LDB_SNR, LDP_2x, LDP_1.5x, LDP_SNR (obtained from JCTVC-N0255) |
| SCE2 test 1.4 [Motion buffer update + Motion compression modification + Inter-layer motion information prediction] | −0.8%, −0.3%, −0.2%, −0.4%, −0.1%, 0.0%, −0.3%, −0.1%, 0.0% For RA_2x, RA_1.5x, RA_SNR, LDB_2x, LDB_1.5x, LDB_SNR, LDP_2x, LDP_1.5x, LDP_SNR | −0.8%, −0.3%, −0.2%, −0.4%, −0.1%, 0.1%, −0.3%, −0.1%, 0.0% For RA_2x, RA_1.5x, RA_SNR, LDB_2x, LDB_1.5x, LDB_SNR, LDP_2x, LDP_1.5x, LDP_SNR (obtained from JCTVC-N0241) |
| SCE2 test 2.3 [Motion buffer update] | 0.0%, 0.0%, −0.4%, −0.2%, −0.2%, 0.0%, 0.0%, 0.0%, 0.0%, 0.0%, 0.0% For AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LDB_2x, LDB_1.5x, LDB_SNR, LDP_2x, LDP_1.5x, LDP_SNR | 0.0%, 0.0%, −0.5%, −0.2%, −0.2%, 0.0%, 0.0%, 0.0%, 0.0%, 0.0%, 0.0% For AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LDB_2x, LDB_1.5x, LDB_SNR, LDP_2x, LDP_1.5x, LDP_SNR (obtained from JCTVC-N0252) |

From the above it may be concluded that the impact on coding efficiency of the proposed changes is negligible.

# Conclusion

The source code modifications were studied and found to be consistent with the algorithm description provided in the contribution document JCTVC-N0233. The R-D results match those provided by the proponents.

# References

[1] J. Xu, A. Tabatabai, K. Sato, “Non-SCE2: scaled MV in motion field buffer update,” JCTVC-N0233, 14th JCTVC Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[2] X. Li, J. Boyce, P. Onno, and Y.Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model,” JCTVC-M1009, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.

[3] K. Sato, C. Gisquet, “HEVC Scalable Extensions Core Experiment 2 (SCE2): Combination of inter-layer syntax prediction and motion data compression,” JCTVC-M1102, 13th JCTVC Meeting: Incheon, KR, 18–26 Apr. 2013.

[4] K. Misra, A. Segall, K. Sato, J. Xu, “SCE2 test 1.3 results,” JCTVC-N0255, 14th JCTVC Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[5] C. Gisquet, K. Sato, J. Xu, K. Misra, “SCE2: Result of Test 1.4,” JCTVC-N0241, 14th JCTVC Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[6] K. Misra, A. Segall, J. Zhao, “SCE2 test 2.3 motion buffer modification results,” JCTVC-N0252, 14th JCTVC Meeting: Vienna, AT, 25 July – 2 Aug. 2013.