| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0265 |
| --- | --- |

| Title: | AHG13: Complexity reduction of the up-sampling process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Rickard Sjöberg Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

Multi-loop scalable coding has a significant impact on average decoder complexity. To make it possible for applications to trade off performance versus complexity it is proposed to enable use of 4-tap filters in addition to the current 8-tap filters for luma up-sampling.

The effect in comparison to SHM-2.0 when always deploying 4-tap filters for luma up-sampling is for common conditions a BDR loss of 0.9% for luma with a 20% reduction in average number of multiplications and additions for MC and block based up-sampling.

For coding with B or P pictures the BDR loss is 0.9% for 2x scalability and 0.4% for 1.5x scalability with a 10%/20% reduction on average of multiplications and additions for MC and up-sampling for respectively PU and picture based up-sampling. The effect on intra coding only is a BDR loss of 1.9% for 2x scalability and 1.1% for 1.5x scalability with more than 40% reduction on average of multiplications and additions for both PU based and picture based up-sampling.

One example of trading complexity versus performance is also shown where 8-tap luma up-sampling filters is used for intra slices and proposed 4-tap luma up-sampling filters for other slices. This results in an average BDR loss of 0.4% for random access compared to SHM-2.0.

# Problem

Multi-loop scalable coding has a significant impact on average decoder complexity. Let’s assume that single layer HEVC has an average decoding time of 100% for the highest resolution, then the SHVC decoder has an average  decoding time of about 125% for 2x spatial scalability and for 1.5x the decoding time will be about 144%. A potential solution to reduce the average decoder complexity is to use up-sampling filters with fewer filter coefficients than currently used as described in following sections.

# Solution

It is proposed to enable a possibility to use 4-tap filters for luma up-sampling. Thereby luma and chroma can share same function/logic for up-sampling.

## Filter derivation for luma up-sampling

The 4-tap filter has been designed from the corresponding SHM-2.0 8-tap filters. The purpose is to enable re-use of 8-tap filter coefficients and to get a filter that behaves similar to the 8-tap filter.

Consider 8 tap filter: f_8tap = [f0, f1, f2, f3, f4, f5, f6, f7]

The 4 tap filter is derived as: f_4tap = [(f0+f1+ f2), f3, f4, (f5+f6+f7)]

| Phase shift | SHM-2.0 Luma up-sampling filter | Proposed 4 taps Luma up-sampling filter |
| --- | --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 | { 0, 64, 0, 0}/64 |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1}/64 | { -8, 52, 26, -6}/64 |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 | {-8, 40, 40, -8}/64 |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1}/64 | {-6, 26, 52, -8}/64 |

## Signalling of filter size for up-sampling

We propose to add one flag in SPS extension that indicates if the filter length for up-sampling is fixed for the whole sequence and another flag in both SPS extension and slice header  that selects 4-tap or 8-tap filter for luma up-sampling.

# Results

## BDR performance versus SHM-2.0[1]

The average performance loss for common conditions is 0.9% for luma and 0.6% for the chroma components when always deploying 4-tap filters for luma up-sampling. For inter coding the performance loss is 0.9% for 2x and 0.4% for 1.5x and for intra coding 1.9% for 2x and 1.1% for 1.5x. Encoding and decoding time are measured on a linux cluster is unreliable.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 3,4% | 2,5% | 2,5% |  |  |  |
| Class B | 1,3% | 1,3% | 1,2% | 1,1% | 1,1% | 1,1% |
| Overall (Test vs Ref) | 1,9% | 1,6% | 1,6% | 1,1% | 1,1% | 1,1% |
| Overall (Test vs single layer) | 14,9% | 16,8% | 16,4% | 11,8% | 11,1% | 10,6% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 3,2% | 3,0% | 3,0% | 2,6% | 2,8% | 2,8% |
| Enc Time[%] | 104,2% |  |  | 104,6% |  |  |
| Dec Time[%] | 108,7% |  |  | 109,7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1,9% | 0,6% | 0,7% |  |  |  |
| Class B | 0,7% | 0,4% | 0,2% | 0,4% | 0,3% | 0,3% |
| Overall (Test vs Ref) | 1,0% | 0,4% | 0,4% | 0,4% | 0,3% | 0,3% |
| Overall (Test vs single layer) | 20,5% | 33,9% | 32,4% | 16,6% | 29,2% | 29,5% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% |
| EL only (Test vs Ref) | 1,8% | 1,2% | 1,1% | 1,0% | 1,0% | 1,0% |
| Enc Time[%] | 108,2% |  |  | 108,7% |  |  |
| Dec Time[%] | 113,1% |  |  | 110,0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1,4% | 0,4% | 0,3% |  |  |  |
| Class B | 0,4% | 0,1% | 0,2% | 0,4% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,7% | 0,2% | 0,2% | 0,4% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 29,5% | 39,3% | 40,1% | 25,2% | 33,2% | 36,0% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% |
| EL only (Test vs Ref) | 1,2% | 0,7% | 0,8% | 0,7% | 0,5% | 0,5% |
| Enc Time[%] | 110,7% |  |  | 109,8% |  |  |
| Dec Time[%] | 116,2% |  |  | 113,0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1,6% | 0,4% | 0,3% |  |  |  |
| Class B | 0,6% | 0,2% | 0,1% | 0,6% | 0,2% | 0,2% |
| Overall (Test vs Ref) | 0,9% | 0,3% | 0,2% | 0,6% | 0,2% | 0,2% |
| Overall (Test vs single layer) | 27,7% | 38,4% | 39,4% | 23,5% | 33,0% | 35,8% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 1,5% | 0,9% | 0,8% | 1,2% | 0,8% | 0,8% |
| Enc Time[%] | 108,5% |  |  | 107,7% |  |  |
| Dec Time[%] | 110,2% |  |  | 106,9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

## Complexity assessment (multiplications and additions)[2]

Here it is shown how many multiplications and additions per pixel that is required on average using SHM-2.0 and how much it can be reduced in percent using the proposed 4-tap filter. For common conditions the proposed up-sampling filter can reduce the average number of multiplications and additions with about 20% for block based up-sampling and about 30% for picture based up-sampling.

For all intra scalability the proposed up-sampling can reduce the average number of multiplications and additions by more than 40% for both PU based and picture based processing. For inter coding using PU based processing the average computations can be reduced by about 10%. For inter coding using picture based processing the average computations can be reduced by more than 20%.

|  | SHM-2.0 | Proposed | SHM-2.0 | Proposed | SHM-2.0 | Proposed | SHM-2.0 | Proposed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mult Blk | reduction | mult Pic | reduction | add Blk | Reduction | add Pic | reduction |
|  | [op/pixel] | [%] | [op/pixel] | [%] | [op/pixel] | [%] | [op/pixel] | [%] |
| Intra 1.5x | 23,42 | -43,53 | 23,15 | -42,03 | 19,92 | -50,21 | 19,69 | -48,87 |
| Intra 2x | 23,47 | -43,56 | 25,90 | -41,29 | 19,96 | -50,23 | 22,02 | -48,22 |
| RA 1.5x | 25,05 | -7,30 | 36,62 | -18,56 | 21,24 | -8,49 | 31,07 | -21,61 |
| RA 2x | 22,06 | -5,97 | 37,05 | -20,71 | 18,69 | -6,94 | 31,42 | -24,16 |
| LDB 1.5x | 22,87 | -9,46 | 33,52 | -20,08 | 19,38 | -11,01 | 28,44 | -23,38 |
| LDB 2x | 19,77 | -7,19 | 34,35 | -22,18 | 16,74 | -8,35 | 29,14 | -25,87 |
| CC Avg | 22,77 | -19,50 | 31,76 | -27,48 | 19,32 | -22,54 | 26,96 | -32,02 |
| LDP 1.5x | 15,31 | -12,90 | 26,34 | -25,22 | 12,99 | -14,93 | 22,36 | -29,34 |
| LDP 2x | 13,63 | -9,19 | 28,60 | -26,50 | 11,55 | -10,59 | 24,28 | -30,86 |
| Avg | 20,70 | -17,39 | 30,69 | -27,07 | 17,56 | -20,09 | 26,05 | -31,54 |

## Memory access [3]

Here the pure memory access compared to SHM-2.0 is shown. For common conditions there is a 5% reduction in pure memory access for PU based processing. For intra there is a 10% reduction in pure memory access for PU based processing. For inter coding the memory access is only reduced by 1-2%.

|  | SHM-2.0 | Proposed | SHM-2.0 | Proposed |
| --- | --- | --- | --- | --- |
|  | pureBlk | reduction | purePic | reduction |
|  |  | [%] |  | [%] |
| Intra 1.5x | 6511056 | -12,2519 | 8773598 | -2,05677 |
| Intra 2x | 7235373 | -11,2789 | 10432990 | -1,60777 |
| RA 1.5x | 9674608 | -1,61679 | 14181257 | -1,10359 |
| RA 2x | 12560365 | -0,9056 | 17861909 | -0,67095 |
| LDB 1.5x | 9674608 | -1,61679 | 14181257 | -1,10359 |
| LDB 2x | 12560365 | -0,9056 | 17861909 | -0,67095 |
| CC Avg | 9702729 | -4,7626 | 13882153 | -1,20227 |
| LDP 1.5x | 7164954 | -2,25086 | 11622646 | -1,31579 |
| LDP 2x | 9395321 | -1,17233 | 14731693 | -0,86519 |
| Avg | 9347081 | -3,99985 | 13705907 | -1,17433 |

## BDR results for deploying 8-tap for I-slices and 4-tap for other slices

Here we make the encoder to select SHVC 8-tap filtering for I-slices and the proposed 4-tap filters for other slices. The average performance loss for common conditions is 0.3% for luma and 0.1% for the chroma components. For random access the BDR loss is reduced from 0.7% to 0.4%.  Encoding and decoding time are measured on a linux cluster is unreliable.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Test vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 104,5% |  |  | 104,4% |  |  |
| Dec Time[%] | 112,2% |  |  | 110,3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1,2% | 0,1% | 0,2% |  |  |  |
| Class B | 0,4% | 0,1% | 0,1% | 0,2% | 0,1% | 0,0% |
| Overall (Test vs Ref) | 0,6% | 0,1% | 0,1% | 0,2% | 0,1% | 0,0% |
| Overall (Test vs single layer) | 20,0% | 33,4% | 32,1% | 16,4% | 28,9% | 29,2% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% |
| EL only (Test vs Ref) | 1,2% | 0,6% | 0,6% | 0,5% | 0,4% | 0,4% |
| Enc Time[%] | 108,8% |  |  | 109,4% |  |  |
| Dec Time[%] | 111,3% |  |  | 110,7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1,3% | 0,2% | 0,3% |  |  |  |
| Class B | 0,4% | 0,1% | 0,2% | 0,3% | 0,1% | 0,2% |
| Overall (Test vs Ref) | 0,7% | 0,1% | 0,2% | 0,3% | 0,1% | 0,2% |
| Overall (Test vs single layer) | 29,4% | 39,2% | 40,0% | 25,2% | 33,2% | 36,1% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% |
| EL only (Test vs Ref) | 1,2% | 0,7% | 0,7% | 0,7% | 0,5% | 0,5% |
| Enc Time[%] | 110,8% |  |  | 111,5% |  |  |
| Dec Time[%] | 117,0% |  |  | 117,2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1,5% | 0,3% | 0,4% |  |  |  |
| Class B | 0,6% | 0,3% | 0,1% | 0,5% | 0,1% | 0,2% |
| Overall (Test vs Ref) | 0,8% | 0,3% | 0,2% | 0,5% | 0,1% | 0,2% |
| Overall (Test vs single layer) | 27,6% | 38,4% | 39,4% | 23,5% | 33,0% | 35,8% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 1,4% | 0,9% | 0,7% | 1,2% | 0,8% | 0,8% |
| Enc Time[%] | 109,5% |  |  | 108,8% |  |  |
| Dec Time[%] | 112,3% |  |  | 108,8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Proposed changes to the SHVC working draft

The changes to the working draft [4] is high-lighted in yellow.

G.7.3.2.2.1	Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_left_offset[ i ] | se(v) |
| scaled_ref_layer_top_offset[ i ] | se(v) |
| scaled_ref_layer_right_offset[ i ] | se(v) |
| scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| sps_luma_upsampling_filter_length_flag | u(1) |
| sps_luma_upsampling_filter_length_fixed_flag | u(1) |
| } |  |

- G.7.4.3.2.1 Sequence parameter set extension semantics

sps_luma_upsampling_filter_length_flag equal to 0 specifies that the length of the up-sampling filter is 8. When sps_luma_upsampling_filter_length_flag is 1 the length of the up-sampling filter is 4.

sps_luma_upsampling_filter_length_fixed_flag equal to 1 specifies that the luma up-sampling filter length is given by sps_luma_upsampling_filter_length_flag and is fixed for the whole CVS. When sps_luma_upsampling_filter_length_fixed_flag is 0 the length of the up-sampling filter can vary between 4 and 8.

- G7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| if( num_extra_slice_header_bits > 0 ) |  |
| discardable_flag | u(1) |
| for( i = 1; i < num_extra_slice_header_bits ; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| if(! sps_luma_upsampling_filter_fixed_flag ) |  |
| luma_upsampling_filter_length_flag | u(1) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( nuh_layer_id > 0 && NumMotionPredRefLayers[ nuh_layer_id ] > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumMotionPredRefLayers[ nuh_layer_id ] > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- G.7.4.7.1 General slice segment header semantics

luma_upsampling_filter_length_flag equal to 0 specifies that the luma up-sampling filter length is equal to 8. If luma_upsampling_filter_length_flag is equal to 1 it specifies that the luma filter length is equal to 4. When not present the value of luma_upsampling_filter length_flag is inferred to be equal to sps_luma_upsampling_filter_length_flag.

- G.8.1.4.1.3 Luma sample interpolation process

Inputs to this process are

–	the luma reference sample array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture.

–	luma_upsampling_filter_length_flag

Output of this process is a interpolated luma sample value intLumaSample.

- Table G-1 specifies the 8-tap filter coefficients fL[ p, x ] with p = 0 ... 15 and x = 0 ... 7 used for the luma resampling process when luma_upsampling_filter_length_flag is equal to 0.

Table G-1 – 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 4 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 6 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 7 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 10 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 11 | −1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 12 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 14 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 15 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |

If the luma_upsampling_filter_length_flag is equal to 1 a 4-tap filter is derived from Table G-1:

- Filter coefficients fL[ p, x ] at position x==2 and phases p==5, p==8 and p==11 are set equal to  fL[ p, 0 ]+ fL[ p, 1 ]+ fL[ p, 2 ].

- Filter coefficients fL[ p, x ] at position x==5 and phases p==5, p==8 and p==11 are set equal to  fL[ p, 5 ]+ fL[ p, 6 ]+ fL[ p, 7 ].

- The filter length N is set equal to 4 and a filter offset M is set equal to 2.

Otherwise the filter length N is set equal to 8 and a filter offset M is set equal to 0.

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )	(G-1)

xPhase = ( xRef16 ) % 16	(G-2)

- The variables yRef and yPhase are derived as follows

yRef     = ( yRef16 >> 4 )	(G-3)

yPhase = ( yRef16 ) % 16	(G-4)

- The sample value tempArray[ n] with n = 0..N, is derived as follows.

yPosRL = Clip3( 0, RefLayerPicHeightInSamplesL – 1, yRef + n – 1 ) 	(G-5)

refW      = RefLayerPicWidthInSamplesL

tempArray[n] =0

for(k=0; k<N; k++)

tempArray[n]  += fL[ xPhase, n ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 3+n), yPosRL ] +
	fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 2), yPosRL ] +
	fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 1), yPosRL ] +
	fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef      ), yPosRL ] +
tempArray[n]  += fL[ xPhase, k+M ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – N/2+1+k), yPosRL ] 	(G-6)
	fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 2), yPosRL ] +
	fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 3), yPosRL ] +
	fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 4), yPosRL ]

- The interpolated luma sample value intLumaSample is derived as follows.

for(n=0; n<N; n++)

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
intLumaSample  +=	fL[ yPhase, n+M ] * tempArray [ n ] 	(G-7)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + (1<<11)) >> 12

intLumaSample = Clip3( 0, ( 1 << BitDepthY) – 1 , (intLumaSample+(1<<11)) >> 12 )	(G-8)

# Conclusions

It has been shown that the computational complexity significantly can be reduced when always deploying 4-tap luma up-sampling filters compared to current 8-tap luma up-sampling filters with some loss in coding efficiency. By enabling the possibility to select the length of the luma up-sampling filter either same for the whole coded sequence or on a slice basis it is possible to trade complexity versus performance according to the needs of the applications. We propose to include the possibility to use 4-tap filters for luma up-sampling to the working draft.

# References

[1] X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-M1009, April, 2013.

[2] E. François, A. Tabatabai, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440 version 6, January, 2013.

[3] E. François, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-M0455 version 2, April, 2013,

[4] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Working Draft 2”, JCTVC-M1008 version 2, April, 2013,

# Patent rights declaration(s)

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).