| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0266 |
| --- | --- |

| Title: | Non-RCE1: Inter Color Component Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Woo-Shik Kim Jianle Chen Krishna Rapaka Liwei Guo Joel Sole Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-7749 wei.pu@qti.qualcomm.com wooshikk@qti.qualcomm.com cjianle@qti.qualcomm.com krapaka@qti.qualcomm.com liweig@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the 13th JCT-VC meeting, seven experiments were included in RCE1 to study inter-component decorrelation methods. This proposal presents an asserted improvement of RCE1 Experiment 3. In the proposed method, the chroma residual is predicted using the scaled luma residual signal. The scaling factor is signaled for each TU. Compared to Experiment 3, a wider range of scaling values are allowed in order to improve the prediction performance. A flag is signaled to switch on/off the method. In case of intra coding, the method is allowed only when DM mode is used as chroma prediction mode. For common coding conditions, the average BD-rate of the method is -20.2%, 2.1%, -1.1% for Y, U, V, respectively. For the screen content sequences, the BD-rate is -23.0%, -13.7%, -13.8% for Y, U, V, respectively.

# Introduction

In JCTVC-M0230 [2], an inter-color-component residual prediction method was proposed as follows.

(1)

where

rCx,y denotes the final chroma reconstructed residual sample at a position ,
 denotes the reconstructed chroma residual sample from the bit-stream at a position ,
 denotes the reconstructed residual sample in the luma component at a position , and

p∈0,…,N, where the skipping of prediction is given by p=0.

This method shows good coding gain in RGB test sequence. For YCbCr sequences, the method shows no benefits.

# Technical description

## Prediction of chroma residual signal using scaled luma residual signal

In this proposal, the chroma residual is predicted at the encoder side as:

rC'(x,y)=rC(x,y)−α×rL(x,y)>>3 (2)

and it is compensated at the decoder side as:

rC(x,y)=rC'(x,y)+α×rL(x,y)>>3 (3)

The scaling parameter α is calculated at the encoder side and signaled. The decoder does not need to re-calculate α, so there is no additional computing operations except equation (3) and no additional memory access at the decoder side.

This prediction is performed both for intra- and inter-coded blocks. However, in case of intra coding, only those with DM chroma mode are allowed to use this prediction.

## Signaling of scaling parameters

In [2], only positive α is allowed. For example, in a typical setting, α is chosen from {0, 2, 4, 8}. Such restriction affects the coding efficiency of YCbCr sequences as the residual signals of chroma and luma can be mapped with a negative slope.

In this proposal, α is allowed to be chosen from −8, −4, −2, −1, 0, 1, 2, 4, 8, inclusively. This approach allows better utilization of the correlation between luma and chroma residual signals, even when they are negatively correlated.

When α=0, the inter-component residual prediction is turned off. The proposed method includes TU level on/off switch signaling to efficiently handle this case.

To code α using CABAC, one bin is used to indicate whether it equals to 0 or not. If α is not equal to 0, another bin is used to code the sign of α. Then abs(α) is mapped using the following table before binarization.

Table I. α Mapping Table.

| abs(α) | Mapped, M(α) |
| --- | --- |
| 8 | 3 |
| 4 | 2 |
| 2 | 1 |
| 1 | 0 |

M(α) is binarized using truncated unary code and coded using CABAC. At the decoder side, after CABAC decoding, M(α) is inversely mapped back to abs(α).

# Syntax and Semantics

## Sequence parameter set RBSP syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_format_idc = = 3 ) |  |
| inter_comp_residual_pred_flag | u(1) |

inter_comp_residual_pred_flag equal to 1 specifies that inter component residual prediction is enabled. It equal to 0 specifies that inter component residual prediction is disabled. If this flage is not presented, it is set to 0.

## Transform unit syntax and semantics

| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( log2TrafoSize > 2 ¦ ¦ ChromaArrayType = = 3 ) { |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if (cbf_luma[x0][y0][trafoDepth] && inter_comp_residual_pred_flag) |  |
| alpha_coding(x0, y0 + ( tIdx << log2TrafoSizeC ), 1) |  |
| if( cbf_cb[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + ( tIdx << log2TrafoSizeC ), log2TrafoSizeC, 1 ) |  |
| } |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if (cbf_luma[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] && inter_comp_residual_pred_flag) |  |
| alpha_coding(x0, y0 + ( tIdx << log2TrafoSizeC ), 2) |  |
| if( cbf_cr[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + ( tIdx << log2TrafoSizeC ), log2TrafoSizeC, 2 ) |  |
| } |  |
| } else if( blkIdx = = 3 ) { |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if( cbf_luma[ xBase ][ yBase ][ trafoDepth ] && inter_comp_residual_pred_flag) |  |
| alpha_coding( xBase, yBase + ( tIdx << log2TrafoSize ), 1 ) |  |
| if( cbf_cb[ xBase ][ yBase ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase + ( tIdx << log2TrafoSize ), log2TrafoSize, 1 ) |  |
| } |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if( cbf_luma[ xBase ][ yBase ][ trafoDepth ] && inter_comp_residual_pred_flag) |  |
| alpha_coding( xBase, yBase + ( tIdx << log2TrafoSize ), 2 ) |  |
| if( cbf_cr[ xBase ][ yBase ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase + ( tIdx << log2TrafoSize ), log2TrafoSize, 2 ) |  |
| … |  |

## Alpha coding syntax and semantics

| alpha_coding( x0, y0, cIdx ) { | Descriptor |
| --- | --- |
| alpha_abs | ae(v) |
| if (alpha_abs) |  |
| alpha_sign | ae(v) |
| } |  |

alpha_abs specifies the absolute value of alpha. When alpha_abs is not present, it is inferred to be equal to 0.

alpha_sign equal to 1 sepcifies that sign of alpha is positive. alpha_sign equal to 0 specifies that alpha is negative.

The variable alpha[ x0, y0 ] is derived as

- If alpha_abs is equal to 0, alpha[ x0, y0 ] is set equal to 0.

- Otherwise,
	alpha[ x0, y0 ] = alpha_sign * alpha_abs

## Decoding process for chroma residual blocks

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

…

– 	a variable alpha[xCb + xB0, yCb + yB0] specifying the inter component residual prediction coefficient,

–	an array resSamplesLuma of luma residual samples.

…

- The (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows, for i = 0..nTbS − 1, j = 0..nTbS − 1:

If ChromaArrayType is equal to 3 and alpha[ xCb + xB0, yCb + yB0 ] is not equal to 0,

resSamples[ (xCb + xB0)/SubWidthC + i, (yCb + yBN)/SubHeightC + j ] =

transformBlock[ i, j ] + ( ( alpha[xCb + xB0, yCb + yB0] * resSamplesLuma[ xCb + xB0 + i, yCb + yBN + j ] ) >> 3 )

- Test results

The method implemented on top of HM10.1_RExt3.0 and the experiments are performed using the common test conditions defined for RCE1 [1].

The tables below summarize the RD performance of the tested method. For RGB sequences, the internal chroma bit-depth is increased by 1, which corresponds to a chroma QP increment of 6.

Table 1. BD-rate reduction compared with HEVC Range Extension 3.0 Anchor (Main-tier)

| All Intra Main-tier. Y; U; V; G psnr; B psnr; R psnr; psnrYUV; psnrGBR; psnrGBRm. RGB 4:4:4; -44.8%; -6.8%; -24.4%; -53.0%; 4.6%; -2.1%; -38.7%; -41.2%; -24.4%. YCbCr 4:4:4; -1.5%; -6.7%; -6.9%; -2.8%. YCbCr 4:2:2; -0.2%; -3.9%; -3.1%; -0.8%. SC RGB 4:4:4; -36.2%; -25.4%; -28.2%; -40.9%; -20.7%; -22.4%; -33.4%; -35.2%; -28.8%. SC YCbCr 4:4:4; -6.9%; -10.0%; -8.7%; -7.7%. RExt; -16.8%; -5.9%; -12.0%; -53.0%; 4.6%; -2.1%; -15.2%; -41.2%; -24.4%. SC; -21.6%; -17.7%; -18.4%; -40.9%; -20.7%; -22.4%; -20.6%; -35.2%; -28.8%. Overall; -19.0%; -11.2%; -14.9%; -46.6%; -8.8%; -12.8%; -17.6%; -38.0%; -26.7%. Enc Time[%]; 112%. Dec Time[%]; 99%. Random Access Main-tier. Y; U; V; G psnr; B psnr; R psnr; psnrYUV; psnrGBR; psnrGBRm. RGB 4:4:4; -48.3%; -6.6%; -22.6%; -56.6%; 6.2%; -2.6%; -42.7%; -45.1%; -28.3%. YCbCr 4:4:4; -0.5%; -8.6%; -7.4%; -2.2%. YCbCr 4:2:2; 0.1%; -4.6%; -2.9%; -0.4%. SC RGB 4:4:4; -34.5%; -22.1%; -25.0%; -39.5%; -16.6%; -18.2%; -31.3%; -33.2%; -25.8%. SC YCbCr 4:4:4; -6.0%; -9.2%; -8.4%; -6.9%. RExt; -17.7%; -6.6%; -11.5%; -56.6%; 6.2%; -2.6%; -16.4%; -45.1%; -28.3%. SC; -20.2%; -15.7%; -16.7%; -39.5%; -16.6%; -18.2%; -19.1%; -33.2%; -25.8%. Overall; -18.8%; -10.7%; -13.9%; -47.6%; -5.9%; -10.9%; -17.6%; -38.8%; -27.0%. Enc Time[%]; 110%. Dec Time[%]; 100%. Low delay B Main-tier. Y; U; V; G psnr; B psnr; R psnr; psnrYUV; psnrGBR; psnrGBRm. RGB 4:4:4; -51.3%; 0.5%; -13.2%; -59.6%; 14.1%; 4.7%; -43.8%; -46.7%; -26.9%. YCbCr 4:4:4; -0.3%; -6.3%; -5.9%; -1.6%. YCbCr 4:2:2; 0.2%; -2.6%; -1.6%; -0.1%. SC RGB 4:4:4; -34.6%; -20.5%; -24.0%; -39.8%; -14.6%; -16.3%; -31.1%; -33.0%; -25.0%. SC YCbCr 4:4:4; -5.9%; -9.2%; -8.1%; -6.8%. RExt; -18.7%; -2.7%; -7.2%; -59.6%; 14.1%; 4.7%; -16.5%; -46.7%; -26.9%. SC; -20.3%; -14.9%; -16.1%; -39.8%; -14.6%; -16.3%; -18.9%; -33.0%; -25.0%. Overall; -19.4%; -8.1%; -11.2%; -49.1%; -1.1%; -6.4%; -17.6%; -39.4%; -25.9%. Enc Time[%]; 104%. Dec Time[%]; 100%. Table 2. BD-rate reduction compared with HEVC Range Extension 3.0 Anchor (High-tier) All Intra High-tier. Y; U; V; G psnr; B psnr; R psnr; psnrYUV; psnrGBR; psnrGBRm. RGB 4:4:4; -41.4%; -3.3%; -19.6%; -49.4%; 8.3%; 2.6%; -33.7%; -36.0%; -18.1%. YCbCr 4:4:4; -1.8%; -4.5%; -6.2%; -2.8%. YCbCr 4:2:2; -0.2%; -2.1%; -2.2%; -0.6%. SC RGB 4:4:4; -33.2%; -23.6%; -26.2%; -37.5%; -19.5%; -20.7%; -30.7%; -32.2%; -26.3%. SC YCbCr 4:4:4; -6.9%; -8.1%; -8.2%; -7.3%. RExt; -15.7%; -3.3%; -9.8%; -49.4%; 8.3%; 2.6%; -13.3%; -36.0%; -18.1%. SC; -20.1%; -15.9%; -17.2%; -37.5%; -19.5%; -20.7%; -19.0%; -32.2%; -26.3%. Overall; -17.7%; -9.0%; -13.1%; -43.1%; -6.4%; -9.7%; -15.9%; -34.0%; -22.5%. Enc Time[%]; 111%. Dec Time[%]; 98%. Random Access High-tier. Y; U; V; G psnr; B psnr; R psnr; psnrYUV; psnrGBR; psnrGBRm. RGB 4:4:4; -49.9%; 3.1%; -19.0%; -58.4%; 19.8%; 8.9%; -41.3%; -43.6%; -21.4%. YCbCr 4:4:4; -0.9%; -6.3%; -7.8%; -2.4%. YCbCr 4:2:2; 0.1%; -2.0%; -2.1%; -0.3%. SC RGB 4:4:4; -32.1%; -20.5%; -23.2%; -37.0%; -15.3%; -16.5%; -29.0%; -30.6%; -23.6%. SC YCbCr 4:4:4; -6.3%; -7.8%; -8.0%; -6.8%. RExt; -18.4%; -1.5%; -10.0%; -58.4%; 19.8%; 8.9%; -15.9%; -43.6%; -21.4%. SC; -19.2%; -14.1%; -15.6%; -37.0%; -15.3%; -16.5%; -17.9%; -30.6%; -23.6%. Overall; -18.7%; -7.2%; -12.5%; -47.1%; 1.2%; -4.6%; -16.8%; -36.7%; -22.6%. Enc Time[%]; 111%. Dec Time[%]; 100%. Low delay B High-tier. Y; U; V; G psnr; B psnr; R psnr; psnrYUV; psnrGBR; psnrGBRm. RGB 4:4:4; -52.8%; 7.1%; -11.8%; -61.3%; 26.3%; 16.3%; -42.7%; -45.4%; -20.6%. YCbCr 4:4:4; -0.7%; -4.4%; -6.5%; -1.8%. YCbCr 4:2:2; 0.1%; -1.1%; -0.9%; -0.1%. SC RGB 4:4:4; -32.4%; -19.2%; -22.3%; -37.5%; -13.6%; -14.8%; -28.9%; -30.6%; -22.9%. SC YCbCr 4:4:4; -6.4%; -7.9%; -7.8%; -6.9%. RExt; -19.4%; 0.9%; -6.7%; -61.3%; 26.3%; 16.3%; -16.1%; -45.4%; -20.6%. SC; -19.4%; -13.6%; -15.1%; -37.5%; -13.6%; -14.8%; -17.9%; -30.6%; -22.9%. Overall; -19.4%; -5.6%; -10.4%; -48.7%; 5.2%; -0.2%; -16.9%; -37.6%; -21.8%. Enc Time[%]; 105%. Dec Time[%]; 99%. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 3. BD-rate reduction compared with HEVC Range Extension 3.0 Anchor (super High-tier)

|  | All Intra Super High-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 | -35.0% | -0.5% | -13.1% | -42.9% | 11.2% | 6.1% | -27.3% | -29.6% | -12.7% |
| YCbCr 4:4:4 | -1.9% | -3.2% | -4.5% |  |  |  | -2.5% |  |  |
| YCbCr 4:2:2 | -0.1% | -1.3% | -1.3% |  |  |  | -0.4% |  |  |
| SC RGB 4:4:4 | -30.9% | -21.0% | -23.3% | -35.1% | -16.3% | -17.0% | -28.3% | -29.7% | -23.7% |
| SC YCbCr 4:4:4 | -6.4% | -6.7% | -7.1% |  |  |  | -6.6% |  |  |
| RExt | -13.4% | -1.6% | -6.6% | -42.9% | 11.2% | 6.1% | -10.8% | -29.6% | -12.7% |
| SC | -18.6% | -13.9% | -15.2% | -35.1% | -16.3% | -17.0% | -17.4% | -29.7% | -23.7% |
| Overall | -15.8% | -7.1% | -10.5% | -38.8% | -3.3% | -6.1% | -13.8% | -29.7% | -18.5% |
| Enc Time[%] | 111% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |  |

# Conclusions

In this proposal, inter-color-component residual prediction is proposed. Compared to HEVC Range Extension 3.0, significant BD-rate reduction for both RGB and YCbCr sequences are observed. Compared to RCE1 Experiment 3, more flexibility is provided by allowing wider range of scaling factor, while the complexity is reduced by applying it only for the intra DM chroma mode.

# References

- T. Nguyen, J. Sole, and J. Kim, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods,” JCTVC-M1121, Incheon, Korea, Apr. 2013.

- T. Nguyen, A. Khairat, D. Marpe, “Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive inter-plane prediction for RGB content,” JCTVC-M0230, Incheon, Korea, Apr. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “HEVC Range Extensions Draft 3,” JCTVC-M1005, Incheon, Korea, Apr. 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).