| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0268 WG11 Doc: m29917 |
| --- | --- |

| Title: | RCE2 Test A.1: Rice parameter signaling for transform-skip blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-858-845-1795 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

As part of RCE2, cRiceParam signalling methods were tested. In the first test, cRiceParameter is signalled from the set {0, 1, 2, 3, 4}. In the second test, cRiceParam is signalled from the subset {0, 2, 4}. Truncated unary (TU) binarization method is used to code cRiceParameter for both test cases.

The lossless results for method 2 have been shown as follows;

|  | AI Main | RA Main | LD( B )Main |
| --- | --- | --- | --- |
| Class F | -1.8% | -0.9% | -0.6% |
| Class B | -0.6% | -1.3% | -1.3% |
| SC RGB 444 | -8.3% | -6.9% | -6.5% |
| SC YUV 444 | -6.3% | -5.7% | -5.9% |
| RangeExt | -2.9% | -2.1% | -2.1% |

The lossy results for method 2 have been shown as follows;

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.2% | -0.9% | -0.8% | -0.7% | -1.4% | -1.3% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.1% | -2.1% | -2.1% | -4.9% | -4.8% | -4.8% | -7.4% | -7.6% | -7.6% |
| SC YUV 444 | 0.2% | 0.1% | 0.2% | -0.7% | -0.6% | -0.5% | -2.3% | -2.3% | -2.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.8% | -1.8% | -1.8% | -4.0% | -3.9% | -3.9% |  |  |  |
| SC YUV 444 | 0.4% | 0.3% | 0.4% | -0.2% | -0.2% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.7% | 0.1% | -0.2% | 0.3% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -1.5% | -1.4% | -1.4% | -4.1% | -4.0% | -4.1% |  |  |  |
| SC YUV 444 | 0.5% | 0.7% | 0.4% | -0.1% | 0.0% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |

# Proposed solution

In the proposed method, rice parameter (cRiceParam) is signalled for each transform-skip block and the existing rice-parameter update process is removed. Method 1and Method 2 select cRiceParam from the given set {0, 1, 2, 3, 4} and {0, 2, 4}, respectively. The binarization for the cRiceParm is shown.

Table 1 TU binarization for cRiceParam (Method 2)

| Rice-parameter | Bin string |
| --- | --- |
| 0 | 1 |
| 2 | 01 |
| 4 | 00 |

# Results

Experiment conditions are described in [2]. The two methods (Method 1 and Method 2) are tested in lossy and lossless coding.

Table 1 Method 1 – Lossless

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.5% | 31.7 | 32.0 | -0.7% | 49.8 | 50.0 | -0.5% |
| Class B | 2.2 | 2.3 | -0.8% | 2.6 | 2.6 | -1.4% | 2.6 | 2.6 | -1.3% |
| SC RGB 444 | 10.1 | 10.8 | -6.3% | 100.4 | 108.1 | -5.2% | 381.6 | 413.8 | -4.9% |
| SC YUV 444 | 11.4 | 12.2 | -5.0% | 128.9 | 138.6 | -4.4% | 325.6 | 357.9 | -4.6% |
| RangeExt | 2.4 | 2.4 | -2.9% | 2.5 | 2.6 | -2.5% | 2.5 | 2.6 | -2.5% |

Table 2 Method 1- Lossy

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.6% | -0.5% | -0.5% | -1.1% | -0.9% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.4% | -1.4% | -1.4% | -3.7% | -3.6% | -3.6% | -5.9% | -6.1% | -6.1% |
| SC YUV 444 | 0.4% | 0.4% | 0.5% | -0.3% | -0.2% | -0.1% | -1.6% | -1.7% | -1.8% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.2% | 0.1% | -0.3% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.2% | -1.1% | -1.1% | -3.1% | -3.0% | -3.0% |  |  |  |
| SC YUV 444 | 0.5% | 0.5% | 0.6% | 0.0% | 0.1% | 0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.6% | 0.5% | -0.1% | 0.3% | 0.3% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -0.8% | -0.8% | -0.8% | -3.1% | -3.1% | -3.1% |  |  |  |
| SC YUV 444 | 0.3% | 0.5% | 0.4% | 0.0% | 0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |

Table 3 Method 2 – Lossless

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.8% | 31.7 | 32.2 | -0.9% | 49.8 | 50.2 | -0.6% |
| Class B | 2.2 | 2.3 | -0.6% | 2.6 | 2.6 | -1.3% | 2.6 | 2.6 | -1.3% |
| SC RGB 444 | 10.1 | 11.1 | -8.3% | 100.4 | 110.9 | -6.9% | 381.6 | 425.5 | -6.5% |
| SC YUV 444 | 11.4 | 12.5 | -6.3% | 128.9 | 142.0 | -5.7% | 325.6 | 368.6 | -5.9% |
| RangeExt | 2.4 | 2.4 | -2.9% | 2.5 | 2.6 | -2.1% | 2.5 | 2.6 | -2.1% |

Table 4 Method 2- Lossy

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.2% | -0.9% | -0.8% | -0.7% | -1.4% | -1.3% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.1% | -2.1% | -2.1% | -4.9% | -4.8% | -4.8% | -7.4% | -7.6% | -7.6% |
| SC YUV 444 | 0.2% | 0.1% | 0.2% | -0.7% | -0.6% | -0.5% | -2.3% | -2.3% | -2.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.8% | -1.8% | -1.8% | -4.0% | -3.9% | -3.9% |  |  |  |
| SC YUV 444 | 0.4% | 0.3% | 0.4% | -0.2% | -0.2% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.7% | 0.1% | -0.2% | 0.3% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -1.5% | -1.4% | -1.4% | -4.1% | -4.0% | -4.1% |  |  |  |
| SC YUV 444 | 0.5% | 0.7% | 0.4% | -0.1% | 0.0% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |

An experiment is additionally performed based on Method 2. In the experiment, slice level flag is considered to determine whether or not the proposed rice-parameter signaling method is activated. The proposed rice parameter signaling method is used only if the slice level flag is equal to 1. Specifically, in the supplemental experiment, slice level flag is set to one when current slice QP is smaller than 32.

Table 5 Method 2- Lossy (with Slice Level flag)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.3% | -0.4% | -0.9% | -0.7% | -0.8% | -1.4% | -1.2% | -1.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.1% | -2.1% | -2.1% | -4.7% | -4.7% | -4.6% | -7.2% | -7.5% | -7.5% |
| SC YUV 444 | 0.1% | 0.1% | -0.1% | -0.8% | -0.7% | -0.6% | -2.2% | -2.3% | -2.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.1% | -0.1% | -0.5% | -0.4% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -1.7% | -1.7% | -1.7% | -3.8% | -3.8% | -3.8% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.1% | -0.4% | -0.4% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.3% | -0.3% | -0.1% | 0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -1.9% | -1.9% | -1.8% | -4.2% | -4.1% | -4.2% |  |  |  |
| SC YUV 444 | 0.1% | 0.2% | 0.1% | -0.4% | -0.3% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |

# References

- S.H. Kim, A. Segall, “Non-RCE1/Non-RCE2: Simplified level coding for Transform skip and lossless coding”, JCTVC-M0304, Incheon, KR, April 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, Incheon, KR, April 2013.

# Text Changes

# - Changes to residual coding syntax and semantics

#### 7.3.8.11		Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| lastScanPos = 16 |  |
| lastSubBlock = ( 1 << ( log2TrafoSize − 2 ) ) * ( 1 << ( log2TrafoSize − 2 ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| } |  |
| firstSigScanPos = 16 |  |
| lastSigScanPos = −1 |  |
| numGreater1Flag = 0 |  |
| lastGreater1ScanPos = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( numGreater1Flag < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numGreater1Flag++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && lastGreater1ScanPos = = −1 ) |  |
| lastGreater1ScanPos = n |  |
| } |  |
| if( lastSigScanPos = = −1 ) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| } |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 && !cu_transquant_bypass_flag ) |  |
| if( lastGreater1ScanPos != −1 ) |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !sign_data_hiding_enabled_flag ¦ ¦ !signHidden ¦ ¦ ( n != firstSigScanPos ) ) ) |  |
| coeff_sign_flag[ n ] |  |
| } | ae(v) |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if(transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦¦ cu_transquant_bypass_flag ) { |  |
| cRiceParam |  |
| } |  |
| else { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| } |  |
| coeff_abs_level_remaining[ n ] |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_enabled_flag && signHidden ) { | ae(v) |
| sumAbsLevel += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( ( n = = firstSigScanPos ) && ( ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| numSigCoeff++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 9.3.3.9 Binarization process for RiceParam

Input to this process is a request for a binarization for the syntax element RiceParam.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element RiceParam is specified in Table 9-X.

Table 9-X TU binarization for cRiceParam (for Method 1)

| RiceParam | Bin string |
| --- | --- |
| 0 | 1 |
| 1 | 01 |
| 2 | 001 |
| 3 | 0001 |
| 4 | 0000 |

Table 9-X TU binarization for cRiceParam (for Method 2)

| RiceParam | Bin string |
| --- | --- |
| 0 | 1 |
| 2 | 01 |
| 4 | 00 |

#### 9.3.3.10 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, and baseLevel.

Output of this process is the binarization of the syntax element.

- If this process is invoked for the first time for the current sub-block scan index i, cLastRiceParam are set from the decoded information when the following condition is satisfied

if((transform_skip_flag[ x0 ][ y0 ][ cIdx ] || cu_transquant_bypass_flag) )

where transform_skip_flag[ x0 ][ y0 ][ cIdx ] indicates the transform skip flag for the current sub-block.

- Otherwise the variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block scan index i, cLastAbsLevel and cLastRiceParam are set equal to 0.

- Otherwise (this process is not invoked for the first time for the current sub-block scan index i), cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

The variable cAbsLevel is set equal to baseLevel + coeff_abs_level_remaining[ n ].

The variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-1)

# - Changes required to the initialization process for context variables

The following changes to Table 9-3 are required for the introduction of the inter_rdpcm_flag and inter_rdpcm_dir_flag binary flags. Moreover, one initialization table (Table 9X) is added.

Table 9-3 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao( ) | sao_merge_left_flag sao_merge_up_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sao_type_idx_luma sao_type_idx_chroma | Error: Reference source not found | 0 | 1 | 2 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
| prediction_unit( ) | merge_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc[ ][ ] | Error: Reference source not found |  | 0..4 | 5..9 |
|  | ref_idx_l0[ ][ ], ref_idx_l1[ ][ ] | Error: Reference source not found |  | 0..1 | 2..3 |
|  | mvp_l0_flag[ ][ ], mvp_l1_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | cbf_luma[ ][ ][ ] | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb[ ][ ][ ], cbf_cr[ ][ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | Error: Reference source not found |  | 0 | 2 |
|  | abs_mvd_greater1_flag[ ] | Error: Reference source not found |  | 1 | 3 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
| residual_coding( ) | transform_skip_flag[ ][ ][ 0 ] | Table 9- | 0 | 1 | 2 |
|  | transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Table 9- | 3 | 4 | 5 |
|  | cRiceParam_prefix | Table 9X | 0 | 1 | 2 |
|  | last_sig_coeff_x_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | last_sig_coeff_y_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | coded_sub_block_flag[ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | sig_coeff_flag[ ][ ] | Error: Reference source not found | 0..41 | 42..83 | 84..125 |
|  | coeff_abs_level_greater1_flag[ ] | Error: Reference source not found | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag[ ] | Error: Reference source not found | 0..5 | 6..11 | 12..17 |

Table 9-X – Values of initValue for ctxIdx of cRiceParam_prefix

| Initialization variable | ctxIdx of cRiceParam_prefix |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 153 | 153 | 153 |

# Changes required to the binarization process

Table 9-4 is changed to define the binarization process for the inter_rdpcm_flag and inter_rdpcm_dir_flag flags.

| Table 9-4 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| slice_segment_data( ) | end_of_slice_segment_flag | FL | cMax = 1 |
|  | end_of_sub_stream_one_bit | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | FL | cMax = 1 |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] |  | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| prediction_unit( ) | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | Error: Reference source not found | nPbW, nPbH |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cr[ ][ ][ ] | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
| residual_coding( ) | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | c_riceParam_prefix | FL | cMax = 1 |
|  | c_riceParam_suffix | FL | cMax = 1 |
|  | last_sig_coeff_x_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater1_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater2_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_remaining[ ] |  | current sub-block scan index i, baseLevel |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |

# Changes required to the derivation of context index variables and bypass

The following rows are added to Table 9-5 for the inter_rdpcm_flag and inter_rdpcm_dir_flag flags.

| Table 9-5 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_slice_segment_flag | terminate | na | na | na | na | na |
| end_of_sub_stream_one_bit | terminate | na | na | na | na | na |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_offset_sign[ ][ ][ ][ ] | bypass | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass | bypass | bypass | na | na | na |
| sao_eo_class_chroma | bypass | bypass | bypass | na | na | na |
| split_cu_flag[ ][ ] | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| cu_transquant_bypass_flag | 0 | na | na | na | na | na |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |
| part_mode log2CbSize > MinCbLog2SizeY | 0 | 1 | 3 | bypass | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| prev_intra_luma_pred_flag[ ][ ] | 0 | na | na | na | na | na |
| mpm_idx[ ][ ] | bypass | bypass | na | na | na | na |
| rem_intra_luma_pred_mode[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] | 0 | bypass | bypass | na | na | na |
| rqt_root_cbf | 0 | na | na | na | na | na |
| merge_flag[ ][ ] | 0 | na | na | na | na | na |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | na | na |
| inter_pred_idc[ x0 ][ y0 ] | ( nPbW + nPbH ) != 12 ? CtDepth[ x0 ][ y0 ] : 4 | 4 | na | na | na | na |
| ref_idx_l0[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| ref_idx_l1[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| mvp_l0_flag[ ][ ] | 0 | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 | na | na | na | na | na |
| split_transform_flag[ ][ ][ ] | 5 − log2TrafoSize | na | na | na | na | na |
| cbf_cb[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_cr[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_luma[ ][ ][ ] | trafoDepth = = 0 ? 1 : 0 | na | na | na | na | na |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |
| cu_qp_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| cu_qp_delta_sign_flag | bypass | na | na | na | na | na |
| transform_skip_flag[ ][ ][ ] | 0 | na | na | na | na | na |
| last_sig_coeff_x_prefix | 0..17 (subclause Error: Reference source not found) |  |  |  |  |  |
| last_sig_coeff_y_prefix | 0..17 (subclause Error: Reference source not found) |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| coded_sub_block_flag[ ][ ] | 0..3 (subclause Error: Reference source not found) | na | na | na | na | na |
| sig_coeff_flag[ ][ ] | 0..41 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater1_flag[ ] | 0..23 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater2_flag[ ] | 0..5 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_remaining[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| coeff_sign_flag[ ] | bypass | na | na | na | na | na |
| Rice_param_prefix | 0 | na | na | na | na | na |
| Rice_param_suffix | bypass | na | na | na | na | na |

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).