| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July–5 Aug. 2013 | Document: JCTVC-N0272r1 |
| --- | --- |

| Title: | Non-SCE1: On handling re-sampling phase offsets with fixed filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon, Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com ajay.luthra@arrisi.com |
| Source: | Arris Group, Inc. |  |  |

_____________________________

# Abstract

This document discusses issues with re-sampling using fixed filters, and reports the results of comparison of fixed filters with different phase resolution tested in SCE1 Test 1.2 [1] with that proposed in SCE1 Test 2.2 [2].  For the sequences tested with BL sequences generated with a downsampling offset, the results reportedly show that even with 1/16 phase resolution accuracy, there is a loss in luma performance of 0.6% for 1.5x scalability compared to adaptive filter signaling proposed in SCE1 Test 2.2.  In order to compensate for limited filter phase precision computation using fixed filters, it is proposed here that offsets be signaled per filter to improve performance when fixed filters are used.

# Introduction

In order to provide good interlayer prediction, it is important that the BL upsampling process compensate for the downsampling operation in generation of the BL.  This document describes some issues with re-sampling using fixed filters, compares the performance of fixed filters with adaptive filters, and proposes how performance can be improved when fixed filters are used.

# Fixed vs. adaptive filters

One problem with fixed re-sampling filters is that since they have a fixed phase resolution, they cannot properly compensate for general scalability ratios or downsampling offsets that may have been used in generation of the BL.  Since downsampling is non-normative, adaptive filter signaling can specify upsampling filters that are matched to the downsampling filters.  Note that adaptive filtering can also benefit the case when the spatial resolution does not change.  Computation and rounding operations needed to determine the filter phase index in the fixed filter case are avoided when adaptive filters are used.

In addition, it has been observed that some of the fixed filters used in SCE1 do not have phase delays that accurately correspond to the filter index.  Although such non-uniform spacing of phase delays may benefit some scalability ratios and offsets, it adversely affects performance of other cases.  Adaptive signaling of the upsampling filters can provide accurate phase compensation and improve coding efficiency for arbitrary scaling and offsets.  By signaling the filter information at the PPS level, it is possible to accommodate differences in luma and chroma phase offsets such as in the case of field pictures.

# Performance comparison

The results reported in [1] show the performance relative to SHM-2.0 over sequences generated with a downsampling phase offset.  Clearly, as the results show, higher phase offset resolution accuracy for the 1.5x case provides better performance.  However, even the highest offset resolution tested may not provide enough accuracy to completely compensate for the downsampling phase offset.  For example, for the 1.5x scalability case tested, it was determined that the luma downsampling phase offset was -1/4.  The luma upsampling phase offset needed to match this downsampling offset is 1/6.  This means that the upsampling filters need phases of 1/6, 1/2, and 5/6.  However, with even a 1/16 phase offset resolution the first and last phases have an offset error of 0.667/16 (for one-third of pixels) and 0.333/16 (for another one third of pixles), respectively.  Table 1 shows the offset error for different phase offset resolutions, and the numbers in the first three rows compare the performance of the results reported in [1] to Test 2.2 where filters with the exact phase offset compensation are signaled.  Note that even with 1/16 resolution in Test 1.2, there is a luma BDR loss of 0.6% compared to Test 2.2.  The results in Table 1 show that the loss increases non-linearly even with small increases in the offset error.

The offset errors in Table 1 are coming from adding two limited precision numbers. The first one is the upsampling filter phase accuracy which is 0.5/16 for filters proposed in N0273 (with uniform grid) and ~0.667/16 for SHM2.0 filters (comes from 5/16 and 11/16 phase filters). The second error comes from the phase offset signalled in the bitstream which can have a varying accuracy depending on the accuracy of the signalled value, e.g. as tested in SCE1 Test1.2 with errors of 0.5/16, 1/16 and 2/16.

Since it is possible to reduce the phase offset error, as proposed in this contribution for one index in the 1/16 phase resolution test in [1], Table 1 also reports the result in the last row (in red) when the phase offset signaling error is eliminated (e.g. by mapping as proposed in N0272) .  Although there is still a loss in performance of 0.2% compared to Test 2.2 (due to using fixed upsampling grid), the performance improves by 0.4% over Test 1.2.

| resolution | index | ¦ offset error * 16 ¦ | % BDR rate |
| --- | --- | --- | --- |
| 1/16 (SCE1-test1.2) | 2, 8, 13 | 0.667, 0, 0.333 | 0.6% |
| 1/8 | 3, 9, 14 | 0.333, 1, 0.667 | 1.3% |
| 1/4 | 1, 7, 12 | 1.667, 1, 1.333 | 3.3% |
| 1/16 N0272 | 3, 8, 13 | 0.333, 0, 0.333 | 0.2% |

Table 1.  Performance of phase offset resolution vs. Test 2.2 (exact phase offset) for luma 1.5x scalability

# Fixed filters with phase index re-mapping

As described in the previous section for the case of fixed filters, in addition to the phase offset error due to fixed phase precision, there are also errors that can result from fixed precision arithmetic in computing the best phase index for the upsampling filter.  One possibility for correcting for phase offsets per filter is to re-map the filter index.  One example of this for the case of 16 filters is shown in Table 2, where the signaling (highlighted) occurs at the PPS level, and the following syntax elements are signaled.

luma_offset_flag[ 0 ] equal to 1 indicates that the filter index offsets for upsampling the rows of the luma component are signaled.  luma_offset_flag[ 0 ] equal to 0 indicates that the filter index offsets for upsampling the rows of the luma component are not signaled.

luma_offset_flag[ 1 ] equal to 1 indicates that the filter index offsets for upsampling the columns of the luma component are signaled.  luma_offset_flag[ 1 ] equal to 0 indicates that the filter index offsets for upsampling the columns of the luma component are not signaled.

chroma_offset_flag[ 0 ] equal to 1 indicates that the filter index offsets for upsampling the rows of the chroma component are signaled.  chroma_offset_flag[ 0 ] equal to 0 indicates that the filter index offsets for upsampling the rows of the chroma component are not signaled.

chroma_offset_flag[ 1 ] equal to 1 indicates that the filter index offsets for upsampling the columns of the chroma component are signaled.  chroma_offset_flag[ 1 ] equal to 0 indicates that the filter index offsets for upsampling the columns of the chroma component are not signaled.

luma_filter_offset[ i ][ j ] specifies the offset value used to modify the luma filter index j in the upsampling process along direction i.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

chroma_filter_offset[ i ][ j ] specifies the offset value used to modify the chroma filter index j in the upsampling process along direction i.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| for( i = 0; i < 2; i++ ) { |  |
| luma_offset_flag[i] | u(1) |
| if ( luma_offset_flag[i] ) { |  |
| for( j = 0; j < 16; j++ ) |  |
| luma_filter_offset[ i ][ j ] | se(v) |
| } |  |
| chroma_offset_flag[i] | u(1) |
| if ( chroma_offset_flag[i] ) { |  |
| for( j = 0; j < 16; j++ ) |  |
| chroma_filter_offset[ i ][ j ] | se(v) |
| } |  |
| } |  |
| . . . |  |
| } |  |

Table 2.  Syntax for signaling of filter offsets

Corresponding modifications to semantics in the draft SHVC text are indicated below, where deletions are crossed out and additions are highlighted in yellow.

G.8.1.4.1.3	Luma sample interpolation process

- The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )

if ( luma_offset_flag[0] )

temp_xPhase = ( xRef16 ) % 16

xPhase = ( temp_xPhase + luma_filter_offset[ 0 ][ temp_xPhase ] ) % 16

else

xPhase = ( xRef16 ) % 16

- The variables yRef and yPhase are derived as follows:

yRef     = ( yRef16 >> 4 )

if ( luma_offset_flag[1] )

temp_yPhase = ( yRef16 ) % 16

yPhase = ( temp_yPhase + luma_filter_offset[ 1 ][ temp_yPhase ] ) % 16

else

yPhase = ( yRef16 ) % 16

G.8.1.4.1.4	Chroma sample interpolation process

- The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx and chroma sample location ( xPc, yPc ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )

if ( chroma_offset_flag[0] )

temp_xPhase = ( xRef16 ) % 16

xPhase = ( temp_xPhase + chroma_filter_offset[ 0 ][ temp_xPhase ] ) % 16

else

xPhase = ( xRef16 ) % 16

- The variables yRef and yPhase are derived as follows:

yRef     = ( yRef16 >> 4 )

if ( chroma_offset_flag[1] )

temp_yPhase = ( yRef16 ) % 16

yPhase = ( temp_yPhase + chroma_filter_offset[ 1 ][ temp_yPhase ] ) % 16

else

yPhase = ( yRef16 ) % 16

- References

[1]   J. Dong, Y. Ye, Y. He, X. Li, J. Chen, L. Guo, M. Karczewicz, K. Ugur, J. Lainema, “SCE1: Results of test 1.1 and 1.2 on sampling offset signaling”, JCTVC-N0182, 14th JCT-VC Meeting, Vienna, AT, July 2013.

[2]   K. Minoo, D. Baylon, A. Luthra, “SCE1: Results of Test 2.2 on phase compensation by signaling filter coefficients at PPS with sample shift”, JCTVC-N0078, 14th JCT-VC Meeting, Vienna, AT, July 2013.

# Patent rights declaration(s)

Arris Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).