| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July–5 Aug. 2013 | Document: JCTVC-N0273r1 |
| --- | --- |

| Title: | On the selection of fixed filters for upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com |
| Source: | Arris Group, Inc. |  |  |

_____________________________

# Abstract

This document proposes luma and chroma upsampling filters for some phase indices that have better phase characteristics than the proposed filters in SCE1 Test 1.2.

# Introduction

In examining the proposed filters in SCE1 Test 1.2 [1] upsampling filters, it was observed that some of the filters had poor phase characteristics.  For the upsampling filter process, it is important that both the filter magnitude and phase responses be well-behaved.

Although magnitude response is one characteristic to consider in the design of upsampling filters, the phase response is at least as important as this determines the position of the reconstructed sub-pixel and any distortion in phase response of a filter, potentially, can have greater subjective impact compared to distortions in the magnitude of a skipped coded block. .  Since it is known that re-sampling and any associated phase offset changes the pixel positions, it is critical that the upsampling filters have accurate and linear phase delay for pixel reconstruction at the proper positions.  An adaptive approach such as in [2] provides a way to specify upsampling filters with phases that can precisely compensate for the downsampling filtering.

In an alternate but sub-optimal approach, a set of fixed filters can be used for the upsampling.  However, only a fixed number of phases can be used in the upsampling process corresponding to the number of upsampling filters.  For the case of 16 filters, 16 fixed phase positions can be used.  In order to minimize phase offset error for these positions, a uniform phase offset spacing of 1/16 should be used.  Otherwise, non-uniform spacing can reduce the worst case performance.  It should be noted the phase spacing between filters in SCE1 Test 1.2 is not uniform; in fact, some of the filters (e.g. indices 5 and 11) favor positions of 1.5x scalability with zero phase offset  and do not accurately represent the phase delay of their associated index.  Although this increases performance for the special case tested in [1] of 1.5x, performance of other ratios and offsets would be adversely affected.

Assuming an upsampling grid of 16 phases for a uniform upsampling grid, the worst case phase error is 0.5/16, while for the current SCE1 Test 1.2, this value is ~0.667/16. As pointed out in N0272[3] the coding efficiency impact is not linear with the phase error so it is expected that the worst case coding performance with non-uniform upsampling filters will be much higher than filters proposed in this contribution which are uniform.

In the next section, luma and chroma upsampling filters are proposed for certain phase indices that both maintain good magnitude response and provide more accurate and linear phase delay.

# Proposed filters

For upsampling filtering it is proposed that the filters specified below be used for the selected luma and chroma phases.  Frequency response comparisons are shown in the following plots.  The SCE1 Test 1.2 filters are shown in blue and the proposed filters are shown in red.  In addition, the ideal magnitude and phase responses are plotted for reference.  The phase responses of the proposed filter have more accurate and linear response in the low frequency passband region than do corresponding SCE1 Test 1.2 filters.  Although there are some ripples in the magnitude response of both SCE1 Test 1.2 filters and proposed filters, this is not too unreasonable since this is expected to compensate for the pass band ripples and non-ideal transition band of typical practical downsampling filters as illustrated below in Figure 1. Also for reference the downsampling filters for creating CTC and SCE1 base layers are ploted in Figure 2. An example of a filter which is designed to match the SCE1 downsampling filter for 1.5x is shown in Figure 3. In fact it was observed matching down sampling filters for SCE1 test 2.2 improved the coding efficiency by 0.2% for 2x and 0.1% for 1.5x.

Figure 1.  Illustration of magnitude response of typical downsampling (left) and upsampling (right) filters

| (a) | (b) |
| --- | --- |

Figure 2.  Illustration of magnitude and phase response for 2x (a) and 1.5x (b) filters used for generation CTC and SCE1 base layers.

Figure 3.  Illustration of magnitude and phase response for 1/4 pel shift filters for SCE1 upsampling

Luma:

Phase 2    	{-1     3    -7    63     8    -3     1     0}

Phase 5    	{-1     4   -11    54    23    -7     3    -1}

Phase 6    	{-1     3    -9    47    31   -10     4    -1}

Phase 10 	{-1     4   -10   31    47     -9     3     -1}

Phase 11 	{-1     3     -7   23    54    -11    4    -1}

Phase 14 	{0     1     -3     8    63      -7    3    -1}

Chroma:

Phase 1    	{0    58     8    -2} (preferred), or {-2    64     2     0}

Phase 3    	{-4    58    12    -2}, or {-2    53    17    -4} (preferred)

Phase 5    	{-4    48    24    -4}

Phase 6    	{-4    44    28    -4}

Phase 7    	{-6    44    30    -4}

Phase 9    	{-4   30   44   -6}

Phase 10 	{-4   28   44   -4}

Phase 11 	{-4   24   48   -4}

Phase 13 	{-2   12   58   -4}, or {-4   17   53   -2} (preferred)

Phase 15 	{-2   8   58   0} (preferred), or {0   2   64   -2}

Figure 4.  Luma Phase 2 filter

Figure 5.  Luma Phase 5 filter

Figure 6.  Luma Phase 6 filter

Figure 7.  Chroma Phase 1 filter

Figure 8.  Chroma Phase 3 filter

Figure 9.  Chroma Phase 5 filter

Figure 10.  Chroma Phase 6 filter

Figure 11.  Chroma Phase 7 filter

# Simulations Results

For reference the results of applying the modified filters which affect the 1.5x on CTC content, is provided in the attached Excel sheet. A drop of 0.35% in terms of BDR has been observed for interpolating   1/3 and 2/3 sample position with 5/16 and 11/16 phase shift interpolation filters. The worst case and average statistical memory bandwidth is unchanged.

# References

[1]   J. Dong, Y. Ye, Y. He, X. Li, J. Chen, L. Guo, M. Karczewicz, K. Ugur, J. Lainema, “SCE1: Results of test 1.1 and 1.2 on sampling offset signaling”, JCTVC-N0182, 14th JCT-VC Meeting, Vienna, AT, July 2013.

[2]   K. Minoo, D. Baylon, A. Luthra, “SCE1: Results of Test 2.2 on phase compensation by signaling filter coefficients at PPS with sample shift”, JCTVC-N0078, 14th JCT-VC Meeting, Vienna, AT, July 2013.

[3] K. Minoo, D. Baylon, A. Luthra, “Non SCE1: On handling resampling phase offsets with fixed filters”, JCTVC-N0272, 14th JCT-VC Meeting, Vienna, AT, July 2013.

# Patent rights declaration(s)

Arris Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).