| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0274 |
| --- | --- |

| Title: | AHG14: Cross-checking of JCTVC-N0168 from Technicolor on color gamut scalable video coding using 3D LUT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification of the 10 bits results in JCTVC-N0168. Cross-check simulation confirms that the BD-rate results match with those from the proponent. Study of the corresponding software also confirms that the implementation matches the description in JCTVC-N0168.

# Introductions

JCTVC-N0168 [1] reported results of color gamut scalable video coding using 3D color Look-Up Tables (LUT) originally presented in JCTVC-M0197 [2]. It included results from color graded sequences in 8 bits and 10 bits. This contribution confirms the 10 bits results and that the software is consistent with the description in JCTVC-N0168 [1].

The proponent of JCTVC-N0168 [1] implemented the 3D LUT Color Difference Prediction model (SHM2.0-CGS) in the SHM2.0. It has the following compile time options to (1) activate color gamut scalability and (2) to fix the upsampling of 10 bits base layer video.

- #define COLOR_GAMUT_SCALABILITY      1

- #define FIX_UPSAMPLING_10BITS        1 ///< Dirty fix for up-sampling 10-bit-depth

In addition to the SHM-2.0 files, the source code distributed by the proponent has four new files for color gamut scalability:

- TComLut3D.cpp and TComLut3D.h processes the 3D LUT.

- TComToneMapping.cpp, TComToneMapping.h performs the color prediction.

The software was implemented with sequence dependent LUT. For each sequence, the LUT are loaded from an external file, and it is pruned by the encoder at the initialization stage. After the initial pruning, the LUT is encoded in the PPS and it is fixed for the coding of the entire sequence.

# Simulation and results

The cross-check was conducted with the common SHM test conditions and software reference configuration parameters [4], and the Color graded sequences (class B) provided in AHG14 [4]. The software was compiled with Microsoft Visual C++ 64bit compiler. The simulations were performed with a Microsoft HPC cluster with various Intel Xeon CPUs, consequently the reported timings are not accurate.

The cross-checking 10 bits results in Table 1 matched the results in JCTVC-N0168 [1].

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |  |  |
| Class B | -6.6% | -4.3% | -6.9% | -8.4% | -6.6% | -9.9% |  |  |  |
| Overall (Test vs Ref) | -6.6% | -4.3% | -6.9% | -8.4% | -6.6% | -9.9% |  |  |  |
| Overall (Test vs single layer) | 14.9% | 17.8% | 14.3% | 12.6% | 14.4% | 9.8% |  |  |  |
| Overall (Ref vs single layer) | 23.0% | 23.3% | 22.7% | 23.0% | 22.8% | 21.8% |  |  |  |
| Overall (Test EL+BL vs single EL+BL) | -21.4% | -19.5% | -21.9% | -31.8% | -30.8% | -33.5% |  |  |  |
| EL only (Test vs Ref) | -11.2% | -8.9% | -11.4% | -20.5% | -18.4% | -21.8% |  |  |  |
| Enc Time[%] | 88.2% |  |  | 95.9% |  |  |  |  |  |
| Dec Time[%] | 105.5% |  |  | 109.7% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Not matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! |  |  |  | #VALUE! | #VALUE! | #VALUE! |
| Class B | -4.4% | -1.7% | -4.3% | -8.1% | -5.2% | -8.6% | -5.6% | -3.5% | -5.6% |
| Overall (Test vs Ref) | -4.4% | -1.7% | -4.3% | -8.1% | -5.2% | -8.6% | -5.6% | -3.5% | -5.6% |
| Overall (Test vs single layer) | 27.5% | 34.5% | 28.7% | 26.7% | 35.6% | 27.9% | 27.5% | 33.0% | 27.9% |
| Overall (Ref vs single layer) | 33.3% | 36.8% | 34.6% | 38.0% | 43.3% | 40.0% | 35.2% | 38.0% | 35.5% |
| Overall (Test EL+BL vs single EL+BL) | -12.7% | -8.1% | -11.9% | -23.3% | -18.1% | -22.6% | -16.9% | -13.3% | -16.6% |
| EL only (Test vs Ref) | -7.3% | -4.6% | -7.2% | -17.4% | -14.4% | -17.6% | -9.7% | -7.6% | -9.6% |
| Enc Time[%] | 98.8% |  |  | 88.8% |  |  | 99.1% |  |  |
| Dec Time[%] | 112.7% |  |  | 119.1% |  |  | 151.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Not matched |  |  | Matched |  |  |

Table 1: The 10 bits results matched the results from JCTVC-N0168 [1].

# References

- Philippe Bordes, Pierre Andrivon, Patrick Lopez ,Franck Hiron, “AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results”, JCTVC-N0168_r2, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

- P.Bordes, P.Andrivon, R. Zakizadeh, “AHG14: Color Gamut Scalable Video Coding using 3D LUT,” JCTVC-M0197, 13th JCTVC Meeting, Incheon, KR, 18–26 Apr. 2013.

- Pierre Andrivon, Philippe Bordes, “AHG14: Wide Color Gamut Test Material Creation”, JCTVC-N0163, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

- X.Li, J.Boyce, P.Onno, Y.Ye, “Common SHM test conditions and software reference configurations,” JCTVC-M1009, 13th Meeting: Incheon, KR, 18–26 Apr. 2013.