| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0275 WG11 Doc: m29926 |
| --- | --- |

| Title: | AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than or equal to 14 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-858-845-1795 kimse@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In the current transform-skip design the least significant bits of residual information are discarded when the source bit depth is greater than or equal to 14. This unnecessarily restricts the transform-skip mode to operating at high fidelity to the original signal.  This contribution proposes a modified scaling factor for transform-skip blocks to support high fidelity coding of residuals for source bit depths 14 and higher. The scaling factor is modified for only transform-skip blocks and no change is made for transform blocks.

Evaluating the proposed approach with HM-10.1+RExt3.1 anchor for 14-bit source data at QP=-36, -32, -28, -24 results in average luma BD rate of -8.5% and -7.1% for all-intra and random-access configurations respectively.

# Introduction

Existing profiles of the HEVC standard supports source bit depths ranging from 8-bits to 10-bits (inclusive). The current range extension working draft aims at supporting higher bit depths. However for source bit depths greater than equal to 14-bits the least significant bits of residual information is discarded.

In order to support higher bit depths and address the precision loss at low QP in higher bit depth coding, the effects of transform matrix precision were studied in [1]. The followings were then proposed to be changed to the transform to support higher bit depths.

- The transform matrix precision should be at least bitDepth - 2

- MAX_TR_DYNAMIC_RANGE should be at least bitDepth + 5

In this proposal, we consider an approach that does not change the existing transform.  Instead, we only modify the scaling factor for transform-skip block to support higher bit depths.

# Problem Description

## Inverse transform skip

In the HEVC specification, the output of the inverse transform skip process for dequantized coefficient values d[x][y] is determined based on the following equations listed below:

r[ x ][ y ] = ( d[ x ][ y ]  <<  7 )	(8-267)

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-268)

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-269)

In Equation 8-269, if source bit depth >= 14, then bdShift <=6. Hence, a subset of LSB bits in r[ x ][ y ] are always set to 0. The table below lists these bit positions:

| Source bit depth | bdShift | Bit positions always set to 0 (LSB position index=0) |
| --- | --- | --- |
| 14 | 6 | 0 |
| 15 | 5 | 0, 1 |
| 16 | 4 | 0,1, 2 |

## Forward transform skip

The corresponding forward transform skip process is then:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC

bdShift = bdShift – 7

r[ x ][ y ] = ( r[ x ][ y ] + ( 1 << bdShift ) )  >>  bdShift

From the forward transform skip equations it can be seen that for source bit depth >= 14; bdShift <= 6. As a result, within the forward transform skip process the residual LSBs are discarded. This manifests as LSBs being set to 0 within the inverse transform skip process. This loss in residual information bits prevents transform skip from achieving higher fidelity.

# Proposed solution

The following modification prevents the loss of LSBs within the transform skip process:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-268)

When transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the value of bdShift is modified as follows:

bdShift = Max( 7, bdShift )	(8-xxx)

An alternative solution would be to move the original bdShift calcualtion prior to the first left bit-shift and limit the left bit-shift as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  Min( 7, bdShift ) ) 	(8-267)

Note, both the above listed approaches result in the same output for the inverse transform skip process. The working draft changes corresponding to the above two options is listed at the end of the document.

# Results

The proposed method has been implemented based on Range Extension software version 3.0. The detailed test conditions are described in Table 1 and the coding performance has been compared in Table 2. In figure 1 and figure 2, the rate-distortion performance has been shown for all intra and random access.

Table 1 Test conditions

| Test Sequences (12 bits source extended to 14 bits) | RGB : Traffic (2560x1600) YUV420 : CrowdRun (1920x1080), DucksTakeOff (1920x1080), InToTree (1920x1080), OldTownCross (1920x1080), ParkJoy (1920x1080) |
| --- | --- |
| QP | -36, -32, -28, -24 |
| Note that source bit depth is extened to 14 bits by adding Gaussian LSB to 12 bit source sequence. |  |

Table 2 Comparison of coding performance between anchor and proposed method

| AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V |
| -8.5% | -8.3% | -3.5% | -7.1% | -33.0% | -61.8% |

Figure 1 RD curve (All Intra, ParkJoy)

Figure 2 RD curve (Random Access, ParkJoy)

Supplement two experiments have been performed using 16 bit test sequences used in [1].

Table 2 Test conditions

| Test Sequences (16 bits source) | RGB444 : CrowdRun (1920x1080), DucksTakeOff (1920x1080), InToTree (1920x1080), OldTownCross (1920x1080), ParkJoy (1920x1080) |
| --- | --- |
| QP | -36, -32, -28, -24 (14 bits test) * 2 LSB are discarded -42, -38, -34, -30 (15 bits test) * 1 LSB is discarded |

Figure 3 RD curve (All Intra, InToTree, 14 bit tesing from 16 bit source)

Figure 4 RD curve (All Intra, InToTree, 15 bit tesing from 16 bit source)

Figure 5 RD curve (All Intra, ParkScene, 15 bit tesing from 16 bit source)

# References

- K. Sharman, N. Saunders, J. Gamei, “AHG5: Range Extensions and High Bit Depths”, JCTVC-M0178, Incheon, KR, April 2013.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Working Draft Text:

Changes to the working draft text are highlighted in yellow:

Option 1:

### 8.6.2 Scaling and transformation process

…

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  7 )	(8-1)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-2)

When transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the value of bdShift is modified as follows:

bdShift = Max( 7, bdShift )	(8-xxx)

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-3)

Option 2:

### 8.6.2 Scaling and transformation process

…

- The scaling process for transform coefficients as specified in subclause 8.6.3 is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-xxx)

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  Min( 7,  bdShift ) )	(8-xxx)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-4)

…