| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 14th Meeting: Vienna, AT, 25 July – 2 Aug., 2013 | Document: JCTVC-N276-r1 |
| --- | --- |

| Title: | A hardware oriented implementation of HEVC encoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ryoji Hashimoto Seiji Mochizuki Kenichi Iwata 5-20-1 Josuihoncho, Kodaira, Tokyo 187-8588, Japan | Tel: Email: | +81-42-312-9038 ryoji.hashimoto.te@renesas.com seiji.mochizuki.zc@renesas.com kenichi.iwata.yb@renesas.com |
| Source: | Renesas Electronics Corporation |  |  |

_____________________________

# Abstract

This contribution presents a hardware oriented implementation of HEVC encoding, which makes control logic in encoder easier. A hardware encoder can extend input images to multiple of CTB size by utilizing conformance window syntax to simplify control logic. However, extension of images requests more bits to encode, especially in large CTB size. To minimize overhead of bits in extended area, truncation of coefficients in quantization and optimization of TU partitioning and process are adopted. As a result, adopted methods in developed hardware can save 6.3% the BD rate in maximum.

# Introduction

This contribution introduces a suitable method for hardware encoder based on HEVC coding tools. An image whose size is not multiple of CTB size requests complex control logic at right and bottom corner. While HM[1] extends input image to multiple of CU size, it is more suitable for a hardware encoder to extend input image to multiple of CTB size in terms of simplifying control logic. Extension to multiple CTB size and utilizing conformance window syntax can lead simple control logic. However, this requests more bits to encode, which causes degradation of coding efficiency in total and inhibits usage of larger CTB size. In order to minimize overheads, optimization of transform unit partitioning and truncation of quantized coefficients are adopted. Adopting these methods in a developed hardware codec can save 10% bitrate in maximum.

# Coefficient truncation methods

An image whose size is not multiple of CTB size requests complex logic at right and bottom corner. For example, CU size decision algorithm needs to select the size which does not exceed picture boundary. Extension of image size to multiple CTB size can avoid complex control. On the other hands, more bits are necessary to encode extended image, especially in large CTB size. This leads degradation of coding efficiency in total. However, extended area is not displayed in decoder side. In other words, users do not realize image quality of “Non-displayed area” in Figure 2-1 at all.

Motivated this tendency, truncation of quantized coefficients in extended area is useful for reducing bits without loss of image quality in displayed area.

As shown in Figure 2-1, the hardware, such as shown in Section 3, extends the image size to multiple of CTB size such as 32 and 64 by using padding. “Displayed area” is equal to size of original image, “Non-displayed area” is the extended area and not displayed in decoder side.

Based on position of top left sample in a TU, quantization process truncates all quantized coefficients in the TU to 0. If top left sample in TU locates “Non-displayed area”, quantization process truncates all quantized coefficients to 0. Otherwise, output of quantization process is not changed. Truncating quantized coefficients to 0 degrades image quality in “Non-displayed area” instead of reducing bits. However, users do not realize this at all.

Moreover, TU size decision algorithm does not select the TU size which exceeds picture boundary. This increases the number of TU which locates in non-displayed area. In other words, increasing the number of coefficients to be truncated can reduce more bits.

Figure 2-1 Example of extended image

# Specification of target hardware

An example of hardware implementation which extends image size to CTB size is as shown in following tables. Table 3-1 and Table 3-2 show the specification of the hardware and supported tools, respectively.

Table 3-1 Specification of hardware

| Supported function | Encode/Decode |
| --- | --- |
| Profile/Level | Main/Level5.1 |
| Maximum resolution | 4096x2176@30 fps |
| Maximum bitrate | 120 Mbps |
| Operation frequency | 260MHz for 4096x2176@30fps |
| Bus I/F | AMBA3 |

Table 3-2 List of supported tools

| Tool name | Encode | Decode |
| --- | --- | --- |
| Tile | - | O |
| Wave front | - | O |
| 64x64 CTB | O | O (also supports 32x32, 16x16 CTB) |
| Variable CU size | O | O |
| Variable TU | O | O |
| AMP | - | O |
| SAO | - | O |
| Weighted prediction | - | O |

# Experimental results

The three conditions are compared in this section with HM11.0. The 1st one (condition 1) is “no extension of image size”, which is extension to multiple of CU size. The 2nd one (condition 2) is “extension of image size to multiple of CTB size without coefficient truncation”. The last one (condition 3) is “extension of image size to multiple of CTB size with coefficient truncation.” In these experiments, the right or bottom pixel in input image is copied by padding process.

Table 4-3, Table 4-4 and Table 4-5 show the results comparing condition 1 and 2, 1 and 3, and 2 and 3, respectively. From these results, proposed method (condition 3) can save -0.86%/-1.26%/-2.40% BD rate in comparison to condition 2 and have 0.15%/0.92%/0.93% overheads in comparison to condition 1. The maximum BD rate reduction is 6.3% in a sequence “BQ Square” with LP-main condition.

Table 4-3 BD-rate by condition2 against condition 1

|  | AI-Main |  |  | RA-Main |  |  | LP-Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |
| Class B | 0.22 | -0.08 | -0.04 | 0.29 | 0.19 | 0.21 | 0.48 | 0.22 | 0.08 |
| Class C | 1.19 | 0.99 | 1.02 | 2.85 | 2.55 | 2.58 | 3.39 | 3.17 | 3.12 |
| Class D | 2.68 | 1.94 | 2.07 | 6.37 | 4.90 | 5.44 | 7.29 | 5.46 | 5.96 |
| Overall | 1.03 | 0.73 | 0.78 | 2.25 | 1.81 | 1.95 | 3.47 | 2.73 | 2.77 |

Table 4-4 BD-rate by condition3 against condition 1

|  | AI-Main |  |  | RA-Main |  |  | LP-Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |
| Class B | 0.11 | -0.02 | 0.01 | 0.43 | 0.28 | 0.20 | 0.26 | 0.15 | 0.17 |
| Class C | 0.10 | 0.14 | 0.15 | 1.04 | 1.12 | 1.21 | 0.62 | 0.99 | 0.77 |
| Class D | 0.32 | 0.45 | 0.48 | 2.34 | 2.65 | 2.94 | 2.07 | 2.50 | 2.53 |
| Overall | 0.15 | 0.14 | 0.17 | 0.92 | 0.97 | 1.04 | 0.93 | 1.12 | 1.07 |

Table 4-5 BD-rate by condition3 against condition 2

|  | AI-Main |  |  | RA-Main |  |  | LP-Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |
| Class B | -0.11 | 0.06 | -0.05 | 0.14 | 0.09 | -0.01 | -0.22 | -0.07 | 0.09 |
| Class C | -1.08 | -0.85 | 1.02 | -1.76 | -1.40 | -1.34 | -2.68 | -2.11 | -2.28 |
| Class D | -2.30 | -1.46 | 2.07 | -3.78 | -2.15 | -2.37 | -4.83 | -2.81 | -3.25 |
| Overall | -0.86 | -0.57 | 0.78 | -1.26 | -0.81 | -0.88 | -2.40 | -1.54 | -1.67 |

Moreover, condition 2 and condition 3 is compared with various major image sizes such as CIF in LP-main condition. In comparison, all class B image cut downed to test size are used. For example, to compare with CIF size, top left 352x288 samples in a class B image is used. Figure 4-2 and Figure 4-3 show the average BD rate reduction in all class B sequences with various image sizes and average BD rate reduction based on the rate of pixels in Non-displayed area, respectively. The rate is defined with following equations.

rate = (PIX_IN_ALL - PIX_IN_DISPLAY) / PIX_IN_ALL * 100

PIX_IN_DISPLAY = pic_width * pic_height

PIX_IN_ALL = ((pic_width + ctb_size - 1) / ctb_size) * ((pic_height + ctb_size - 1) / ctb_size) * (ctb_size * ctb_size),

where pic_width and pic_height is picture width and picture height in original image, respectively.

From these results, as rate is higher, proposed method can improve more BD rate.

Figure 4-2 Average BD rate reduction with various image size in LP-main condition

Figure 4-3 Average BD rate reduction based on rate of pixels in Non-displayed area

# Conclusion

This contribution has provided hardware oriented encoding method. In order to control encoding easily, proposed hardware extends images to multiple of CTB size by utilizing conformance window syntax. To minimize increase of bits in extended area, proposed hardware adopts truncation of quantized coefficients in quantization process and optimization of TU size partitioning. As a result, adopted methods can save 6.3% BD rate in maximum compared with the encoder without coefficient truncation and TU size optimization.

# Patent rights declaration(s)

Renesas Electronics Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1]  K. McCann, B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan, “High Efficiency Video Coding (HEVC) Test Model 11 (HM 11) Encoder Description,” JCTVC-M1002, Meeting Inchon, KR, 18-26, April, 2013.