| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0280r1 |
| --- | --- |

| Title: | AHG13: Cross-check of JCTVC-N0265 on Complexity reduction of the up-sampling process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard François Canon Research France | Tel: Email: | Edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This document provides the cross-check results of the proposal JCTVC-N0265 on Complexity reduction of the up-sampling process from Ericsson. All the BD-rate results are reported to match exactly those provided by the proponent. It is asserted that observed BDR loss when using the 4-tap luma upsampling filters looks reasonable considering the benefit of reduced average complexity of the encoding/decoding processes. It is also suggested to evaluate possible alternate 4-tap filters.

# Introduction

JCTVC-N0265 proposes to replace the current 8-tap filter used for the upsampling of luma components by a 4-tap filter in SHVC. The 4-tap filter has been designed from the corresponding SHM-2.0 8-tap filters as follows:

Consider 8 tap filter: f_8tap = [f0, f1, f2, f3, f4, f5, f6, f7]

The 4 tap filter is derived as: f_4tap = [(f0+f1+ f2), f3, f4, (f5+f6+f7)]

They are shown in the table below.

| Phase shift | SHM-2.0 up-sampling filter | Proposed 4 taps up-sampling filter |
| --- | --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 | { 0, 64, 0, 0}/64 |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1}/64 | { -8, 52, 26, -6}/64 |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 | {-8, 40, 40, -8}/64 |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1}/64 | {-6, 26, 52, -8}/64 |

One flag is added in the SPS extension to indicate if the filter length for up-sampling is fixed for the whole sequence. Another flag is added in both SPS extension and slice header to select 4-tap or 8-tap filter for luma up-sampling.

# Cross-checking description

## Implementation verification

The software patch and results to be cross-checked have been delivered by Ericsson on July 05, 2013.

Two tests are performed, one with 4-tap applying both for intra and inter luma-blocks (test 1), one with 4-tap applying only for inter luma-blocks (test 2).

The following macro setting is used to enable the proposed tool:

Test 1

#define LUMA_UP_FILTER_SELECTION         1      // currently 4 tap filtering is always forced

#define FOUR_TAP_FUNCTION                1      // separate 4 tap and 8 tap filter functions

#if LUMA_UP_FILTER_SELECTION

#define SELECT_8TAP_FOR_I_SLICE_BASE     0      // when 1 use 8 tap when there is i-slice in base, need to adjust complexity assessment taps per slice if evaluating this

#endif

Test 2

#define LUMA_UP_FILTER_SELECTION         1      // currently 4 tap filtering is always forced

#define FOUR_TAP_FUNCTION                1      // separate 4 tap and 8 tap filter functions

#if LUMA_UP_FILTER_SELECTION

#define SELECT_8TAP_FOR_I_SLICE_BASE     1      // when 1 use 8 tap when there is i-slice in base, need to adjust complexity assessment taps per slice if evaluating this

#endif

The software implementation has been verified and is confirmed to be in line with the tool description.

## Simulations

Tests were done using the SHM2 test conditions. The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

Two tests are performed, one with 4-tap applying both for intra and inter luma-blocks (test 1), one with 4-tap applying only for inter luma-blocks (test 2).

The full cross-check results are in the attached xls files. Results provided in JCTVC-N0265 are confirmed.

## Complexity assessment

The complexity modules, integrated into the software, have been checked. To the best knowledge of the cross-checker, no major issue has been detected in this implementation. Complexity results, gathered only for of test 1, generated when decoding the streams with these complexity modules are confirmed by the cross-checker (see attached file JCTVC-N0280_xCheck_N0265_avgComplexity.xls).

# Conclusions

The implementation, BDR results and complexity evaluation of JCTVC-N0265 presented by Ericsson are confirmed by the cross-checker.

The observed BDR loss when using the 4-tap luma upsampling filters looks reasonable considering the benefit of reduced average complexity of the encoding/decoding processes. There were discussions with the proponent about the potential usage of the existing 4-tap chroma filters. The proponent asserts that chroma filters produce much smoother signal and leed to higher losses. It would be worth to study other 4-tap filters (such as the filters from SVC specification).