| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0281 WG11 Doc: m29932 |
| --- | --- |

| Title: | Non-RCE2 Rice parameter extension for transform-skip blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-858-845-1795 kimse@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution presents two modifications to cRiceParam update process of coeff_abs_level_remaining based on rice-parameter signaling method in JCTVC-N0268. The two modifications are: (1) signaling cRiceParam update method in slice level  and (2) cRiceParam increase in maximum value from 4 to 6. Specifically rice parameter expansion is considered to deal with large level values from lossless coding and high bitrate coding for high bit depths.

# Motivation and Proposed text changes

Transform-skip blocks play an important role for screen content coding. Specifically it gives more impact to coding performance at high bitrate coding.  Considering some applications like lossless and low QP coding for high depths such as greater than 8 bits the maximum rice-parameter needs to be increased. In this contribution, rice parameter maximum range is increased from 4 to 6. In addition, a modified rice parameter update is considered based on the expanded rice parameter range.

# Proposed Metod

In the proposed method, rice parameter (cRiceParam) is signalled for each transform-skip block and the existing rice-parameter update process is removed as shown in [1] [2]. Specifically, rice parameter (cRiceParam) is signalled from the given set {0, 2, 4, 6}. The binarization for the cRiceParm is shown.

Table 1 TU binarization for extended cRiceParam

| Rice-parameter | Bin string |
| --- | --- |
| 0 | 1 |
| 2 | 01 |
| 4 | 001 |
| 6 | 000 |

# Results

Proposed methods were tested under common test condition described in [3].  Table 2 and Table 3 show performance of the proposed method for lossless and lossy test conditions, respectively.

Table 2 Result (Lossless)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.4% | 31.7 | 32.4 | -1.2% | 49.8 | 50.3 | -0.7% |
| Class B | 2.2 | 2.3 | -0.5% | 2.6 | 2.6 | -1.3% | 2.6 | 2.6 | -1.3% |
| SC RGB 444 | 10.1 | 12.2 | -14.9% | 100.4 | 123.3 | -12.6% | 381.6 | 479.5 | -11.9% |
| SC YUV 444 | 11.4 | 12.8 | -7.9% | 128.9 | 146.3 | -7.2% | 325.6 | 451.0 | -7.5% |
| RangeExt | 2.4 | 2.4 | -3.2% | 2.5 | 4.5 | -2.1% | 2.5 | 4.5 | -2.1% |

Table 3 Result (Lossy)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.3% | -0.3% | -0.8% | -0.6% | -0.6% | -1.2% | -1.1% | -1.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.9% | -1.9% | -1.9% | -4.3% | -4.2% | -4.2% | -7.2% | -7.5% | -7.5% |
| SC YUV 444 | -0.1% | 0.0% | 0.0% | -0.6% | -0.5% | -0.5% | -2.0% | -2.0% | -2.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.0% | -0.1% | -0.4% | -0.2% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.6% | -1.6% | -1.6% | -3.6% | -3.5% | -3.5% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.1% | -0.5% | -0.4% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.3% | 0.0% | 0.0% | 0.2% | 0.1% |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -1.6% | -1.6% | -1.6% | -3.9% | -3.8% | -3.9% |  |  |  |
| SC YUV 444 | 0.1% | 0.2% | 0.2% | -0.4% | -0.3% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |

- References

- S.H. Kim, A. Segall, “Non-RCE1/Non-RCE2: Simplified level coding for Transform skip and lossless coding”, JCTVC-M0304, Incheon, KR, April 2013.

- S.H. Kim, K. Misra, A. Segall, “RCE2 Test A.1: Rice parameter signaling for transform-skip blocks”, JCTVC-N0268, Vienna, AT, July - 2 Aug. 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, Incheon, KR, April 2013.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).