| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0282 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On handling of filtered inter-layer reference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu, and Shawmin Lei MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Tel: Email: | polin.lai@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents high-level syntax and decoding process for handling situations wherein filtered inter-layer references are inserted into the RPS to be utilized as reference pictures in the reference picture lists. The presented changes allow both filtered and non-filtered reference pictures from the same layer to co-exist in RPS and reference picture lists, facilitating block-level switching between filtered and non-filtered reference pictures from the same layer.

# Introduction

In SCE3 [1], multiple coding tools utilize filtered inter-layer reference picture from the base-layer (BL) to improve coding efficiency. In one scenario, to encode a given enhancement-layer (EL) picture, two versions of a reference pictures from the same BL, filtered and non-filtered, are used for inter-layer prediction. Therefore, prediction using filtered and non-filtered BL reference picture can be switched at PU-level. In these proposed coding tools, the insertion of filtered BL reference picture is hardcoded at encoder and decoder, such that the same procedure is followed for correct handling of the reference pictures. To fully and properly support this functionality, this contribution presents related changed in high-level syntax and decoding process in the spec.

# Considerations

Follow closely the design principle of handling inter-layer reference pictures in the current spec [2].

High-level control specified in VPS, indicating which (if any) layers of the inter-layer reference pictures may be filtered.

Slice-level refinement of the management of filtered inter-layer reference specified in slide header, via controlling the construction of inter-layer RPS.

Given modified RPS which copes the filtered inter-layer reference pictures, initialization of the default reference List 0 temp and List 1 temp are modified.

# Proposal

Additions are highlighted and removals are strikethrough.

## Video parameter set extension syntax and semantics

- F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| max_one_active_ref_layer_flag | u(1) |
| filtered_inter_layer_ref_present_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) { |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| if(filtered_inter_layer_ref_present_flag) |  |
| direct_dependency_filtered_flag[i][j] | u(1) |
| } |  |
| single_layer_for_non_irap_flag | u(1) |
| } |  |

filtered_inter_layer_ref_present_flag equals to 1 specifies that there may exit at least one direct reference layer in which filtered inter-layer reference picture is used for inter-layer prediction. This is a present flag to indicate the existence of the corresponding syntax and decoding process for filtered inter-layer reference pictures. filtered_inter_layer_ref_present_flag equals to 0 indicates there is no filtered direct reference layer.

direct_dependency_filtered_flag[i][j] equal to 0 specifies that there is no filtered version of the layer with index j to be used as a direct reference layer for the layer with index i. direct_dependency_filtered_flag[ i ][ j ] equal to 1 specifies that a filtered version of the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_filtered_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

F.7.4.3.1.1 Video parameter set extension semantics … The variables NumSamplePredRefLayers[ i ], NumMotionPredRefLayers[ i ], SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ], NumDirectRefLayers[ i ], DirectRefLayerIdx[ i ][ j ], RefLayerId[ i ][ j ], RefLayerIdFiltFlag[ i ][ j ], MotionPredRefLayerId[ i ][ j ], and SamplePredRefLayerId[ i ][ j ] are derived as follows: for( i = 0; i < 64; i++ ) { NumSamplePredRefLayers[ i ] = 0 NumMotionPredRefLayers[ i ] = 0 NumDirectRefLayers[ i ] = 0 for( j = 0; j < 64; j++ ) { SamplePredEnabledFlag[ i ][ j ] = 0 MotionPredEnabledFlag[ i ][ j ] = 0 RefLayerId[ i ][ j ] = 0 RefLayerIdFiltFlag[ i ][ j ] = 0 SamplePredRefLayerId[ i ][ j ] = 0 MotionPredRefLayerId[ i ][ j ] = 0 } } for( i = 1; i <= vps_max_layers_minus1; i++ ) { iNuhLId = layer_id_in_nuh[ i ] for( j = 0; j < i; j++ ) if( direct_dependency_flag[ i ][ j ] ) { DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ] RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ] RefLayerIdFiltFlag [ iNuhLId ][ NumDirectRefLayers[ iNuhLId ] ] = direct_dependency_filtered_flag[i][j] SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 ) NumSamplePredRefLayers[ iNuhLId ] += SamplePredEnabledFlag[ iNuhLId ][ j ] MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 ) NumMotionPredRefLayers[ iNuhLId ] += MotionPredEnabledFlag[ iNuhLId ][ j ] } }

## General slice segment header semantics

- G.7.4.7.1 General slice segment header semantics The variables RefPicLayerId[ i ] and flags RefPicLayerIdFiltFlag[i] of for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, NumActiveMotionPredRefLayers, and ActiveMotionPredRefLayerId[ j ] for each value of j in the range of 0 to NumActiveMotionPredRefLayers − 1, inclusive, are derived as follows: for( i = 0, j = 0; i < NumActiveRefLayerPics; i++) RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] RefPicLayerIdFiltFlag[i] = RefLayerIdFiltFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] if( MotionPredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] ) ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] } NumActiveMotionPredRefLayers = j All slices of a picture shall have the same value of inter_layer_pred_layer_idc[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

In the above manner, for each inter-layer reference picture specified by RefPicLayerId[ i ], a corresponding flag RefPicLayerIdFiltFlag[i] is derived from RefLayerIdFiltFlag to indicate whether a filtered version of the inter-layer reference picture specified by RefPicLayerId[ i ] is used.

## RPS and Reference Pic Lists

- Method 1: With a new RPS, RefPicSetInterLayerFiltered

In this first method, a new RPS, RefPicSetInterLayerFiltered, is used to accommodate filtered inter-layer reference pictures. The RefPicSetInterLayerFiltered is constructed together with RefPicSetInterLayer, by checking RefPicLayerIdFiltFlag[i].

After both RPS RefPicSetInterLayer and RefPicSetInterLayerFiltered are constructed, the initial default RefPicListTemp0 and RefPicListTemp1 are created. There are various ways of ordering the different RPS. For example:

RefPicListTemp0: RPS Temporal (before/after/long) + RefPicSetInterLayerFilt + RefPicSetInterLayer

RefPicListTemp1: RPS Temporal (before/after/long) + RefPicSetInterLayer + RefPicSetInterLayerFilt

Or

RefPicListTemp0: RPS Temporal (before/after/long) + RefPicSetInterLayer + RefPicSetInterLayerFilt

RefPicListTemp1: RPS Temporal (before/after/long) + RefPicSetInterLayerFilt + RefPicSetInterLayer

Or

RefPicListTemp0: RPS Temporal (before/after/long) + RefPicSetInterLayerFilt + RefPicSetInterLayer

RefPicListTemp1: RPS Temporal (before/after/long) + RefPicSetInterLayer

In the text below, the first RPS ordering example is used.

- G.8.1.2 Decoding process for inter-layer reference picture set …. Initialize k = 0. for( i = 0; i < NumActiveRefLayerPics; i++ ) { if( there is a picture picX in the DPB that is in the same access unit as the current picture and has nuh_layer_id equal to RefPicLayerId[ i ] ) { an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] given as inputs RefPicSetInterLayer[ i ] = rsPic RefPicSetInterLayer[ i ] is marked as "used for long-term reference" if (RefPicLayerIdFiltFlag[i]) { RefPicSetInterLayerFiltered [ k ] = rsPic_filtered (derived by invoking the “filtering process”) RefPicSetInterLayerFiltered [ k ] is marked as "used for long-term reference" k++ } } else RefPicSetInterLayer[ i ] = "no reference picture" } NumActiveRefLayerPicsFilt = k - G.8.1.3 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 … for( i = 0; i < NumActiveRefLayerPics; i++ ) RefPicSetInterLayer[ i ] is marked as "used for short-term reference" for( i = 0; i < NumActiveRefLayerPicsFilt; i++ ) RefPicSetInterLayerFiltered[ i ] is marked as "used for short-term reference" - F.7.4.7.2 Reference picture list modification semantics … NumPocTotalCurr = 0 for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++) if(UsedByCurrPicS0[ CurrRpsIdx ][ i ] ) NumPocTotalCurr++ for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++) (G-1) if(UsedByCurrPicS1[ CurrRpsIdx ][ i ] ) NumPocTotalCurr++ for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) if( UsedByCurrPicLt[ i ] ) NumPocTotalCurr++ NumPocTotalCurr += NumActiveRefLayerPics NumPocTotalCurr += NumActiveRefLayerPicsFilt G.8.3.4 Decoding process for reference picture lists construction … rIdx = 0 while( rIdx < NumRpsCurrTempList0 ) { if( InterRefEnabledInRPLFlag ) { for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) (G-2) RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ] for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ] } for( i = 0; i < NumActiveRefLayerPicsFilt; rIdx++, i++ ) RefPicListTemp0[ rIdx ] = RefPicSetInterLayerFiltered[ i ] for( i = 0; i < NumActiveRefLayerPics; rIdx++, i++ ) RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] } … rIdx = 0 while( rIdx < NumRpsCurrTempList1 ) { if( InterRefEnabledInRPLFlag ) { for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ] for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) (G-3) RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ] for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ] } for( i = 0; i< NumActiveRefLayerPics; rIdx++, i++ ) RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ] for( i = 0; i< NumActiveRefLayerPicsFilt; rIdx++, i++ ) RefPicListTemp1[ rIdx ] = RefPicSetInterLayerFiltered[ i ] }

Note that in this method 1, if there are multiple filtered inter-layer reference pictures, i.e. from different reference layers, the default RefPicListTemp0/1 will be initialized with filtered inter-layer reference pictures order together, separated from the non-filtered inter-layer reference pictures. For example “filtered ILRP-L1, filtered ILRP-L0, ILRP-L1, ILRP-L0”. If it is desired to put filtered and non-filtered reference pictures from the same layer together, Ref Pic List Modification (RPLM) commands are needed to rearrange them.

- Method 2: Putting filtered inter-layer references within RefPicSetInterLayer

In this second method, RefPicSetInterLayer is used to accommodate both filtered and non-filtered inter-layer reference pictures. By checking RefPicLayerIdFiltFlag[i], filtered inter-layer reference pictures are inserted. Each of them can be inserted before the corresponding non-filtered one, or after the corresponding non-filtered one.

Thus the initial default RefPicListTemp0 and RefPicListTemp1 are constructed the same way as currently done in G.8.3.4, while the ordering of all the inter-layer reference pictures (filtered and non-filtered) will the same for RefPicListTemp0 and RefPicListTemp1since they came from the same RefPicSetInterLayer.

In the text below, the filtered inter-layer reference pictures are inserted after the corresponding non-filtered one in the construction of RefPicSetInterLayer.

- G.8.1.2 Decoding process for inter-layer reference picture set …. Initialize k = 0. for( i = 0; i < NumActiveRefLayerPics; i++ ) { if( there is a picture picX in the DPB that is in the same access unit as the current picture and has nuh_layer_id equal to RefPicLayerId[ i ] ) { an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] given as inputs RefPicSetInterLayer[ k ] = rsPic RefPicSetInterLayer[ k ] is marked as "used for long-term reference" k++ if (RefPicLayerIdFiltFlag[i]) { RefPicSetInterLayer [ k ] = rsPic_filtered RefPicSetInterLayer [ k ] is marked as "used for long-term reference" k++ } } else RefPicSetInterLayer[ k ] = "no reference picture" } NumActiveRefLayerPicsAddFilt = k - G.8.1.3 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 … for( i = 0; i < NumActiveRefLayerPics NumActiveRefLayerPicsAddFilt; i++ ) RefPicSetInterLayer[ i ] is marked as "used for short-term reference" - F.7.4.7.2 Reference picture list modification semantics … NumPocTotalCurr = 0 for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++) if(UsedByCurrPicS0[ CurrRpsIdx ][ i ] ) NumPocTotalCurr++ for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++) (G-4) if(UsedByCurrPicS1[ CurrRpsIdx ][ i ] ) NumPocTotalCurr++ for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) if( UsedByCurrPicLt[ i ] ) NumPocTotalCurr++ NumPocTotalCurr += NumActiveRefLayerPics NumActiveRefLayerPicsAddFilt

Note again that the ordering of all the inter-layer reference pictures (filtered and non-filtered) will the same for RefPicListTemp0 and RefPicListTemp1since they came from the same RefPicSetInterLayer.

# Conclusions

To fully and properly support this functionality of having filtered inter-layer references co-exist with non-filtered ones in the reference picture list, this contribution presents related changed in high-level syntax and decoding process in the spec. Follow closely the design principle of handling inter-layer reference pictures in the current spec, high-level control is specified in VPS, with Slice-level refinement of the management of filtered inter-layer reference specified in slide header. Given modified RPS which copes the filtered inter-layer reference pictures, initialization of the default reference List 0 temp and List 1 temp are modified.

# References

[1] “HEVC Scalable Extensions Core Experiment 3 (SCE3): Inter-layer filtering”, JCTVC-M1103, Apr. 2013

[2] “SHVC working draft 2”, JCTVC-M1103, Apr. 2013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).