| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 24 Jul. – 02 Apr. 2013 | Document: JCTVC-N0283 |
| --- | --- |

| Title: | Aspect ratio scalability based on SHM-2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yiqun Liu, Joern Ostermann Appelstr. 9A 30165 Hannover | Tel: Email: | 0049 511 7625308 yiqun.liu@tnt.uni-hannover.de |
| Source: | Leibniz Universitaet Hannover |  |  |

_____________________________

# Abstract

This document represents one approach for scalable extension considering different aspect ratio. It is assumed here that a SDTV base layer represents a vertical outtake of a HDTV signal. The BaseLayerOffset describes the horizontal position of the base layer (SDTV) with respect to the enhancement layer (HDTV). In this approach, inter-layer reference (ILR) was generated with a given BaseLayerOffset written in sequence parameter set (SPS) for enhancement layer and the use of ILR in REF_LIST was also according to ILR restricted. The implementations were integrated into SHM-2.0 software. The implemented scheme achieves a luma BD-rate reduction of 6.5% compared to simulcast. The proposed speeding up method brings an average of 2.0% reduction of encoding time.

# Introduction

The requirement of aspect ratio scalability was first raised in [1]. In scalable extension of HEVC, spatial scalability with factor 2 or 1.5 and SNR scalability are implemented so far. This contribution, however, aims at aspect ratio scalability between e.g. SDTV (4:3) and HDTV (16:9). This approach is based on SHM-2.0 with the addition of an offset which was written into SPS to show the horizontal distance between up-sampled SDTV frame and HDTV frame.

Since no relevant standard sequence for SDTV (4:3) is available, the generation of base layer is first explained. In this process, BaseLayerOffset could be chosen according to the wish of applications and would be later stored into a configuration file.

In order to keep the usage of the RefIdx framework, up-sampled base layer frames should be adjusted to the resolution of the enhancement layer. A padding process with the aid of location information accessed from BaseLayerOffset is involved in this part.

For the enhancement layer coding, in case one CU does not belong to the co-located base layer, neither Inter-mode nor ILRuni-prediction should be used. Therefore, one extra bool variable bInsideUpsampledRegion is used during encoding.

# Base layer Generation

Base layer of the experiment was first down-sampled from the standard HDTV sequences (Basketball-Drive, Kimono etc) according to [2]. In the next step, the undesired part in each frame in the sequence was cut with a changeable coefficient BaseLayerOffset, which shows the x-position of first pixel of the generated 4:3 frame in that of the down-sampled 16:9 frame (Fig. 1). This offset could also be set differently for each frame in the sequence.

Fig.1 Base layer cut out from down-sampled HDTV enhancement layer

Meanwhile BaseLayerOffsets were stored in configuration file “BaseLayerOffset.cfg”, which was used later by the encoder.

# Proposed Method

BaseLayerOffset was read from “BaseLayerOffset.cfg” at the very beginning of encoding and was later written in the SPS for enhancement layer. By decoding, this BaseLayerOffset is read from SPS for each enhancement layer and used also to generate ILR. Two major changes based on it were made in the process.

First, BaseLayerOffset was introduced in the function of base layer up-sampling. Due to the difference of aspect ratio, WidthEL is no longer twice of WidthBL. In order to preserve the functionality of up-sampling, a picture buffer with width of 2*WidthBL was temporarily allocated and it stored the up-sampled base layer of aspect ratio 4:3. After that, constant padding (either use 0, 128 or border pixel padding) was applied to left and/or right columns which were once cut in base layer generation. The particular method for padding may also be signalled in the SPS on a frame basis or otherwise. However, this is not considered here.

BaseLayerOffset gave the information about the exact position for padding. A padding result (Fig. 2) would be a frame of aspect ratio 16:9, which has the same resolution as enhancement layer and would be stored later at the last position in REF_LIST_0 and REF_LIST_1 as ILR. Thus, RefIdx was not influenced by the change from spatial scalability to aspect ratio scalability.

Encoder coded BaseLayerOffsets in SPS of enhancement layer by writing the number of frames to be coded in function codeSPS, followed by value of offsets for each frame from configuration file. This process was executed when first enhancement frame was encoded.

WRITE_CODE( pcSPS->getVPSId (), 4, "sps_video_parameter_set_id" );

WRITE_CODE( pcSPS->getMaxTLayers() - 1, 3, "sps_max_sub_layers_minus1" );

…

WRITE_FLAG( pcSPS->getUseStrongIntraSmoothing(), "sps_strong_intra_smoothing_enable_flag" );

WRITE_FLAG( pcSPS->getVuiParametersPresentFlag(), "vui_parameters_present_flag" );

if (pcSPS->getVuiParametersPresentFlag())

{ codeVUI(pcSPS->getVuiParameters(), pcSPS);}

if( pcSPS->getLayerId() > 0 ) {

WRITE_UVLC( pcSPS->getNumberOfFrames(), "number_of_frames" );

for(UInt i=0;i<pcSPS->getNumberOfFrames();i++)

{WRITE_UVLC( pcSPS->getBaseLayerOffset(i), "baselayeroffset[i]" );}

}

if( pcSPS->getLayerId() > 0 ) {

Window scaledWindow = pcSPS->getScaledRefLayerWindow();

WRITE_SVLC( scaledWindow.getWindowLeftOffset() >> 1, "scaled_ref_layer_left_offset" );

WRITE_SVLC( scaledWindow.getWindowTopOffset() >> 1, "scaled_ref_layer_top_offset" );

WRITE_SVLC( scaledWindow.getWindowRightOffset() >> 1, "scaled_ref_layer_right_offset" );

WRITE_SVLC( scaledWindow.getWindowBottomOffset() >> 1, "scaled_ref_layer_bottom_offset" );

}

…

Fig. 2 Base layer up-sampling and ILR generation

Decoder worked in the same way. Number of encoded frames and BaseLayerOffsets were first read from function parseSPS and then applied in base layer up-sampling with decoded base layer.

READ_UVLC( uiCode, "pps_pic_parameter_set_id"); pcPPS->setPPSId (uiCode);

READ_UVLC( uiCode, "pps_seq_parameter_set_id"); pcPPS->setSPSId (uiCode);

…

READ_FLAG( uiCode, "sps_strong_intra_smoothing_enable_flag" ); pcSPS->setUseStrongIntraSmoothing(uiCode);

READ_FLAG( uiCode, "vui_parameters_present_flag" ); pcSPS->setVuiParametersPresentFlag(uiCode);

if(pcSPS->getLayerId() > 0) {

READ_UVLC ( uiCode, "number_of_frames"); pcSPS->setNumberofFrames(uiCode);

Int *Offsettemp;

Offsettemp = new Int[pcSPS->getNumberOfFrames()];

for(UInt i=0;i<pcSPS->getNumberOfFrames();i++)

{ READ_UVLC ( uiCode, "baselayeroffset[i]"); Offsettemp[i] = uiCode; }

pcSPS->setBaseLayerOffset(Offsettemp);

}

if( pcSPS->getLayerId() > 0 ) {

Int iCode;

Window& scaledWindow = pcSPS->getScaledRefLayerWindow();

READ_SVLC( iCode, "scaled_ref_layer_left_offset" ); scaledWindow.setWindowLeftOffset(iCode<<1);

READ_SVLC( iCode, "scaled_ref_layer_top_offset" ); scaledWindow.setWindowTopOffset (iCode<<1);

READ_SVLC( iCode, "scaled_ref_layer_right_offset" ); scaledWindow.setWindowRightOffset (iCode<<1);

READ_SVLC( iCode, "scaled_ref_layer_bottom_offset" ); scaledWindow.setWindowBottomOffset (iCode << 1);

}

…

Padded values in decoder are identical as used in encoder, as encoder and decoder share the same base layer up-sampling function. Generated ILR pictures are the same as ILR from encoding and would also be saved in the reference picture lists REF_LIST_0 and REF_LIST_1 in the process of decoding. Thus, ILR pictures could be referred by executing decoding inter-prediction and ILRuni-prediction.

Second, BaseLayerOffset is used for encoder speed up. BaseLayerOffset indicates whether one CU may refer to this upsampled ILR in inter-prediction or ILRuni-prediction. Since constant padding areas (grey areas in Fig.(2)) in the ILR could rarely offer meaningful information or make accurate prediction, estimation with ILR there seems to be a waste in computation. Bool bInsideUpsampledRegion may be generated in xEncodeCu with the following optional code:

…

UInt uiLPelX = rpcBestCU->getCUPelX();

UInt uiRPelX = uiLPelX + rpcBestCU->getWidth(0) - 1;

UInt uiTPelY = rpcBestCU->getCUPelY();

UInt uiBPelY = uiTPelY + rpcBestCU->getHeight(0) - 1;

Bool bInsideUpsampledRegion = true;

if (rpcBestCU->getLayerId() > 0) {

UInt BaseLayerOffset = rpcBestCU->getSlice()->getSPS()->getBaseLayerOffset(rpcBestCU->getSlice()->getPOC());

UInt uiWidthBL = rpcBestCU->getSlice()->getBaseColPic()->getPicYuvOrg()->getWidth();

BaseLayerOffset <<= 1; uiWidthBL <<= 1;

if(uiRPelX < BaseLayerOffset ¦¦ uiLPelX >= BaseLayerOffset+uiWidthBL)

{bInsideUpsampledRegion = false; }

}

Int iBaseQP = xComputeQP( rpcBestCU, uiDepth );

Int iMinQP;

Int iMaxQP;

Bool isAddLowestQP = false;

…

If only bInsideUpsampledRegion is true, xCheckRDCostInter would take ILR pic into consideration while looking through REFLISTs and xCheckRDCostILRuni would not skip.

For xCheckRDCostInter, part of code is as followed:

…

Bool testIter = true;

TComPic* pcPic = pcCU->getSlice()->getRefPic( RefPicList(1 - iRefList), iRefIdxBi[1 - iRefList] );

if( !testILR && pcPic->isILR(pcCU->getLayerId()) && ((ePartSize == SIZE_2Nx2N)

¦¦!bInsideUpsampledRegion))

{ testIter = false;}

if(testIter) {

iRefStart = 0;

iRefEnd = pcCU->getSlice()->getNumRefIdx(eRefPicList)-1;

for ( Int iRefIdxTemp = iRefStart; iRefIdxTemp <= iRefEnd; iRefIdxTemp++ ) {

Bool testRefIdx = true;

pcPic = pcCU->getSlice()->getRefPic( RefPicList(iRefList) , iRefIdxTemp );

if( !testILR && pcPic->isILR(pcCU->getLayerId()) && ((ePartSize == SIZE_2Nx2N)

¦¦!bInsideUpsampledRegion))

{ testRefIdx = false;}

…

For xCheckRDCostILRuni, it looks:

…

Bool foundILR = false;

for( Int refIdx = 0; refIdx < pcCU->getSlice()->getNumRefIdx(eRefPicList); refIdx++ )

{

if( pcCU->getSlice()->getRefPic(eRefPicList, refIdx)->isILR(pcCU->getLayerId()) )

{

iRefIdxTemp = refIdx;

foundILR = true;

break;

}

if(!foundILR¦¦!bInsideUpsampledRegion)

{ continue; }

uiBitsTemp = uiMbBits[iRefList];

if ( pcCU->getSlice()->getNumRefIdx(eRefPicList) > 1 )

…

# Experiment Results

## Comparison of padding methods

The first experiment was about how padded value worked in the up-sampling function shared by encoder and decoder. In this contribution, 0, 128 and border pixel padding were compared in terms of Bitrate. The experiment was based on Basketball-drive with first 30 frames. Offsets defined here were from 50 to 79.

| Padding | Bit-rate (EL) | Y-PSNR | U-PSNR | V-PSNR | Encoding time |
| --- | --- | --- | --- | --- | --- |
| 0 | 100% | 40.4583 | 44.9289 | 46.3332 | 100% |
| 128 | -0.17% | +0.0016 | -0.0038 | -0.0164 | -0.14% |
| Value on Boundary | -0.10% | +0.0001 | -0.0035 | -0.0152 | -0.12% |

Table above shows that constant padding using 128 is better than padding with 0 or boundary value, in terms of bit-rate, Y-PSNR and Encoding time. Therefore, further simulation on aspect ratio scalability will be executed with padding constant 128.

Later another comparison was made based on test sequence Kimono. This time the result shows that padding constant does not play an important role in this approach. Y(0.0%), U(-0,1%) and V(-0.2%) is the gain generated by given padding constant 0 compared to 128 for Q1=Q0+2, which could be found in “Aspect_Ratio_Padding.xls”.

Further Simulation about padding methods are in process. Once it is finished, results will be updated.

## Aspect ratio scalability compared with simulcast

In following simulations, test sequences and coding condition as shown in [3] were used to evaluate the performance of the proposed approach. Simulcast was used as the anchor. The encoding and decoding were conducted on a Linux cluster with Intel Xeon CPU E5-2670, 2,60 GHz. The encoder and decoder binaries were compiled with gcc 4.1.2 64-bit. Both the proposed encoder/decoder were tested in the same cluster. The average performances of the proposed approaches are shown in the following tables. The detailed results can be found in the enclosed excel sheets.

Table below presents the performance of the proposed aspect ratio scalability compared to Simulcast case. The reason for a not matched-BL is the extra information defined in SHM-2.0 for BL which is not encoded in BL. We have compared the recovered and decoded BL, the outcomes of both are identical.

|  | RA HEVC Aspect Ratio |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -5.3% | 3.0% | 1.0% |
| Class B | -7.0% | -2.2% | -2.7% |
| Overall (Test vs Ref) | -6.5% | -0.7% | -1.6% |
| Overall (Test vs single layer) | 25.2% | 33.0% | 31.7% |
| Overall (Ref vs single layer) | 34.2% | 33.8% | 33.8% |
| EL only (Test vs Ref) | -9.4% | -3.6% | -4.5% |
| Enc Time[%] | 108.7% |  |  |
| Dec Time[%] | 119.5% |  |  |
| Enc Mem[%] | 100.0% |  |  |
| BL Match | Not matched |  |  |

Detailed data could be found in xls “Aspect_Ratio_vs_Simulcast”.

## Encoder speed up

Next table shows the performance of encoder/decoder speeding up compared to the proposed aspect ratio scalability. An average of 2.3% time reduction of encoding and 2.6% of decoding is to be expected from simulation results (Aspect_Ratio_RS).

|  | RA HEVC AR_RS vs AR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 25.2% | 33.0% | 31.6% |
| Overall (Ref vs single layer) | 25.2% | 33.0% | 31.7% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98.0% |  |  |
| Dec Time[%] | 99.0% |  |  |
| Enc Mem[%] | 100.0% |  |  |
| BL Match | Matched |  |  |

# References

[1] 	A. Luthra, J. Ohm and J. Ostermann, “Requirements of the scalable enhancement of HEVC”, Stockholm, 2012.

[2] 	J. Dong, Y. He and Y. Ye, “Downsampling filter for anchor generation for scalable extensions of HEVC”, MPEG-M23485, San Jose, 2012.

[3]		X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-M1009, Incheon, 2013.

# Patent rights declaration(s)

Leibniz Universität Hannover may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).