| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0285 |
| --- | --- |

| Title: | Non-RCE3: Intra motion compensation for screen contents |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or | Junghye Min Min Woo Park Sunil Lee Chanyul Kim | Email: | jh643.min@samsung.com m.w.park@samsung.com sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This proposal presents a motion compensation method for intra coding of screen contents. Exhaustive search for finding matched patterns in the decoded area leads to significant increase in the encoding complexity. The proposed method uses only 4 positions in the decoded area for motion compensation candidates. The proposed method provides gains of -0.4%, 0.0%,-10.2%, -7.6% and 0.0% for classF, classB, SC RGB ,SC YUV and RangeExt classes in AI lossless test conditions.

# Introduction

One of characteristics of screen contents is strong correlation of patterns along horizontal and vertical directions. But exhaustive search for finding matched patterns along two directions leads to significant increase in the encoding complexity. The proposed method employs only 4 positions for pattern matching in the decoded area.

# Proposed method

The proposed method uses following 4 candidate positions (Figure.1) in the decoded area along horizontal and vertical directions. A CU level flag is coded to signal whether Intra motion compensation is used or not. If CU level flag is true, the position index (0..3) is also signaled using unary code. For position index 2 and 3 cases, the distance between the left top position of current CU and candidate area is set to CuWidth+CuWidth>>1.

Figure 1. Candidate positions for intra motion compensation

# Experimental results

Proposed methods were implemented on top of HM10.1_RExt3.0 s/w under common test condition described in JCTVC-M1023 [1].  Table 1 presents performance of the proposed method for lossless test conditions. For SC RGB and SC YUV classes,-10.2% and -7.6% gains were obtained for AI conditions. Encoder complexity optimization has not been applied yet.

Table 1: test results for lossless test

Table 2 presents performance of the proposed method for lossy test conditions. For SC RGB and SC YUV classes,-12.2% and -9.5% gains were obtained for AI main-tier condition.

Table 1: test results for lossless test

# Conclusions

This contribution provides an intra motion compensation method for screen contents. Proposed method provides significant performance gains with simplified intra motion compensation design.

# References

[1] Liwei Guo, HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content, JCTVC-M1123, 13th Meeting: Incheon, KR, 18–26 Apr. 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).