| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0288_r3 |
| --- | --- |

| Title: | Non-RCE2: Transform skip on large TUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiulian Peng, Jizheng Xu, Bin Li Microsoft Corp. Liwei Guo, Joel Sole, Marta Karczewicz Qualcomm Incorporated | Tel: Email: | +86(10)59173019 jzxu@microsoft.com |
| Source: | Microsoft Corp. Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Transform skip technique was adopted into HEVC version 1 to enhance the coding efficiency of screen content. In the current design, the transform skip is applied only on 4x4 TUs. This contribution tests transform skip on larger TUs, i.e., 8x8, 16x16 and 32x32 TUs, in the context of HEVC range extension. It is shown that compared with the HM10.1_RExt3.0 anchor, enabling transform skip on all TUs leads to (0.9%, 2.0%, 1.8%) bit-saving for (class F, SC RGB 444, SC YUV 444) sequences on average for the all intra case. The corresponding numbers are (0.8%, 2.0%, 1.3%) and (0.4%, 1.8%, 1.3%) for the random access and low delay cases. Even with transform skip on 4x4 and 8x8 TUs, the bit-saving numbers for (class F, SC RGB 444, SC YUV 444) sequences are (0.8%, 1.5%, 1.3%), (0.7%, 1.6%, 1.1%) and (0.5%, 1.6%, 1.1%). Thus, it is proposed to extend transform skip design to larger TUs.

# Introduction

At the 9th JCT-VC meeting, transform skip coding mode was adopted into HEVC to enhance the coding efficiency of screen content [1]. Although it was shown that transform skip can work with all TUs and provide coding efficiency benefit, only transform skip on 4x4 TUs was considered when it was adopted. One reason is that transform skip on 4x4 TUs provides enough coding efficiency improvement compared with transform skip on all TUs. The other reason is that transform skip on only 4x4 TUs keeps the change minimal, while HEVC version 1 standardisation process was at a stabilization stage.

Later, it was found that the coding gain brought by transform skip can be much larger on 4:4:4 screen contents [2]. Thus, it is interesting to see how much gains can be further achieved by using transform skip on larger TUs. One solution is to enable transform skip on all TUs. Besides coding gain, potential benefits of enabling transform skip on all TUs include that it can remove one conditional check in the text; it maintains a consistent design among all TU sizes; it maintains a consistent design with transquant_bypass mode, which does not have a constraint on block sizes; it also gives encoders more flexibilities if it is not desired to go to small blocks. The other one is to enable transform skip on only 4x4-8x8 TUs, which can provide majority of the coding gain while the encoder complexity is lower. In our implementation, when transform skip is enabled for TUs larger than 4x4, the current default quantization matrix remains the same. The other option is to use a flat matrix as the default quantization matrix for transform skip TUs, as suggested in JCTVC-J0237.

# Experimental results

We enabled transform skip on TUs larger than 4x4 based on the HM10.1_RExt3.0 software and tested the performance improvement following the test conditions used by RCE2 and RCE3 [3].

Table 1-3 show the test results of enabling transform skip on 4x4-8x8, 4x4-16x16 and 4x4-32x32 respectively, compared with the HM10.1_RExt3.0 anchor. Detailed results can be found in the accompanying excel files. It is shown that consistent coding gains can be achieved by applying transform skip on larger TUs. The improvement is especially significant for 4:4:4 sequences.

In the revised document, we re-ran the random_access test for enabling transform skip on 4x4/8x8 TUs to get a comparatively more precise running time measurement.

Table 4-7 show the coding gain of TS on different TU sizes compared with HM10.1_RExt3.0 without TS. It shows the benefit if an encoder does not want to make decision on small blocks about applying TS or not.

Table 1. Experimental results of transform skip on 4x4/8x8 TUs (Details in JCTVC-N0288_TS_4_8.xlsm).

Table 2. Experimental results of transform skip on 4x4/8x8/16x16 TUs (Details in JCTVC-N0288_TS_4_8_16.xlsm).

Table 3. Experimental results of transform skip on 4x4/8x8/16x16/32x32 TUs (Details in JCTVC-N0288_TS_4_8_16_32.xlsm).

Table 4. Experimental results of transform skip on 4x4 TUs (Details in JCTVC-N0288_TS_4vsNoTS.xlsm)

Table 5. Experimental results of transform skip on 8x8/16x16/32x32 TUs (Details in JCTVC-N0288_TS_8_16_32vsNoTS.xlsm)

Table 6. Experimental results of transform skip on 16x16/32x32 TUs (Details in JCTVC-N0288_TS_16_32vsNoTS.xlsm)

Table 7. Experimental results of transform skip on 32x32 TUs (Details in JCTVC-N0288_TS_32vsNoTS.xlsm)

# Proposed modification

Given the coding gains shown in the previous section, we believe that it is worthy to extend transform skip to other TUs. One solution is to enable transform skip on all TUs. The other one is to enable transform skip on only 4x4-8x8 TUs, which can also obtain significant coding gains. The modification to the decoder is simple since enabling transform skip is just to replace the transform with a bit shift operator.

Modifications to the text include:

7.3.8.11       Residual coding syntax

The modification to enable transform skip on all TUs is as follows.

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| …… |  |

The modification to enable transform skip on 4x4-8x8 TUs is as follows.

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize <= 3 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| …… |  |

### 8.6.2 Scaling and transformation process

…

2. The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  (5+ Log2( nTbS )) )	(8-1)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

# References

[1] C. Lan, J. Xu, G.-J. Sullivan and F. Wu, “Intra transform skipping,” Joint Collaborative Team on Video Coding, Doc. No. JCTVC-I0408, Geneva, Switzerland, May 2012.

[2] R. Cohen and A. Vetro, “AHG8: Impact of transform skip on new screen content material,” Joint Collaborative Team on Video Coding, Doc. No. JCTVC-L0428, Geneva, Switzerland, January 2013.

[3] R. Joshi, P. Amon, R. Cohen, S. Lee and M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” Joint Collaborative Team on Video Coding, Doc. No. JCTVC-M1122, Incheon, Korea, April 2013.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. does not have any current or pending patent rights relating to the technology described in this contribution.