| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0303 |
| --- | --- |

| Title: | SCE3: Verification of test 3.2 on adaptive re-sampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In verified test the performance of adaptive re-sampling filter for SHVC was studied. The coefficients of 2D separable 8-tap re-sampling filter for luma component and 4-tap re-sampling filter for Chroma components are signalled in APS at picture level. The filtering process is applied to both integer and fractional pixel positions. Tool is applied for both Luma and Chroma. It is confirmed that compared with SHM2.0 anchor adaptive re-sampling filter provides an average luma BD-rate reduction of 0,3% in spatial cases and 2,0% in SNR scalability cases.

# Introduction

Verified tool was used in Qualcomm’s CfP on scalable HEVC extensions response [1] and then studied in TE3 (JCTVC-12th) [2], SCE4 (JCTVC-13th) [3] and SCE3 (JCTVC-14th) [4]. We provided the cross-check for this tool with detail analysis already twice [5], [6].

Key points of the design are:

- Number of filters used is different for SNR, and spatial scalability is different and depends on scalability ratio; so amount of filter coefficients to code is different according to test scenario;

- In provided code only SNR, spatial scalability ×2, and ×1,5 are supported;

- Values for adaptive re-sampling filter coefficients are restricted to [-128, 127] at encoder side in order to guarantee no overflow of 16-bits intermediate buffer; but encoder restriction is difficult to control, it is possible to insure dynamic range if coefficients are coded as suggested in [7];

- Tool is applied for both Luma and Chroma.

- Integer-pel positions not filtered in anchor are filtered in proposed tool.

- Even higher performance gain was demonstrated in spatial scalability test by switching between 2 Luma re-sampling filters with fixed predetermined coefficients in [8].

- Almost the same gain was demonstrated by switchable de-noising filter applied for Luma only in SNR case [8].

# Test results

S/w was provided according to SCE3 schedule. Results we have obtained perfectly match to proponent’s data. Performance test results are shown in Table 1. Gain from proposed tool is only 0,3% in terms of Luma BD-rate for spatial scalability. Gain is higher for scalability ×1,5 than for scalability ×2. Major gain is observed in SNR test.

Table 1. Performance test results.

Complexity assessment was provided by proponent for both only PU-based and Picture based inter-layer processing.

For PU-base inter-layer processing both in average and in the worst case memory access doesn’t exceed SHM2.0 level, since the same configuration of re-sampling filter is used (only filter coefficients values are different).

For picture based inter-layer processing memory access in the worst case is higher than anchor, since int-pel samples will be filtered with 8-taps (Luma)/4-taps(Chroma) filter, but not copied.

Performance and complexity measurement for verified adaptive inter-layer filter in compare to similar switchable inter-layer filters (with fixed coefficients) are shown in Table 2. Performance benefits from variable coefficients (compare to Picture based switchable filter) is about 0,5% in SNR test, and there is no performance benefits at all in spatial scalability tests.

Memory access numbers are reported relatively to HM10.1 single layer decoder in Table 2. Average numbers include AI, LD-B, RA and LD-P cases.

Table 2. Comparison of switchable and adaptive inter-later filters studied in SCE3.

| Test |  | spatial |  |  |  |  |  | SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD- | rate | Memory Avg |  | The Worst |  | BD- | rate | Memory Avg |  | The Worst |  |
| criteria | Luma | Chroma | PU-based | Pic-based | PU-based | Pic-based | Luma | Chroma | PU-based | Pic-based | PU-based | Pic-based |
| SHM2.0 | 0,00 | 0,00 | 93,2% | 142,3% | 145,0% | 160,0% | 0,00 | 0,00 | 97,8% | 160,2% | 200,0% | 218,0% |
| 3.1Pic | -0,14 | -0,02 | 79,7% | 121,8% | 145,0% | 160,0% | -1,47 | -0,25 | 97,9% | 160,5% | 200,0% | 219,0% |
| 3.1PU | -0,18 | 0,01 | 79,5% | 163,7% | 145,0% | 204,0% | -1,85 | -0,12 | 97,1% | 219,7% | 200,0% | 237,0% |
| 3.1PicLn0 | -0,30 | -0,03 | 79,6% | 121,8% | 145,0% | 160,0% | -1,47 | -0,25 | 97,9% | 160,5% | 200,0% | 219,0% |
| 3.1PULn0 | -0,50 | 0,27 | 79,4% | 163,5% | 145,0% | 204,0% | -1,85 | -0,12 | 97,1% | 219,7% | 200,0% | 237,0% |
| 3.2AUF | -0,27 | -0,49 | 79,7% | 121,8% | 145,0% | 160,0% | -1,98 | -1,40 | 96,5% | 163,4% | 200,0% | 220,0% |

# Conclusions

BD-rate gain of 0,3% for Luma and 0,5% for Chroma from adaptive re-sampling filter coefficients usage is verified in this report . In SNR test gain is higher for Luma and reaches 1,9% in average (but 0,1% for Chroma).

# Reference

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen (Qualcomm), “Description of scalable video coding technology proposal by Qualcomm (configuration 2),” JCTVC-K0035, Shanghai, China, October 2012.

- J. Chen, X. Li, M. Karczewicz (Qualcomm), “TE-B4: Results of test 4.2.1 on adaptive up-sampling filter,” JCTVC-L0092, Geneva, Switzerland, January, 2013.

- W,Pu, J. Chen, JX. Li, M. Karczewicz (Qualcomm), “SCE4: Results of Test 4.2.2 on Adaptive Re-Sampling Filter,” JCTVC-M0267, Incheon, South Korea, April 2013.

- W,Pu, J. Chen, JX. Li, M. Karczewicz (Qualcomm), “SCE3: Results of Test 3.2 on Adaptive Re-Sampling Filter,” JCTVC-N0047, Vienna, Austria, July 2013

- E.Alshina, A.Alshin, “TEB4: Cross-check of adaptive up-sampling filter performance (test 4.2.1),” JCTVC-L0092, Geneva, Switzerland, January, 2013

- E.Alshina, A.Alshin, “Non-SCE4 Cross-check for 6-Tap Adaptive Re-Sampling Filter (JCTVC-M0271),” JCTVC-M0342, Incheon, South Korea, April 2013

- E.Alshina, A.Alshin, “On adaptive re-sampling filter coefficients coding,” JCTVC-N0153, Vienna, Austria, July 2013

- P.Wei, V.Seregin, J. Chen, X. Li, M. Karczewicz, (Qualcomm), E.Alshina, A.Alshin (Samsung), “SCE3: Results of test 3.1 on switchable alternative inter-layer filter,” JCTVC-N0151, Vienna, Austria, July 2013

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).