| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0304 |
| --- | --- |

| Title: | SCE3: Verification of test 3.3 on high frequency pass inter-layer SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In verified test the performance of high frequency pass inter-layer SAO was studied. In verified design high frequency pass SAO is applied w/o multi-pass. It is confirmed that compared with SHM2.0 anchor verified tool provides an average luma BD-rate reduction of 0,5% in spatial cases and 1,6% in SNR scalability cases.

# Introduction

Inter-layer SAO was used in CfP on scalable HEVC extensions responses by LG-MediTek and Samsung-Vydio. Later impressive gain was reported for applying inter-layer SAO to the high frequency part of inter-layer texture prediction by Qualcomm [3], [4]. Eventually Canon in [5] has found the way to essentially correct by adding offset only high frequency part of inter-layer texture prediction w/o multi-pass. This technology is verified in this report.

The key points of the design are:

- Inter-layer texture prediction is corrected in specific points by adding offset;

- Tool is applied for both Luma and Chroma

- Offset is signalled on Picture basis in enhancement layer slice data (so there is a HEVC design change lower than slice header)

- CABAC is use for inter-layer SAO parameters signalling

- Samples classification is different from HEVC SAO:

  - Idea is conceptually the same: edge detection using 1D pattern

  - More than 1 sample surrounding is used (memory access in SAO should be totally redesigned)

  - Criteria for classification contains extra operations compare to HEVC SAO (number of computations was not taken into account in AhG17 complexity assessment this time)

- Main gain of high-frequency pass inter-layer SAO was demonstrated in SNR test. It was shown [6] that the same effect can be achieved by just removing high-frequency component from inter-layer texture prediction in SNR test.

# Test results

S/w was provided according to SCE3 schedule. Later bug-fix was provided, but time was still enough for comprehensive cross-check. Results we have obtained perfectly match with proponent’s data. Performance test results are shown in Table 1. Gain from proposed tool is 0,5% in terms of Luma BD-rate for spatial scalability. Major gain is observed in SNR test.

Table 1. Performance test results.

Complexity assessment in spatial scalability test is adequate and we confirm 7-8% extra memory usage in the worst case and 1-2% extra memory usage in average compare to SHM2.0. But in SNR test complexity assessment seams not 100% adequate. This can be explained that AhG17 discussion about complexity assessment in SNR case was long and recent modification for complexity assessment module in SNR test was made just before the meeting.

For picture based inter-layer processing the complexity of high-frequency pass inter-layer SAO is slightly higher than for 5-taps de-noising filter studied in SCE3 test 3.1. Block extension for inter-layer processing will be the same (2 samples) but verified tool is used for both Luma and Chroma while de-noising tested in SCE3 test 3.1 used for Luma only. But for both tools the worst case memory access increment compare to anchor is only 1-2% in SNR test.

# Conclusions

BD-rate gain of 0,5% for Luma and 0,5% for Chroma from high-frequency pass inter-layer SAO is verified in this report. In SNR test gain is higher for Luma and reaches 1,6% in average for Luma and 1,3% for Chroma.

# Reference

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon (LG Electronics), S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei (MediaTek), “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on),” JCTVC-K0033, Shanghai, China, October 2012

- K. McCann, J. H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y Cho, J. Y. Choi, F. C. A. Fernandes, Z. Ma (Samsung), J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy (Vidyo), “Description of high efficiency scalable video coding technology proposal by Samsung and Vidyo,” JCTVC-K0044, Shanghai, China, October 2012

- W. Pu, J. Chen, K. Rapaka, M. Karczewicz, “High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-L0234, January 2013.

- W. Pu, J. Chen, X. Li, M. Karczewicz, “SCE4: High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-M0265, April 2013

- G. Laroche, P. Onno, J. Taquet, C. Gisquet, E. François (Canon), “Non-SCE4: On Interlayer SAO in SHVC”, Joint Collaborative Team on Video Coding, JCTVC-M0114, April 2013.

- E.Alshina, A.Alshin, “SCE4: De-noising filter for SNR scalability”, Joint Collaborative Team on Video Coding, JCTVC-M0087, April 2013

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).