| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0307 |
| --- | --- |

| Title: | non-SCE3: Cross-check for inter-layer SAO design from MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In verified test the performance of simplified version for high frequency pass inter-layer SAO was studied. It is confirmed that compared with SHM2.0 anchor verified tool provides an average luma BD-rate reduction of 0,4% in spatial cases and 1,5% in SNR scalability cases.

# Introduction

Inter-layer SAO design proposed by MediaTek is simplified version of SCE3 test 3.3 done by Canon. Main changes compare to SCE3 test 3.3 is shown in Figures 1 and 2. Instead high-frequency pass SAO processing for all 4 possible templates (test 3.3, Fig1) in verified design HEVC SAO is used for 2 diagonal templates (Fig.2), but still high-frequency pass SAO processing is used with vertical and horizontal templates.

Another important modification is VLC is used for ilter-layer SAO parameter coding (instead CABAC in original design of SCE3 test 3.3).

Fig. 1.  Sample classification for the four EO classes for IL-SAO with highpass processing

Fig. 2.  Sample classification for the four EO classes for the verfied IL-SAO

# Test results

S/w was provided few days before the meeting start. Performance test results we have obtained perfectly match with proponent’s data. Performance test results are shown in Table 1. Gain from proposed tool is 0,4% in terms of Luma BD-rate for spatial scalability. Major gain is observed in SNR test (1,5%).

Table 1. Performance test results.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1,0% | -1,4% | -1,5% |  |  |  |  |  |  |
| Class B | -0,2% | -0,4% | -0,4% | -0,2% | -0,4% | -0,4% |  |  |  |
| Overall (Test vs Ref) | -0,4% | -0,7% | -0,7% | -0,2% | -0,4% | -0,4% |  |  |  |
| Overall (Test vs single layer) | 12,3% | 14,1% | 13,8% | 10,4% | 9,4% | 8,9% |  |  |  |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |  |  |  |
| EL only (Test vs Ref) | -0,9% | -1,2% | -1,2% | 0,0% | -0,3% | -0,4% |  |  |  |
| Enc Time[%] | 101,1% |  |  | 101,3% |  |  |  |  |  |
| Dec Time[%] | 106,6% |  |  | 110,1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,1% | -0,8% | -1,2% |  |  |  | -2,3% | -1,6% | -2,0% |
| Class B | -0,3% | -0,4% | -0,4% | -0,2% | -0,4% | -0,4% | -0,9% | -0,8% | -0,6% |
| Overall (Test vs Ref) | -0,5% | -0,5% | -0,6% | -0,2% | -0,4% | -0,4% | -1,3% | -1,0% | -1,0% |
| Overall (Test vs single layer) | 18,6% | 32,6% | 31,2% | 15,9% | 28,4% | 28,6% | 12,9% | 30,8% | 32,8% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -0,9% | -0,9% | -1,0% | -0,2% | -0,4% | -0,5% | -2,0% | -1,7% | -1,7% |
| Enc Time[%] | 101,7% |  |  | 102,6% |  |  | 101,6% |  |  |
| Dec Time[%] | 110,7% |  |  | 110,6% |  |  | 113,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,0% | -0,9% | -1,3% |  |  |  | -2,0% | -1,6% | -2,1% |
| Class B | -0,4% | -0,7% | -0,6% | -0,3% | -0,8% | -1,0% | -0,8% | -1,1% | -1,1% |
| Overall (Test vs Ref) | -0,5% | -0,8% | -0,8% | -0,3% | -0,8% | -1,0% | -1,1% | -1,2% | -1,4% |
| Overall (Test vs single layer) | 27,8% | 38,0% | 38,7% | 24,4% | 32,1% | 34,7% | 22,9% | 33,1% | 37,6% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% | 24,3% | 34,7% | 39,5% |
| EL only (Test vs Ref) | -0,9% | -1,1% | -1,2% | -0,2% | -0,7% | -0,9% | -1,6% | -1,8% | -1,9% |
| Enc Time[%] | 101,8% |  |  | 103,0% |  |  | 101,7% |  |  |
| Dec Time[%] | 111,1% |  |  | 112,6% |  |  | 112,7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,1% | -1,2% | -1,6% |  |  |  | -3,3% | -2,3% | -2,5% |
| Class B | -0,5% | -1,0% | -0,9% | -0,7% | -1,4% | -1,2% | -1,7% | -1,6% | -1,5% |
| Overall (Test vs Ref) | -0,7% | -1,1% | -1,1% | -0,7% | -1,4% | -1,2% | -2,1% | -1,8% | -1,8% |
| Overall (Test vs single layer) | 25,7% | 36,6% | 37,7% | 22,0% | 31,1% | 34,1% | 20,7% | 32,3% | 37,0% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% | 23,4% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -1,2% | -1,5% | -1,6% | -0,7% | -1,5% | -1,3% | -3,1% | -2,7% | -2,7% |
| Enc Time[%] | 101,6% |  |  | 103,3% |  |  | 101,9% |  |  |
| Dec Time[%] | 112,4% |  |  | 114,7% |  |  | 115,8% |  |  |
| BL Match | Not matched |  |  | Matched |  |  | Matched |  |  |

Complexity assessment was not performed due to the lack of time.

# Conclusions

BD-rate gain of 0,4% for Luma and 0,7% for Chroma from high-frequency pass inter-layer SAO is verified in this report. In SNR test gain is higher for Luma and reaches 1,5% in average for Luma and 1,4% for Chroma.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).