| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0308 |
| --- | --- |

| Title: | SCE1: Verification of test 2.1 on picture and sequence level adaptive re-sampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In verified test the performance of adaptive re-sampling filter when non-zero phase offset exists was studied. To compensate the phase offset, re-sampling filter coefficients are signalled at sequence level. As supplementary results, performance of filter coefficient update at picture level was also reported and here compared with the same tool test in SCE3. The gain from picture level re-sampling filter coefficients adaptation is 0,3% (Luma)/ 0,5%(Chroma) on content generated with zero-phase shift. If down-sampler phase shift is not zero then average gain shown from picture level coefficients adaptation is 0,4%(Luma)/0,6%(Chroma).

# Introduction

Down-sampling process is non-normative issue. Test sequences used under ctc were generated with so-called zero-phase shift. Alternative test sequences were provided by InterDigital using non-zero phase shift which leads to relative displacement between base layer and enhancement layers pictures. If re-sampling process is performed w/o information about this displacement (assuming zero displacement) then there is about 6% performance drop. The goal of SCE1 was to study the performance impact of signalling displacement information to the decoder and potential benefits of usage non-zero phase shift in down-sampling process.

There are 2 ways to inform decoder about displacement between base and enhancement layer pictures. First is to use predetermined set of filters (16 is SHM2.0 design) and signal which filters to use. It could be done by signalling only one number per sequence. This approach was studied in test 1.2 of SCE1. The alternative is to signal filter coefficients to use. Amount of filters involved into re-sampling process depends on scalability ratio.  Both Luma and Chroma coefficients should be signalled, so side information size is high in this case, but the possibility for adaptation is higher as well. In order to reduce side information the difference between SHM2.0 and selected coefficients is signalled. This second approach was studied in verified test.

Additionally the possibility of picture-based refinement for re-sampling filter coefficients was studied in verified test. Design and implementation is identical to test 3.2 in SCE3, so key points of design have just been copied from our cross-check report for SCE3 [1]:

- Number of filters used is different for SNR, and spatial scalability is different and depends on scalability ratio; so amount of filter coefficients to code is different according to test scenario;

- In provided code only SNR, spatial scalability ×2, and ×1,5 are supported;

- Values for adaptive re-sampling filter coefficients are restricted to [-128, 127] at encoder side in order to guarantee no overflow of 16-bits intermediate buffer; but encoder restriction is difficult to control, it is possible to insure dynamic range if coefficients are coded as suggested in [2];

- Tool is applied for both Luma and Chroma.

- Integer-pel positions not filtered in anchor are filtered in proposed tool.

- Even higher performance gain was demonstrated in spatial scalability test by switching between 2 Luma re-sampling filters with fixed predetermined coefficients in [3].

- Almost the same gain was demonstrated by switchable de-noising filter applied for Luma only in SNR case [3].

Specifically for SCE1 tests of adaptive re-sampling filter SHM2.0 cfg files were modified and default filter coefficients according to known displacement between base and enhancement layer pictures were included into cfg-files. This hint was used for both sequence level and picture level adaptation.

# Test results

S/w was provided according to SCE1 schedule. Results we have obtained perfectly match with proponent’s data. Performance test results for both sequence level and picture level adaptation are summarised in Table 1. The performance of alternative solution with fixed filter coefficients is also shown in Table 1 for comparison. Average gain in AI, LD-B, RA and LD-P tests is shown.

Table 1. Summary of performance test vs SHM2.0 using non-zero down-sampler phase shift (non ctc sequences).

| Test | x2 |  |  | x1,5 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| 1.2-Fixed-coeff (1/16) | -6,7 | -5,2 | -5,1 | -6,3 | -5,9 | -5,5 |
| 2.1-AUF-Seq | -6,6 | -5,2 | -5,1 | -6,3 | -5,8 | -5,5 |
| 2.1-AUF-Pic | -6,8 | -5,6 | -5,5 | -6,9 | -6,7 | -6,2 |

Since the same coefficients were used in test 1.2 and verified here test 2.1 in case of sequence level adaptation the performance shown of these two tests is almost identical.

If picture level refinement is enabled then better adaptation is possible and 0,2...0,6% extra BD-rate gain for Luma and 0,7…1,2% extra BD-rate gain for Chroma can be achieved. These results are pretty similar to gain demonstrated by picture level coefficients adaptation vs fixed coefficients in SCE3 for ctc sequences.

Table 2. Summary of performance test vs HEVC single layer.

| Down-sampler | Re-sampling | x2 |  |  | x1,5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| 0-phase shift | SHM2.0 | 21,8 | 31,3 | 31,3 | 18,6 | 26,1 | 27,5 |
| Non-0 phase shift | 1.2-Fixed-coeff (1/16) | 22,2 | 31,5 | 31,7 | 19,4 | 26,8 | 28,1 |
|  | 2.1-AUF-Seq | 22,3 | 31,6 | 31,8 | 19,5 | 26,8 | 28,1 |
|  | 2.1-AUF-Pic | 22,0 | 31,0 | 31,3 | 18,7 | 25,8 | 27,2 |

Table 2 summaries the BD-rate performance of scalable system compare to HM10.1 single layer codec. These data help to understand how down sampler modification affects to scalable system performance. First line in Table2 shows that scalable system shows 18,6...21,8% drop compare to HM10.1 single layer codec if zero-phase shift is used during down-sampling.

The usage of non-zero phase shift in down-sampler doesn’t help performance improvement of scalable system. Even with picture level coefficients adaptation it is impossible to get performance better than SHM2.0 shows for down-sampled with zero-phase shift sequences.

So, the series of tests had done in SCE1 show no evidence for usage of non-zero phase shift during down-sampling process.

# Conclusions

BD-rate gain of 6,4% for Luma and 5,4% for Chroma from sequence level filter coefficients adaptation is verified for specific content generated with displacement between base layer and enhancement layer pictures. The same gain can be achieved with fixed coefficients but phase-shift signalling. No performance improvement of scalable codec compare to single layer HEVC is demonstrated from usage of non-zero phase shift during down-sampling.

# Reference

- E.Alshina, A.Alshin, “SCE3: Verification of test 3.2 on adaptive re-sampling filter,” JCTVC-N0303, Vienna, Austria, July 2013

- E.Alshina, A.Alshin, “On adaptive re-sampling filter coefficients coding,” JCTVC-N0153, Vienna, Austria, July 2013

- P.Wei, V.Seregin, J. Chen, X. Li, M. Karczewicz, (Qualcomm), E.Alshina, A.Alshin (Samsung), “SCE3: Results of test 3.1 on switchable alternative inter-layer filter,” JCTVC-N0151, Vienna, Austria, July 2013

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Test results details

Table 3. Sequence level AUF performance with non—ctct sequences.

Table 4.  Picture  level AUF performance with non—ctct sequences.