| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0309 |
| --- | --- |

| Title: | Signalling of chroma sampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku, Kawasaki, 211-8588 Japan Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | FUJITSU LABORATORIES LTD., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

JCTVC-M0146 provided a framework to send recommended up-sampling and down-sampling filter coefficients for chroma format conversion by using VUI and SEI message. This follow-up contribution proposes the modified signalling scheme of chroma sampling filter taking the discussion on JCTVC-M0146 from the last meeting into consideration.

Apology

The authors sincerely apologize for the delay with which this contribution was submitted and would greatly appreciate its consideration.

# Introduction

During the discussion on JCTVC-M0146 at the last meeting, the following comments were expressed from the floor [2].

- It was questioned why a specific entry in the VUI table was proposed for a particular filter that is not specified in any standard instead of just using a user defined entry.

- An alternative would be to use only an SEI message instead of a hybrid solution between a VUI and an SEI message.

- The previous contribution did not contain information about the normalization stage.

In order to answer those remarks, the following signalling scheme and semantics are proposed.

# Proposed syntax and semantics

## Variation 1: Signalling in VUI

### Sequence parameter set RBSP syntax

| sps_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_extension_flag ) { |  |
| if (vui_parameters_present_flag ) |  |
| vui_extension_parameters_present_flag | u(1) |
| if ( vui_extension_parameters_present_flag ) |  |
| vui_extension_parameters( ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Sequence parameter set RBSP semantics

sps_extension_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. sps_extension_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for sps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all sps_extension_data_flag syntax elements that follow the value 1 for sps_extension_flag in an SPS NAL unit.

vui_extension_parameters_present_flag equal to 1 specifies that the vui_extension_parameters( ) syntax structure as specified in Annex E is present. vui_parameters_present_flag equal to 0 specifies that the vui_extension_parameters( ) syntax structure as specified in Annex E is not present.

### VUI extension parameters syntax

| vui_extension_parameters( ) { | Descriptor |
| --- | --- |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_filter_info_present_flag | u(1) |
| if( chroma_filter_info_present_flag) { |  |
| ver_chroma_filter_idc | u(8) |
| hor_chroma_filter_idc | u(8) |
| ver_filtering_process_flag | u(1) |
| if( ver_chroma_filter_idc == 1 ¦¦ hor_chroma_filter_idc == 1 ) { |  |
| target_format_idc | ue(v) |
| perfect_reconstruction_flag | u(1) |
| if( ver_chroma_filter_idc == 1 ) { |  |
| num_vertical_filters | ue(v) |
| NumVerFilters = num_vertical_filters |  |
| if( NumVerFilters > 0 ) { |  |
| for( i=0; i < NumVerFilters; i++) { |  |
| ver_tap_length_minus1[ i ] | ue(v) |
| for( j=0; j <= ver_tap_length_minus1[ i ]; j++) { |  |
| ver_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( hor_chroma_filter_idc == 1 ) { |  |
| num_horizontal_filters | ue(v) |
| NumHorFilters = num_horizontal_filters |  |
| if( NumHorFilters > 0 ) { |  |
| for( i=0; i < NumHorFilters; i++) { |  |
| hor_tap_length_minus1[ i ] | ue(v) |
| for( j=0; j <= hor_tap_length_minus1[ i ]; j++) { |  |
| hor_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### VUI extension parameters semantics

chroma_filter_info_present_flag equal to 1 specifies that chroma sampling filter information is present. If chroma_filter_info_present_flag is equal to 0, chroma sampling filter information is not present in the bitstream.

ver_chroma_filter_idc specified the preferred chroma sampling filter in vertical direction for the output decoded pictures and for input pictures at an encoder which takes the output decoded pictures as an input.

When ver_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in vertical direction are signalled in VUI. When ver_chroma_filter_idc is not equal to 0, the coefficients of chroma sampling filter are specified in Table 1.

hor_chroma_filter_idc specified the preferable chroma sampling filter in horizontal direction for the output decoded pictures and possibly for input pictures at an encoder which takes the output decoded pictures as an input.

When hor_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in horizontal direction are signalled in VUI. When hor_chroma_filter_idc is not equal to 0, the coefficients of chroma sampling filter are specified in Table 2.

Table 1 speficies chroma sampling filter in vertical direction.

Table 1	Chroma filter index in vertical direction

| Value | Vertical chroma sampling filter | Informative Remark |
| --- | --- | --- |
| 0 | Unspecified | Chroma filter is unknown or is determined by the application. |
| 1 | User-defined | Filter coefficients are specified in VUI |
| 2 | SMPTE RP 2050-1:2012 | SMPTE EG2050-2:2012 provides details on a possible implementation of the filter defined in SMPTE RP2050-1:2012. Chroma sample type shall be 0. |
| 3 … 255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

Table 2 speficies chroma sampling filter in horizontal direction.

Table 2	Chroma filter index in horizontal direction

| Value | Horizontal chroma sampling filter | Informative Remark |
| --- | --- | --- |
| 0 | Unspecified | Chroma filter is unknown or is determined by the application. |
| 1 | User-defined | Filter coefficients are specified inVUI |
| 2 | ITU-T Rec. T.800 ¦ ISO/IEC15444-1, 5/3 filter | Chroma sample type should be 0, 2 or 4. |
| 3 … 255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

ver_filtering_process_flag specified how chroma sampling filter in vertical direction is performed. If ver_filtering_process_flag is equal to 1, chroma sampling filter is applied in field-basis (i.e. only chroma samples of the same parity is used for filtering). Otherwise, chroma sampling filter is applied in frame-basis.

target_format_idc specifies the output of chroma sampling relative to that of the luma sampling as specified in subclause 6.2. The value of target_format_idc shall be in the range of 1 to 3, inclusive.

perfect_reconstruction_flag equal to 1 specifies that the chroma sampling filter coefficients satisfy perfect reconstruction condition which mean that up-sampled results are identical when down-sampling and up-sampling are repeated. perfect_reconstruction_flag equal to 0 indicates that the chroma sampling filter coefficients may not satisfy perfect reconstruction condition.

num_vertical_filters specifies the number of filters used for chroma sampling in vertical direction.

ver_tap_length_minus1[] + 1 specifies the number of tap length of a filter in vertical direction. The value of ver_tap_length_minus1 shall be in the range of 0 to 31, inclusive.

ver_filter_coeff[][] specifies the value of a filter coefficient in vertical direction. The value of ver_filter_coeff[][] shall be in the range of −231 + 1 to 231 − 1, inclusive.

num_horizontal_filters specifies the number of filters used for chroma sampling in horizontal direction.

hor_tap_length_minus1[] + 1 specifies the number of tap length of a filter in horizontal direction. The value of hor_tap_length_minus1 shall be in the range of 0 to 31, inclusive.

hor_filter_coeff[][] specifies the value of a filter coefficient in horizontal direction. The value of hor_filter_coeff[][] shall be in the range of −231 + 1 to 231 − 1, inclusive.

Ideal filtering process when ver_chroma_filter_idc is equal to 1 or hol_chroma_filter_idc is equal to 1 is described as follows:

p(x,y): input image (before scaling) q(x,y): output image (after scaling) p'(x,y)=p(min(max(0,x),width−1),min(max(0,y),height−1)) Vertical sampling q(x,y')=1N(z)p'(x,y+j)*c(z,j)+(N(l)>>1) z=y' % num_vertical_filters N(z)=c(z,j) s(z)=−(ver_tap_length_minus1[z]+1)/2) e(z)=(ver_tap_length_minus1[z]+1)−(ver_tap_length_minus1[z]+1)/2 c(z,j)=ver_filter_coeff[z][j] y'=2ydown_samplingy=2y'upsampling Horizontal sampling q(x',y)=1N(z)p'(x+i,y)*c(z,i)+(N(z)>>1) z=x' % num_horizontal_filters N(z)=c(z,i) s(z)=−(hor_tap_length_minus1[z]+1)/2) e(z)=(hor_tap_length_minus1[z]+1)−(hor_tap_length_minus1[z]+1)/2 c(z,i)=hor_filter_coeff[z][i] x'=2xdown_samplingx=2x'upsampling

## Variation 2: Signalling in SEI

### Chroma sampling filter hint SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else if( payloadType = = XXX) |  |
| chroma_sampling_filter_hint( payloadSize) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

chroma_filter_info_present_flag equal to 1 specifies that chroma sampling filter information is present. If chroma_filter_info_present_flag is equal to 0, chroma sampling filter information is not present in the bitstream.

ver_chroma_filter_idc specified the preferred chroma sampling filter in vertical direction for the output decoded pictures and for input pictures at an encoder which takes the output decoded pictures as an input.

When ver_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in vertical direction are signalled in VUI. When ver_chroma_filter_idc is not equal to 0, the coefficients of chroma sampling filter are specified in Table 1.

hor_chroma_filter_idc specified the preferable chroma sampling filter in horizontal direction for the output decoded pictures and possibly for input pictures at an encoder which takes the output decoded pictures as an input.

When hor_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in horizontal direction are signalled in VUI. When hor_chroma_filter_idc is not equal to 0, the coefficients of chroma sampling filter are specified in Table 2.

| chroma_sampling_filter_hind( payloadSize) { | Descriptor |
| --- | --- |
| if( chroma_loc_info_present_flag ) { |  |
| ver_chroma_filter_idc | u(8) |
| hor_chroma_filter_idc | u(8) |
| ver_filtering_process_flag | u(1) |
| if( ver_chroma_filter_idc == 1 ¦¦ hor_chroma_filter_idc == 1 ) { |  |
| target_format_idc | ue(v) |
| perfect_reconstruction_flag | u(1) |
| if( ver_chroma_filter_idc == 1 ) { |  |
| num_vertical_filters | ue(v) |
| NumVerFilters = num_vertical_filters |  |
| if( NumVerFilters > 0 ) { |  |
| for( i=0; i < NumVerFilters; i++) { |  |
| ver_tap_length_minus1[ i ] | ue(v) |
| for( j=0; j <= ver_tap_length_minus1[ i ]; j++) { |  |
| ver_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( hor_chroma_filter_idc == 1 ) { |  |
| num_horizontal_filters | ue(v) |
| NumHorFilters = num_horizontal_filters |  |
| if( NumHorFilters > 0 ) { |  |
| for( i=0; i < NumHorFilters; i++) { |  |
| hor_tap_length_minus1[ i ] | ue(v) |
| for( j=0; j <= hor_tap_length_minus1[ i ]; j++) { |  |
| hor_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Chroma sampling filter hint SEI message semantics

Table D-1 – Persistence scope of prefix SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| buffering period | - The remainder of the bitstream |
| … | - … |
| chroma sampling filter hint | - The remainder of the bitstream |

The chroma sampling filter hint SEI message provides the recommended coefficients of a chroma sampling filter. If the coefficients of chroma sampling filter both at an encoder and a decoder are designed based on the coefficients signalled in the chroma sampling filter hint SEI message, the color difference degradation will be expected to be minimized.

ver_chroma_filter_idc specifies the preferred chroma sampling filter in vertical direction for the output decoded pictures and for input pictures at an encoder which takes the output decoded pictures as an input.

When ver_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in vertical direction are signalled in the Chroma sampling filter hint SEI. When ver_chroma_filter_idc is not equal to 0, the coefficients of chroma sampling filter are specified in Table 1.

hor_chroma_filter_idc specifies the preferable chroma sampling filter in horizontal direction for the output decoded pictures and possibly for input pictures at an encoder which takes the output decoded pictures as an input.

When hor_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in horizontal direction are signalled in the Chroma sampling filter hint SEI. When hor_chroma_filter_idc is not equal to 0, the coefficients of chroma sampling filter are specified in Table 2.

(The rest is the same as in 2.1.4)

# References

- T.Chujoh at el., “VUI extension and SEI for chroma sampling filter”, Document of Joint Collaborative Team on Video Coding, JCTVC-M0146, April 2013.

- “Meeting report of 13th JCT-VC Meeting”, Document of Joint Collaborative Team on Video Coding, JCTVC-M1000, April 2013.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).