| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0311 |
| --- | --- |

| Title: | Cross check of JCTVC-N0229 on Region based Inter-layer Cross-Color Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard François Canon Research France | Tel: Email: | Edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This document provides the cross-check results of the proposal JCTVC-N0229 on Region based Inter-layer Cross-Color Filtering. The software implementation has been verified and all the BD-rate results are reported to match exactly those provided by the proponent.

# Introduction

JCTVC-N0229 proposes a region based inter-layer cross-color filtering in SHVC. It consists in splitting the enhancement layer picture into 1, 4 or 16 rectangular regions. Then for each region, one set of cross-color filter parameters is signaled. The filter is used to enhance up-sampled chroma components from the high-frequency information extracted from corresponding base luma component (not upsampled one, in order to avoid delay).

A non-separable 8-point cross-shaped filter is used. It is computed at the encoder side by solving the Wiener-Hopf equation.

# Cross-checking description

## Implementation verification

The software patch and results to be cross-checked have been delivered by Qualcomm on July 17, 2013.

The following macro setting is used to enable the proposed modification:

#define INTERCOMP_FILTER	1

The software implementation has been verified and is confirmed to be in line with the tool description.

The filtering of one chroma sample is done as follows:

- The 2D convolution is applied first to the input base layer luma signal

  - tmp1 = Sum( coef[i,j] * luma[r.xbl+i,ybl+j])	i=-1,2 j=-1,1

- this value is scaled by a coded scaling factor:

  - tmp2 = tmp1 * Scale

- the offset applied to the upsampled chroma signal is then derived as

  - nOffset = ( abs( tmp2 ) + 8192 ) >> 14

- Finally the chroma sample is updated as follows :

  - chr[x,y] = Clip(0, 255, chr[x,y]+offset)

It should be noted that the proponent’s contribution does include the syntax changes but the new filtering process description is not yet provided.

## Simulations

Tests were done using the AhG8 common conditions for lossless coding. The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

Only the test with QTC=2 has been verified.

The full cross-check results are in the attached xls files. Results provided in JCTVC-N0229 are confirmed.

## Complexity analysis

The following observations are made:

- The filter is 2D non-separable filter

- Coefficients are coded on 4 bits and can take values from -8 to 7. The scaling factor coefficient is coded using 10 bits, plus one bit to indicate the sign of this scaling factor.

- Number of operations per chroma samples for the chroma upsampling

  - 8 multiplications (4 horiz + 4 vertic)

  - 6 additions (3 horiz + 3 vertic)

  - one right shift

- Number of operations per chroma samples for the cross-color filtering

  - 9 multiplications   (+113% compared to simple chroma upsampling)

  - 8 additions       (+133% compared to simple chroma upsampling)

  - one right shift

  - one clipping between 0 and 255

- In its current implementation, an internal register width of 26 bits is required to derive tmp2 in case of 8 bit-depth signal (bit depth 8 + coefs 4 + nb filter taps 3 + signed scaling factor 11).

The complexity modules, integrated into the software, have been checked. To the best knowledge of the cross-checker, no major issue has been detected in this implementation. Resulting average complexity numbers, gathered only for of test 2 (QTC=2), generated from decoding the streams with these complexity modules are confirmed by the cross-checker.

Regarding the worst case complexity analysis, after some discussions with the proponent, a common understanding of the worst memory access needs has been reached. In spatial scalability case, the worst case is not modified since the luma BL memory access for the cross-color filtering is not larger than the one for the luma upsampling. For the SNR case, a filter size of 3 (cell C25 of ‘WorstCaseMemory’ excel sheet of JCTVC-N0229) is set for the luma BL memory access.

# Conclusions

The implementation, BDR results and complexity evaluation of JCTVC-N0311 presented by Qualcomm are confirmed by the cross-checker.