| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0312_r2 |
| --- | --- |

| Title: | A proposal on HEVC 4:2:2 profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shun-ichi Sekiguchi Akira Minezawa Hiroharu Sakate 5-1-1 Ofuna, Kamakura, Kanagawa, JAPAN Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku, Kawasaki, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp +81 44 754 2639 kazui.kimihiko@jp.fujitsu.com |
| Source: | Mitsubishi Electric Corporation, FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes a new HEVC profile supporting 4:2:2 chroma format. The proposed profile is simple extension to Main 10 profile. It is proposed to include the proposed profile in DAM of range extensions.

# Introduction

In JCTVC-M0094[2], Main 10 4:2:2 profile was proposed as a new RExt profile for supporting YUV4:2:2 video. This profile extends the specification related to chroma sampling format from Main 10 profile without any additional coding tools. In this contribution, we basically support the idea of the Main 10 4:2:2 profile proposed in M0094 with some minor modifications.

# Proposed profile

The proposed profile is based on Main 10 profile and only extends to support 4:2:2 chroma sampling format. The proposed description is shown as follows.

- A.3.5 Main 10 4:2:2 profile

Bitstreams conforming to the Main 10 4:2:2 profile shall obey the following constraints:

– SPSs shall have chroma_format_idc equal to 1 to 2 inclusive.

– SPSs shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

– SPSs shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

– CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

– When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

– When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and

RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to

num_tile_rows_minus1, inclusive.

– The number of times read_bits( 1 ) is called in clauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

– The level constraints specified for the Main 10 4:2:2 profile in clause A.4 shall be fulfilled.

Conformance of a bitstream to the Main 10 4:2:2 profile is indicated by general_profile_idc being equal to 4 or general_profile_compatibility_flag[ 4 ] being equal to 1.

Decoders conforming to the Main 10 4:2:2 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which all of the following conditions apply:

– general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] is equal to 1 or general_profile_compatibility_flag[ 4 ] is equal to 1.

– general_level_idc represents a level lower than or equal to the specified level.

– general_tier_flag represents a tier lower than or equal to the specified tier.

# Level definition

The level definition of the proposed profile is also based on Main 10 profile, but we propose to modify the maximum bitrate and CPB size as follows.

A.4.1	General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 is considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this annex:

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e. the 0-th access unit).

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable CpbBrVclFactor be equal to 1000.

- Let the variable CpbBrNalFactor be equal to 1100.

For Main and Main 10 profiles, let the variable CpbBrVclFactor to be equal to 1000 and let the variable CpbBrNalFactor be equal to 1100.

For Main 10 4:2:2, the value of CpbBrVclFactor to be equal to 1600 and let the variable CpbBrNalFactor be equal to 1920.

Bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

A.4.2  Profile-specific level limits for the Main, Main 10 and Main 10 4:2:2 profiles

# References

- D. Flynn, J. Sole and T. Suzuki “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 3,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M1005, 13th Meeting: Incheon, KR, April, 2013.

- T. Suzuki, K. Sharman and N. Saunders “Proposal on the profile definition of Range extensions,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M0094, 13th Meeting: Incheon, KR, April, 2013.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).