| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0313 |
| --- | --- |

| Title: | 4EVER HEVC demonstrations during Roland Garros tournament |
| --- | --- |
| Status: | Input Document to JCT-VC, Video and Systems |
| Purpose: | Communication |
| Author(s) or Contact(s): | S. Kervadec (Orange Labs), M. Raulet (INSA), J. Le Feuvre (Telecom ParisTech), J. Vieron (ATEME), M. Clare (Orange Labs) on behalf of the 4EVER project |
| Source: | 4EVER Project |

_____________________________

# Abstract

A world first end-to-end live HD HEVC-based delivery chain was demonstrated by the 4EVER project during the French Open Tennis Tournament Roland-Garros.

Many MPEG/ITU technologies were showcased during these demonstrations. Demonstrations were shown during the second week of the tournament, from June 1st, to June 9th.

Three HEVC-based HD End-to-End Live delivery chains were demonstrated: IPTV chain, based on HEVC/MPEG-4 AAC/MPEG-2 TS, delivered using UDP multicast transport over Orange IPTV managed network; Internet chain, based on HEVC/MPEG-4 AAC/ISOBMFF/MPEG DASH, delivered using HTTP over best-effort Internet network, through two different (nominal/backup) HTTP servers; DVB-T2 chain, based on HEVC/MPEG-4 AAC/MPEG-2 TS, delivered over DVB-T2 modulation, using a micro-diffusion in Roland-Garros, and a real diffusion in Rennes.

These demonstrations were operated in the same conditions as any commercial audiovisual service, the IPTV and Internet Live feeds being broadcasted at the same time they were produced on the courts, and being accessible through any Orange access (although requiring a dedicated HEVC decoder).

# 4EVER Project

“4EVERError: Reference source not found, for Enhanced Video ExpeRience, project aims at researching, developing and promoting a significant video Quality of Experience and putting premium audiovisual content access within everyone’s reach, for all populations and in any situation (homes, mobiles, cinemas) thanks to:

- HEVC (new MPEG/ITU standard) codec development;

- Ultra High Definition new format definition and qualification.

Whether for High Definition on mobile and ADSL/terrestrial networks, or Ultra High Definition over fiber or ADSL/terrestrial networks, the 4EVERError: Reference source not found consortium considers the entire production and delivery chain, and aims visible quality gains for all viewers.”

This collaborative project gathers 9 industrial and academic partners (Orange, Ateme, France Télévisions, Technicolor, Teamcast, HTS, Globecast, IETR, Telecom ParisTech ) and is supported by French government and local territories.

The 4EVERError: Reference source not found project has previously investigated support of interlaced sources in HEVC, which led to MPEG contributions m28119 (Geneva) and m29210 (Incheon). The main conclusion was that the 50% gain promised by HEVC can also be achievable on interlaced sources.

# Roland-Garros End-to-End Live HEVC-based delivery chain

Demonstrations were shown during the second week of the tournament, from June 1st, to June 9th.

Three HEVC-based HD End-to-End Live delivery chains were demonstrated:

- IPTV chain, based on HEVC/MPEG-4 AAC/MPEG-2 TS, delivered using UDP multicast transport over Orange IPTV managed network.

- Internet chain, based on HEVC/MPEG-4 AAC/ISOBMFF/MPEG DASH, delivered using HTTP over best-effort Internet network, through two different (nominal/backup) HTTP servers.

- DVB-T2 chain, based on HEVC/MPEG-4 AAC/MPEG-2 TS, delivered over DVB-T2 modulation, using a micro-diffusion in Roland-Garros, and a real diffusion in Rennes.

These demonstrations were operated in the same conditions as any commercial audiovisual service, the IPTV and Internet Live feeds being broadcasted at the same time they were produced on the courts, and being accessible through any Orange access (although requiring a dedicated HEVC decoder).

Two main locations were used for the demonstrations:

In Roland-Garros

- In France Televisions Show Room (RG Labs, public zone): DVB-T2 delivery chain, using a micro-diffusion

- In Orange TV Control Room (in the technical “TV” zone): IPTV and Internet delivery chains, using a regular xDSL or FTTx Orange access.

In Rennes (West of France)

- In “La Cantine Numérique rennaise”: DVB-T2 delivery chain, using a DVB-T2 modulation from TDF Telecommunications tower, the demonstration location being in the town center, at about 5 kms from the modulator, and Internet delivery chain.

720p50 format was chosen as a compromise to address tablets while maintaining temporal fluidity. So the production format delivered by France Televisions, 1080i25, first had to be de-interlaced.

Then, two real-time HEVC HD encoders (provided by Ateme) were set up to enable different configurations for IPTV and OTT streams.

Three different HEVC software decoders and players were used, based on three different implementations.

The MPEG DASH/MP4 conversion was performed in real-time, by a dedicated application running on an http server, based on the MPEG-2 TS multicast.

Tablets were playing the MPEG DASH ISOBMFF Live feed, while DVB-T2 and IPTV streams were displayed on large screens, using desktop computers running the open-source software decoder [OpenHEVC] embedded in an open-source media player [GPAC]. These open-source tools allow reproducing most of these results from already encoded content, especially live MPEG-DASH and live MPEG-2 TS transmission chains.

# MPEG technologies

Several MPEG technologies were used in these demonstrations, based on existing and widely deployed standards, such as MPEG-2 TS, ISOBMFF/AVC FF/MPEG-4 FF and MPEG-4 AAC, and on more recent technologies such as HEVC and MPEG DASH.

Each standard was used in its most up-to-date revision:

- DAM for HEVC encapsulation in AVC FF and MPEG-2 TS,

- HEVC IS, using HM 10.1 as the reference for interoperability.

Although the HEVC standard has been finalized only a few months ago, early implementations are already quite mature: the interoperability between the encoder and the different decoders was quite  good from the start, and the encoder operated 24/7 during nine days without any issue.

Wavefront Parallel Processing was enabled, to allow for decoder parallelism.

We faced a few issues regarding MPEG DASH interoperability, especially on the time anchor when using dynamic @type for the MPD. Static @type was used instead, the MPD being re-generated with a new @start attribute every 10 segments.

MPEG-2 TS transport through Orange IPTV network worked without any issue or any modification to the existing delivery chain. As expected, using the new HEVC codec in MPEG-2 transport streams did not have any impact on the various DTV hardware in the network path.

# References

- [HEVC] ISO/IEC 23008-2 - Information technology -- High efficiency coding and media delivery in heterogeneous environments -- Part 2: High efficiency video coding

- [AAC] ISO/IEC 14496-3:2009 - Information technology -- Coding of audio-visual objects -- Part 3: Audio

- [ISOBMFF] ISO/IEC 14496-12:2012 - Information technology -- Coding of audio-visual objects -- Part 12: ISO base media file format

- [MPEG2TS] ISO/IEC 13818-1:2013- Information technology -- Generic coding of moving pictures and associated audio information: Systems

- [HEVCFF] ISO/IEC FDIS 14496-15:2010 3rd edition Carriage of NAL unit structured video in the ISO BMFF

- [HEVCTS] ISO/IEC 13818-1:201X/DAM 3 Carriage of HEVC

- [m28119] Information on forthcoming 4EverError: Reference source not found project subjective HEVC evaluation - Jean-Charles Gicquel, Jerome Fournier, Jean-Marc Thiesse on Behalf of the 4EverError: Reference source not found Project,

- [m29210] Subjective comparison of HEVC and AVC for HDTV content - Zineb Agyo, Christophe Daguet, Jerome Fournier, Jean-Charles Gicquel, Felix Henry, Jean-Marc Thiesse, Jerome Vieron, on behalf of the 4EverError: Reference source not found Project

- [OpenHEVC] Open-source HEVC decoder, https://github.com/OpenHEVC/openHEVC

- [GPAC] Open-source Media Framework, http://gpac.wp.mines-telecom.fr/