| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0322 |
| --- | --- |

| Title: | AHG 8 Cross-check for JCTVC-N0231: Intra mode coding for screen contents |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for Document JCTVC-N0231 proposed by Samsung and related to the AHG 8 activities. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

Document JCTVC-N0231 proposes to change the most probable mode setting for intra prediction to speed-up the processing, namely for screen content coding. In fact, the proponent observers that given the sharp edges present in screen content videos, the derivation process for the most probable mode currently specified in the HEVC-RExt spec can be simplified as follows:

candModeList[ 0 ] = INTRA_HOR

candModeList[ 1 ] = INTRA_VER

candModeList[ 2 ] = INTRA_ANGULAR_18

The most probable modes are set without checking for any spatial dependency as currently mandated by the HEVC-RExt spec. In the next section the experimental results are presented.

# Test results

The proposal is tested both for lossy and lossless coding using the RCE2 test conditions described in Document JCTVC-M1122. This cross-check document reports the same results in the following tables. Notice that for lossy coding the BD-rates for RGB content presented in this document are ordered differently from the ones released by the proponent. In particular the cross checker generated the anchors with the encoder parameter --SNRInternalColourSpace set to zero while the proponent set it to one. The overall effect of this parameter is a different ordering of the PSNR values according to the used input colour space conversion.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.3% | 31.7 | 31.8 | -0.2% | 49.8 | 49.8 | -0.2% |
| Class B | 2.2 | 2.2 | -0.1% | 2.6 | 2.6 | 0.1% | 2.6 | 2.6 | 0.1% |
| SC RGB 444 | 10.1 | 10.1 | -0.7% | 100.4 | 101.2 | -0.4% | 381.6 | 384.7 | -0.5% |
| SC YUV 444 | 11.4 | 11.5 | -0.7% | 128.9 | 130.0 | -0.6% | 325.6 | 328.0 | -0.4% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 102% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 106% |  |  | 104% |  |  |

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.4% | 0.1% | 0.1% | 0.4% | 0.3% | 0.3% | 0.4% | 0.3% | 0.4% |
| Class B | 1.8% | 0.6% | 0.6% | 1.2% | 0.7% | 0.8% | 0.7% | 0.6% | 0.7% |
| SC RGB 444 | -0.8% | -1.0% | -0.8% | -0.7% | -0.9% | -0.8% | -0.7% | -0.9% | -0.7% |
| SC YUV 444 | -0.2% | -0.3% | -0.3% | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% | -0.1% |
| RangeExt | 1.4% | 0.8% | 0.8% | 0.8% | 0.5% | 0.6% | 0.5% | 0.4% | 0.4% |
| Enc Time[%] | 106% |  |  | 106% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class B | 0.8% | 0.3% | 0.0% | 0.6% | 0.4% | 0.3% |  |  |  |
| SC RGB 444 | -0.8% | -1.0% | -0.8% | -0.7% | -0.9% | -0.7% |  |  |  |
| SC YUV 444 | -0.3% | -0.4% | -0.3% | -0.2% | -0.2% | -0.2% |  |  |  |
| RangeExt | 0.7% | 0.4% | 0.2% | 0.4% | 0.2% | 0.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.4% | -0.5% | -0.1% | -0.1% | -0.2% |  |  |  |
| Class B | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -0.5% | -0.7% | -0.4% | -0.7% | -1.1% | -0.8% |  |  |  |
| SC YUV 444 | -0.5% | -0.2% | -0.5% | -0.5% | -0.4% | -0.4% |  |  |  |
| RangeExt | 0.4% | 0.2% | 0.1% | 0.2% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |

# Source code inspection

The proponent implemented the proposed changes in the HM-10.0-rext-3.0 reference software. In particular, the following macro has been defined:

INTRA_MODE_SET

The macro is then used in the member function “getIntraDirPredictor” for the class “TComDataCU” to set the most probable modes in the array “uiIntraDirPred”. With the aforementioned macro enabled, the processing is stopped after the array values have been set. This behaviour reflects what is proposed in Document JCTVC-N0231.