| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0323 |
| --- | --- |

| Title: | Non-RCE3: Cross-check of JCTVC-N0235 base color merging for MBCIM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-847-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution verified the method description and the BD-rate results of JCTVC-N0235 which is proposed by Sony.

# Introduction

JCTVC-N0235 describes a base color merging method for MBCIM. With the proposed method, the palette of the current CU can be copied from the palette of the top CU or the left CU.

# Experimental results

The JCTVC-N0235 software provided by the proponents has been used to generate the results and compare with results generated with HM-10.1+RExt-3.0 under RCE3 common test conditions [1]. Encoding time and decoding time were obtained using heterogeneous cluster. It was confirmed that for all test cases, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Lossless results

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.6% | 31.7 | 31.7 | -0.1% | 49.8 | 49.8 | -0.1% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 19.1 | -31.1% | 100.4 | 194.9 | -26.6% | 381.6 | 776.8 | -24.5% |
| SC YUV 444 | 11.4 | 15.2 | -13.1% | 128.9 | 171.6 | -12.4% | 325.6 | 467.1 | -12.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 111% |  |  | 104% |  |  | 104% |  |  |
| Dec Time[%] | 107% |  |  | 98% |  |  | 103% |  |  |

Table 2 Lossy results

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -16.1% | -15.0% | -16.5% | -13.4% | -13.6% | -14.3% | -10.7% | -11.6% | -12.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -41.8% | -42.0% | -42.5% | -43.8% | -43.7% | -44.3% | -44.9% | -44.8% | -45.3% |
| SC YUV 444 | -26.5% | -25.2% | -25.7% | -25.9% | -26.8% | -26.8% | -27.0% | -28.3% | -28.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 177% |  |  | 172% |  |  | 169% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 105% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -11.6% | -12.2% | -12.2% | -9.2% | -10.6% | -10.2% |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -35.9% | -36.0% | -36.4% | -37.7% | -37.7% | -38.2% |  |  |  |
| SC YUV 444 | -21.3% | -22.8% | -22.8% | -22.0% | -23.8% | -23.5% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 111% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 116% |  |  | 117% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -8.0% | -8.4% | -7.9% | -6.6% | -7.2% | -6.8% |  |  |  |
| Class B | 0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -30.2% | -30.1% | -30.6% | -33.3% | -33.2% | -33.7% |  |  |  |
| SC YUV 444 | -16.2% | -16.9% | -17.5% | -16.9% | -17.7% | -18.1% |  |  |  |
| RangeExt | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 109% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 120% |  |  | 119% |  |  |  |  |  |

# References

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, KR, April 2013.

# Patent rights declaration(s)