| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0325 |
| --- | --- |

| Title: | Non-RCE2: Cross-verification of JCTVC-N0232 Rice parameter update method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-847-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution verified the method description and the BD-rate results of JCTVC-N0232 which is proposed by Samsung.

# Introduction

JCTVC-N0232 describes two rice parameter update methods. Rice parameter is calculated depending the absCoeff (absolute value of coefficient level) and previous rice parameter as follows.

The first method:

uiGoRiceParam = min<UInt>(uiGoRiceParam+ ((absCoeff[idx]+3)>>(uiGoRiceParam+3)), RICE_ MAX);

The second method:

uiGoRiceParam = min<UInt>(uiGoRiceParam+ ((absCoeff[idx]+4)>>(uiGoRiceParam+3)), RICE_ MAX);

RICE_MAX can be set to 4, 5, and 6.

In this contribution, we verified the software matches the description in the contribution, and also confirmed the results for the case of RICE_MAX = 6.

# Experimental results

The JCTVC-N0232 software provided by the proponents has been used to generate the results and compare with results generated with HM-10.1+RExt-3.0 under RCE2 common test conditions [1]. Encoding time and decoding time were obtained using heterogeneous cluster. It was confirmed that for all test cases, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Lossless results (Method 1)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.4% | 31.7 | 32.1 | -0.8% | 49.8 | 50.1 | -0.5% |
| Class B | 2.2 | 2.3 | -0.7% | 2.6 | 2.6 | -1.1% | 2.6 | 2.6 | -1.1% |
| SC RGB 444 | 10.1 | 11.3 | -9.1% | 100.4 | 113.3 | -7.7% | 381.6 | 438.8 | -7.3% |
| SC YUV 444 | 11.4 | 12.0 | -4.0% | 128.9 | 136.7 | -3.6% | 325.6 | 354.9 | -4.0% |
| RangeExt | 2.4 | 2.4 | -0.5% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 103% |  |  |

Table 2 Lossy results (Method 1)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% |
| Class B | 0.2% | 0.3% | 0.3% | 0.4% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% |
| SC RGB 444 | 0.4% | 0.4% | 0.5% | -0.6% | -0.5% | -0.5% | -2.3% | -2.7% | -2.6% |
| SC YUV 444 | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | -0.2% | -0.4% | -0.3% |
| RangeExt | 0.3% | 0.3% | 0.2% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% | 0.5% |
| Enc Time[%] | 102% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% |  |  |  |
| Class B | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% | 0.1% |  |  |  |
| SC RGB 444 | 0.4% | 0.4% | 0.5% | -0.5% | -0.4% | -0.4% |  |  |  |
| SC YUV 444 | 0.3% | 0.3% | 0.4% | 0.4% | 0.3% | 0.4% |  |  |  |
| RangeExt | 0.1% | 0.0% | 0.1% | 0.2% | 0.2% | 0.2% |  |  |  |
| Enc Time[%] | 104% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.1% | -0.2% | 0.0% | 0.0% | -0.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.1% | -0.2% | -0.2% | -1.2% | -1.1% | -1.2% |  |  |  |
| SC YUV 444 | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 106% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |

Table 3 Lossy results (applied to transform skip block only) (Method 1)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.1% | 0.2% | 0.2% | -1.0% | -0.9% | -0.8% | -2.8% | -3.1% | -3.0% |
| SC YUV 444 | 0.1% | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% | -0.6% | -0.6% | -0.6% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  | 97% |  |  | 97% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.1% | 0.1% | 0.1% | -0.8% | -0.7% | -0.8% |  |  |  |
| SC YUV 444 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 93% |  |  | 93% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.4% | 0.0% | -0.2% | -0.3% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.7% | 0.8% | 0.7% | -0.3% | -0.2% | -0.3% |  |  |  |
| SC YUV 444 | 0.5% | 0.5% | 0.4% | 0.3% | 0.3% | 0.3% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 93% |  |  | 93% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |

Table 4 Lossless results (Method 2)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.4% | 31.7 | 32.1 | -0.7% | 49.8 | 50.1 | -0.3% |
| Class B | 2.2 | 2.3 | -0.3% | 2.6 | 2.6 | -0.2% | 2.6 | 2.6 | -0.2% |
| SC RGB 444 | 10.1 | 11.3 | -9.2% | 100.4 | 113.3 | -7.7% | 381.6 | 439.1 | -7.3% |
| SC YUV 444 | 11.4 | 12.1 | -4.1% | 128.9 | 136.9 | -3.7% | 325.6 | 356.4 | -4.0% |
| RangeExt | 2.4 | 2.4 | -0.9% | 2.5 | 2.5 | -0.4% | 2.5 | 2.5 | -0.4% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  | 100% |  |  |

Table 5 Lossy results (Method 2)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.3% | -0.3% |
| Class B | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% |
| SC RGB 444 | 0.2% | 0.2% | 0.2% | -0.9% | -0.8% | -0.8% | -2.7% | -3.0% | -3.0% |
| SC YUV 444 | 0.1% | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% | -0.6% | -0.6% | -0.7% |
| RangeExt | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% | 0.4% | 0.3% |
| Enc Time[%] | 97% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | 0.0% | 0.0% | 0.1% | -0.8% | -0.8% | -0.8% |  |  |  |
| SC YUV 444 | 0.3% | 0.2% | 0.4% | 0.2% | 0.2% | 0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 108% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.0% | -0.2% | -0.1% | -0.1% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.3% | -0.3% | -0.3% | -1.5% | -1.4% | -1.5% |  |  |  |
| SC YUV 444 | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |  |  |  |

Table 6 Lossy results (applied to transform skip block only) (Method 2)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | 0.0% | -0.1% | -0.2% | -0.2% | -0.2% | -0.5% | -0.5% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.1% | 0.1% | -1.1% | -1.1% | -1.1% | -3.0% | -3.3% | -3.3% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.1% | -0.8% | -0.8% | -0.8% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.1% | 0.0% | 0.0% | -1.1% | -1.0% | -1.0% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.1% | 0.9% | -0.1% | 0.0% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.1% | -0.1% | -0.2% | -1.2% | -1.1% | -1.2% |  |  |  |
| SC YUV 444 | 0.2% | 0.3% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

# Conclusion

This contribution reports the cross-verification results for JCTVC-N0232. It was confirmed that the implementation was consistent with the algorithm description and the test results matched those provided by the proponent.

# References

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, KR, April 2013.

# Patent rights declaration(s)