| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July–2 Aug. 2013 | Document: JCTVC-N0329 |
| --- | --- |

| Title: | RCE2: Cross-check of Test A1(JCTVC-M0304 ) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jing Ye Zhan Ma Meng Xu Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 USA | Email: Tel: | jing.ye@huawei.com +1 408 330 4952 |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution reports the results of a cross-check performed by Huawei on part of JCTVC-N0268- Rice parameter signaling for transform-skip blocks. There are two method proposed in JCTVC-N0268. The verification tests were carried out in Table 1 and Table 3 using the test sequences and conditions specified in JCTVC-M1122. The additional experiment of Method II is shown in Table 5. Our test results have a perfect match, in terms of bitrates, with those presented in the contribution documents.

# Description

The basic idea of JCTVC-N0268 is to propose Rice parameter signaling for transform-skip blocks. In Method I, cRiceParameter is signalled from the set {0, 1, 2, 3, 4}. In Method II, cRiceParam is signalled from the subset {0, 2, 4}. Truncated unary (TU) binarization method is used to code cRiceParameter for both test case. In the additional test, slice level flag is considered to determine whether or not the proposed rice-parameter signaling method is activated.

# Results

The test conditions for RCE2 as specified in JCTVC-M1122 were used in the verification tests. The simulations were run on a 64-bit Windows cluster. Our results have a perfect match with those reported by the proponents. The complete cross-check results are provided in the accompanying spread-sheets. Below shows the summary of the test results.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.5% | 31.7 | 32.0 | -0.7% | 49.8 | 50.0 | -0.5% |
| Class B | 2.2 | 2.3 | -0.8% | 2.6 | 2.6 | -1.4% | 2.6 | 2.6 | -1.3% |
| SC RGB 444 | 10.1 | 10.8 | -6.3% | 100.4 | 108.1 | -5.2% | 381.6 | 413.8 | -4.9% |
| SC YUV 444 | 11.4 | 12.2 | -5.0% | 128.9 | 138.6 | -4.4% | 325.6 | 357.9 | -4.6% |
| RangeExt | 2.4 | 2.4 | -2.9% | 2.5 | 2.6 | -2.5% | 2.5 | 2.6 | -2.5% |
| Enc Time[%] | 113% |  |  | 105% |  |  | 100% |  |  |
| Dec Time[%] | 111% |  |  | 99% |  |  | 97% |  |  |

Table 1: Simplified levelcoding Method I with respect to lossless coding anchor

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.6% | -0.5% | -0.5% | -1.1% | -0.9% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.4% | -1.4% | -1.4% | -3.7% | -3.6% | -3.6% | -5.9% | -6.1% | -6.1% |
| SC YUV 444 | 0.4% | 0.4% | 0.5% | -0.3% | -0.2% | -0.1% | -1.6% | -1.7% | -1.8% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 117% |  |  | 128% |  |  | 130% |  |  |

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.2% | 0.1% | -0.3% | -0.1% | -0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| SC RGB 444 | -1.2% | -1.1% | -1.1% | -3.1% | -3.0% | -3.0% |
| SC YUV 444 | 0.5% | 0.5% | 0.6% | 0.0% | 0.1% | 0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 115% |  |  | 121% |  |  |

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.1% | 0.6% | 0.5% | -0.1% | 0.3% | 0.3% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| SC RGB 444 | -0.8% | -0.8% | -0.8% | -3.1% | -3.1% | -3.1% |
| SC YUV 444 | 0.3% | 0.5% | 0.4% | 0.0% | 0.1% | 0.0% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 119% |  |  | 132% |  |  |

Table 2: Simplified levelcoding Method I with respect to lossy coding anchor

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.8% | 31.7 | 32.2 | -0.9% | 49.8 | 50.2 | -0.6% |
| Class B | 2.2 | 2.3 | -0.6% | 2.6 | 2.6 | -1.3% | 2.6 | 2.6 | -1.3% |
| SC RGB 444 | 10.1 | 11.1 | -8.3% | 100.4 | 110.9 | -6.9% | 381.6 | 425.5 | -6.5% |
| SC YUV 444 | 11.4 | 12.5 | -6.3% | 128.9 | 142.0 | -5.7% | 325.6 | 368.6 | -5.9% |
| RangeExt | 2.4 | 2.4 | -2.9% | 2.5 | 2.6 | -2.1% | 2.5 | 2.6 | -2.1% |
| Enc Time[%] | 104% |  |  | 100% |  |  | 97% |  |  |
| Dec Time[%] | 111% |  |  | 96% |  |  | 93% |  |  |

Table 3: Simplified levelcoding Method II with respect to lossless coding anchor

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.2% | -0.9% | -0.8% | -0.7% | -1.4% | -1.3% | -1.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.1% | -2.1% | -2.1% | -4.9% | -4.8% | -4.8% | -7.4% | -7.6% | -7.6% |
| SC YUV 444 | 0.2% | 0.1% | 0.2% | -0.7% | -0.6% | -0.5% | -2.3% | -2.3% | -2.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  | 104% |  |  | 104% |  |  |
| Dec Time[%] | 121% |  |  | 131% |  |  | 134% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.8% | -1.8% | -1.8% | -4.0% | -3.9% | -3.9% |  |  |  |
| SC YUV 444 | 0.4% | 0.3% | 0.4% | -0.2% | -0.2% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 119% |  |  | 124% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.7% | 0.1% | -0.2% | 0.3% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -1.5% | -1.4% | -1.4% | -4.1% | -4.0% | -4.1% |  |  |  |
| SC YUV 444 | 0.5% | 0.7% | 0.4% | -0.1% | 0.0% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 105% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 122% |  |  | 136% |  |  |  |  |  |

Table 4: Method II with respect to lossy coding anchor

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.3% | -0.4% | -0.9% | -0.7% | -0.8% | -1.4% | -1.2% | -1.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.1% | -2.1% | -2.1% | -4.7% | -4.7% | -4.6% | -7.2% | -7.5% | -7.5% |
| SC YUV 444 | -0.1% | -0.1% | -0.1% | -0.8% | -0.7% | -0.6% | -2.2% | -2.3% | -2.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 118% |  |  | 129% |  |  | 131% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.1% | -0.1% | -0.5% | -0.4% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -1.7% | -1.7% | -1.7% | -3.8% | -3.8% | -3.8% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.1% | -0.4% | -0.4% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 114% |  |  | 120% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.3% | -0.3% | -0.1% | 0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -1.9% | -1.9% | -1.8% | -4.2% | -4.1% | -4.2% |  |  |  |
| SC YUV 444 | 0.1% | 0.2% | 0.1% | -0.4% | -0.3% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 104% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 116% |  |  | 129% |  |  |  |  |  |

Table 5: Slice level flag enabled in Method II with respect to lossy coding anchor

# Conclusions

Our verification test results confirmed the reported coding efficiency gain shown in JCTVC-N0268.