| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0344 |
| --- | --- |

| Title: | Derivation of picture and slice level information for resampled interlayer reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin Xiang Li Krishna Rapaka Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides the specification text of derivation process for picture and slice level information of resampled interlayer reference picture.

# Introduction and problem statement

In current SHVC, the following steps are used to generate an interlayer reference picture for spatial scalability case.

- Resampling of picture sample values: the reference layer picture is upsampled according to the spatial ratio to match the picture size of the current picture.

- Motion field mapping: for each 16x16 block of the inter-layer picture, the co-located block is found in the reference layer picture and then motion vector (scaled), prediction mode, reference indices, inter direction are copied to the inter-layer picture.

The resampled motion field is utilized in TMVP derivation when the interlayer reference picture is used as the collocated picture for TMVP derivation. The TMVP derivation process also needs high level information of the collocated picture, such as picture order count, slice type and reference picture list. Therefore to enable the TMVP derived from resampled interlayer reference picture; such information of resampled interlayer reference picture shall also be appropriately derived.

The picture level information of the resampled picture, such as picture order count, can be easily copied from the corresponding reference layer picture. Slice type and reference picture list information is carried in slice level rather picture level. In case of multiple slices existing in the reference layer picture, the slice partitioning needs to be mapped from the reference layer picture to the resampled picture and then the slice level information is derived for each slice in the resampled picture. As shown in figure 1, the up-scaled slice partition may cross the enhancement layer CTB and the slice partitioning in the enhancement layer has to be adjusted. The slice mapping process with such adjustment could introduce a complication and this adjustment could also lead to inaccuracy in the motion mapping.

This contribution proposes an alternative solution for slice level information derivation. When there is only one slice in the corresponding reference layer picture or when slice type and reference picture list of all slices in the corresponding reference layer picture are identical, there is no need to have slice mapping process. Instead, the whole resampled interlayer picture can be associated to a single slice and the slice level information including slice_type and reference picture list can be simply set equal to those of any slice in the corresponding reference picture. To avoid abovementioned slice mapping process, the TVMP derived from resampled reference layer picture needs to be disabled when any two slices in corresponding reference layer picture have different slice_type or reference picture list information.

Figure 1 Slice mapping process in case of 1.5x ratio spatial scalability

# Proposal

In his contribution, the picture and slice level information of the resampled interlayer reference picture is derived as follow:

- The picture order count of resampled interlayer picture is set equal to the value of picture order count of the corresponding reference picture.

- The whole resampled interlayer picture is associated to one slice and the slice level information including slice_type and reference picture list set equal to those of the first slice in the corresponding reference picture.

- Impose a restriction on the collocated_ref_idx syntax element used to identify a collocated picture, that when the resampled interlayer reference picture is used as collocated picture for TMVP derivation, all slices in the corresponding reference layer picture shall have identical slice type, identical reference picture list 0 and identical reference picture list 1, otherwise such an inter-layer picture cannot be used as a collocated picture for TMVP derivation

Effectively, this bitstream conformance constraint is proposed to avoid the abovementioned slice mapping process, and it does not have impact on coding efficiency with current common test conditions since only one slice is used

# Specification Statement

Modifications are highlighted in yellow.

H.8.1.4	Resampling process for inter layer reference pictures

Input to this process is:

- a decoded reference layer picture rlPic

- a direct reference layer index dRlIdx .

Output of this process is	the resampled reference layer picture rsPic.

The variables PicWidthInSamplesL and PicHeightInSamplesL are set equal to pic_width_in_luma_samples and pic_height_in_luma_samples, respectively. The variable rsPicSample is defined as a group of sample arrays specifying the resampled sample values of rsPic of the luma and chroma components. The variable rsPicMotion is defined as a group of variable arrays specifying the resampled motion field of rsPic.

The variables RefLayerPicWidthInSamplesL and RefLayerPicHeightInSamplesL are set equal to the width and height of the decoded reference layer picture rlPic in units of luma samples, respectively. The variable rlPicSample is defined as a group of sample arrays specifying the sample values of rlPic of the luma and chroma components. The variable rlPicMotion is defined as a group of variable arrays specifying the compressed motion field of rlPic.

The variables PicWidthInSamplesC, PicHeightInSamplesC, RefLayerPicWidthInSamplesC, and RefLayerPicHeightInSamplesC are derived as follows:

PicWidthInSamplesC  = PicWidthInSamplesL / subWidthC				(H-1)
PicHeightInSamplesC = PicHeightInSamplesL / subHeightC				(H-2)
RefLayerPicWidthInSamplesC  = RefLayerPicWidthInSamplesL / subWidthC	(H-3)
RefLayerPicHeightInSamplesC = RefLayerPicHeightInSamplesL / subHeightC	(H-4)

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftOffset = scaled_ref_layer_left_offset[ dRlIdx ] << 1	(H-5)
ScaledRefLayerTopOffset = scaled_ref_layer_top_offset[ dRlIdx]  << 1	(H-6)
ScaledRefLayerRightOffset = scaled_ref_layer_right_offset[ dRlIdx ]  << 1	(H-7)
ScaledRefLayerBottomOffset = scaled_ref_layer_bottom_offset[ dRlIdx ]  << 1	(H-8)

The variables ScaledRefLayerPicWidthInSamplesL and ScaledRefLayerPicHeightInSamplesL are derived as follows:

ScaledRefLayerPicWidthInSamplesL = PicWidthInSamplesL – 
			ScaledRefLayerLeftOffset  – ScaledRefLayerRightOffset	(H-9)
ScaledRefLayerPicHeightInSamplesL = PicHeightInSamplesL – 
			ScaledRefLayerTopOffset – ScaledRefLayerBottomOffset	(H-10)

The variables ScaleFactorX and ScaleFactorY are derived as follows:

ScaleFactorX = ( ( RefLayerPicWidthInSamplesL << 16 ) + ( ScaledRefLayerPicWidthInSamplesL >> 1 ) ) / ScaledRefLayerPicWidthInSamplesL		(H-11)
ScaleFactorY = ( ( RefLayerPicHeightInSamplesL << 16 ) + ( ScaledRefLayerPicHeightInSamplesL >> 1 ) ) / ScaledRefLayerPicHeightInSamplesL		(H-12)

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if PicWidthInSamplesL is equal to RefLayerPicWidthInSamplesL and PicHeightInSamplesL is equal to RefLayerPicHeightInSamplesL and the values of ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are all equal to 0

  - rsPicSample is set equal to rlPicSample,

  - When alt_collocated_indication_flag is equal to 1, rsPicMotion is set equal to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture order count of rsPic is set equal to the value of picture order count of rlPic.

  - The variable rsSlice is defined as a slice header associated to the rsPic and all coding tree blocks in rsPic are associated to rsSlice. The slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 of rsSlice are set equal to the value of slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 of the first slice of rlPic, respectively. When rsSlice is a P or B slice, for i in the range of 0 to num_ref_idx_l0_active_minus1 of rsSlice, inclusive, reference picture with index i in reference picture list 0 of rsSlice is set equal to reference picture with index i in reference picture list 0 of the first slice of rlPic. When rsSlice is a B slice, for i in the range of 0 to num_ref_idx_l1_active_minus1 of rsSlice, inclusive, reference picture with index i in reference picture list 1 of rsSlice is set equal to reference picture with index i in reference picture list 1 of the first slice of rlPic.

  - The variable rsPicSample is defined as a group of sample arrays specifying the resampled sample values of rsPic of the luma and chroma components. The variable rlPicSample is defined as a group of sample arrays specifying the sample values of rlPic of the luma and chroma components. The picture sample resampling process as specified in subclause Error: Reference source not found is invoked with the sample values of rlPicSample as input, and with the resampled sample values of  rsPicSample as output.

  - The variable rsPicMotion is defined as a group of variable arrays specifying the resampled motion field of rsPic. The variable rlPicMotion is defined as a group of variable arrays specifying the compressed motion field of rlPic. When alt_collocated_indication_flag is equal to 1, the picture motion field resampling process as specified in subclause Error: Reference source not found is invoked with rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

H.8.1.4.2	Resampling process of  picture motion field

Input to this process is rlPicMotion specifying the motion field of the picture rlPic.

Output of this process is rsPicMotion specifying the resampled motion field of the resampled picture.

The motion field of rlPic specified by rlPicMotion consists of:

- a ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) array predModeRL specifies the prediction modes of the reference layer picture rlPic,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays refIdxLXRL specify the reference indices of the reference layer picture rlPic, with X = 0,1,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays mvLXRL specify the luma motion vectors of the reference layer picture rlPic, with X = 0,1,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays refPicOrderCntLXRL specify the reference picture order counts of the reference layer picture rlPic, with X = 0, 1,

- two ( RefLayerPicWidthInSamplesL ) x ( RefLayerPicHeightInSamplesL ) arrays predFlagLXRL specify the prediction list utilization flags of the reference layer picture rlPic, with X = 0,1.

The resampled motion field specified by rsPicMotion consists of:

–	a ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) array predMode specifies the prediction modes of the resampled picture,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays refIdxLX specify the reference indexes of the resampled picture, with X = 0,1,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays mvLX  specify the luma motion vectors of the resampled picture, with X = 0,1,

–	two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays refPicOrderCntLX specify the reference picture order counts of the resampled picture, with X = 0, 1

- two ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) arrays predFlagLX specify the prediction list utilization flags of the resampled picture, with X = 0,1.

For each luma sample location xPb = 0 ... ( ( PicWidthInSamplesL + 15 ) >> 4 ) − 1 and yPb = 0 … ( ( PicHeightInSamplesL + 15 ) >> 4) − 1,

–	The variables xP and yP are set to ( xPb  << 4 ) and ( yPb  << 4 ), respectively,

–	The variables predMode[xP][yP], refIdxLX[xP][yP], mvLX[xP][yP] and refPicOrderCntLX[xP][yP], and predFlagLX[xP][yP], with X = 0,1, of the resampled picture are derived by invoking inter layer motion derivation process specified in subclause Error: Reference source not found with the luma location ( xP, yP ), predModeRL, refIdxLXRL, mvLXRL,   refPicOrderCntLXRL, and predFlagLXRL, with X = 0,1, given as input.

H.8.1.4.2.1	Derivation process for inter layer motion

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- the reference layer prediction modearray predModeRL,

- the reference layer reference index arrays refIdxL0RL and refIdxL1RL

- the reference layer motion vector arrays mvL0RL and mvL1RL

- the reference layer reference picture order counts arrays refPicOrderCntL0RL and refPicOrderCntL1RL

- the reference layer prediction list utilization flag arrays predFlagL0RL and predFlagL1RL.

Outputs of this process are

- a derived prediction mode predMode,

- two derived motion vectors mvL0 and mvL1

- two derived reference indices refIdxL0 and refIdxL1

- two derived reference picture order counts refPicOrderCntL0 and refPicOrderCntL1

- two derived prediction list utilization flags predFlagL0 and predFlagL1.

The variables predMode, mvLX, refIdxLX, refPicOrderCntLX, and predFlagLX are derived as follows.

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xP + 8		(H-13)

yPCtr = yP + 8		(H-14)

- The derivation process for reference layer luma sample location specified in subclause H.6.1Error: Reference source not found is invoked with luma location ( xPCtr , yPCtr ) given as the inputs and ( xRef , yRef ) as output.

- The collocated position (xRL, yRL) is derived as follows

xRL = ( xRef >> 4 ) << 4		(H-15)

yRL = ( yRef >> 4 ) << 4		(H-16)

- The reference layer motion vector is derived as follows

- If ( xRL < 0 ) or ( xRL >= RefLayerPicWidthInSamplesL ) or ( yRL < 0 ) or ( yRL >= RefLayerPicHeightInSamplesL ), predMode[ xP ][ yP ] is set to MODE_INTRA.

- Otherwise, predMode[ xP ][ yP ] is derived as follows

predMode[ xP ][ yP ] = predModeRL[ xRL ][ yRL ]	(H-17)

- If predMode[ xP ][ yP ] is equal to MODE_INTER, for each X = 0, 1, the following applies

refIdxLX[ xP ][ yP ] = refIdxLXRL[ xRL ][ yRL ]	(H-18)

refPicOrderCntLX[ xP ][ yP ] = refPicOrderCntLXRL[ xRL ][ yRL ]	(H-19)

predFlagLX[ xP ][ yP ] = predFlagLXRL[ xRL ][ yRL ]	(H-20)

        - If ScaledRefLayerPicWidthInSamplesL is not equal to RefLayerPicWidthInSamplesL, mvLX[ xP ][ yP ][ 0 ] is derived as follows:

scaleFactorMVX = Clip3( −4096, 4095, ( ( ScaledRefLayerPicWidthInSamplesL << 8 ) + ( RefLayerPicWidthInSamplesL >> 1 ) ) / RefLayerPicWidthInSamplesL)	(H-21)

mvLX[ xP ][ yP ][0] = Clip3( −32768, 32767, Sign(scaleFactorMVX * 
mvLXRL[ xRL ][ yRL ][ 0 ] ) * 	( ( Abs ( scaleFactorMVX * mvLXRL[ xRL ][ yRL ][ 0 ] )
 + 127 ) >> 8 ) ) 		(H-22)

        - Otherwise,

mvLX[ xP ][ yP ][ 0 ] = mvLXRL[ xRL ][ yRL ][ 0 ] 	(H-23)

        - If ScaledRefLayerPicHeightInSamplesL is not equal to RefLayerPicHeightInSamplesL, mvLX[ xP ][ yP ][ 1 ] is derived as follows:

scaleFactorMVY = Clip3( −4096, 4095, ( ( ScaledRefLayerPicHeightInSamplesL << 8 ) + ( RefLayerPicHeightInSamplesL >> 1 ) ) / RefLayerPicHeightInSamplesL)	(H-24)

mvLX[ xP ][ yP ][ 1 ] = Clip3( −32768, 32767, Sign(scaleFactorMVY * 
mvLXRL[ xRL ][ yRL ][ 1 ] ) * 	( ( Abs  ( scaleFactorMVY * mvLXRL[ xRL ][ yRL ][ 1 ] )
 + 127 ) >> 8 ) )		(H-25)

        - Otherwise,

mvLX[ xP ][ yP ][ 1 ] = mvLXRL[ xRL ][ yRL ][ 1 ] 	(H-26)

- Otherwise, if predMode[ xP ][ yP ] is equal to MODE_INTRA

        - both components of mvL0[ xP ][ yP ] and mvL1[ xP ][ yP ] are set to 0, refIdxL0[ xP ][ yP ] and refIdxL1[ xP ][ yP ] are set to –1, refPicOrderCntL0[ xP ][ yP ] and refPicOrderCntL1[ xP ][ yP ] are set to –1, predFlagL0[ xP ][ yP ] and predFlagL1[ xP ][ yP ] are set to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).