| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0335 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0288: Non-RCE2: Transform skip on large TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for Document JCTVC-N0288 proposed by Microsoft and Qualcomm Inc. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

Document JCTVC-N0288 presents a study whereby transform skip is tested for block sizes larger than 4×4. Transform skip has already proved to be an effective tool to improve the compression efficiency of those residuals which are already sparse in the spatial domain. The tests performed by the proponent are relative to these three sets of block sizes:

- Set 1: 4×4 and 8×8

- Set 2: 4×4, 8×8 and 16×16

- Set 3: 4×4, 8×8, 16×16 and 32×32

The RD performance for all the three sets of block sizes are tested under the RCE2 coding conditions described in Document JCTVC-M1122. In the next section the cross-checked experimental results are presented.

# Test results

The proposal is tested both for lossy coding using the RCE2 test conditions described in Document JCTVC-M1122. This cross-check document reports the same results in the following tables. Notice that the BD-rates for RGB content presented in this document are ordered differently from the ones released by the proponent. In particular the cross checker generated the anchors with the encoder parameter --SNRInternalColourSpace set to zero while the proponent set it to one. The overall effect of this parameter is a different ordering of the PSNR values according to the used input colour space conversion.

Table 1: BD-rates for block sizes belonging to Set 1 (4×4 and 8×8)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.8% | -0.7% | -0.7% | -0.7% | -0.6% | -0.6% | -0.6% | -0.5% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.8% | -1.5% | -1.8% | -1.8% | -1.4% | -1.8% | -1.6% | -1.3% | -1.7% |
| SC YUV 444 | -1.3% | -1.4% | -1.6% | -1.3% | -1.4% | -1.6% | -1.2% | -1.4% | -1.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 116% |  |  | 117% |  |  | 118% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.7% | -0.5% | -0.6% | -0.5% | -0.4% | -0.5% |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.7% | -1.6% | -1.7% | -1.7% | -1.6% | -1.7% |  |  |  |
| SC YUV 444 | -1.1% | -1.1% | -1.3% | -1.0% | -1.1% | -1.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 106% |  |  | 107% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | 0.1% | -0.3% | -0.4% | 0.1% | -0.2% |  |  |  |
| Class B | -0.1% | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.7% | -1.6% | -1.6% | -1.6% | -1.6% | -1.5% |  |  |  |
| SC YUV 444 | -1.1% | -1.0% | -1.1% | -1.0% | -0.9% | -0.9% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 106% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |

Table 2: BD-rates for block sizes belonging to Set 2 (4×4, 8×8 and 16×16)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.9% | -0.8% | -0.8% | -0.8% | -0.7% | -0.7% | -0.6% | -0.6% | -0.5% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.2% | -1.8% | -2.2% | -2.1% | -1.7% | -2.1% | -1.9% | -1.6% | -2.0% |
| SC YUV 444 | -1.6% | -1.7% | -2.0% | -1.5% | -1.7% | -1.9% | -1.4% | -1.7% | -1.7% |
| RangeExt | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 123% |  |  | 124% |  |  | 125% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -0.8% | -0.8% | -0.7% | -0.7% | -0.7% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -2.2% | -2.0% | -2.2% | -2.2% | -2.0% | -2.2% |  |  |  |
| SC YUV 444 | -1.5% | -1.5% | -1.6% | -1.3% | -1.4% | -1.5% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 111% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.6% | -0.3% | -0.2% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% |  |  |  |
| SC RGB 444 | -1.8% | -1.7% | -1.7% | -1.9% | -1.9% | -1.8% |  |  |  |
| SC YUV 444 | -1.4% | -1.1% | -1.3% | -1.3% | -1.2% | -1.2% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.3% | 0.0% | 0.1% | 0.5% |  |  |  |
| Enc Time[%] | 110% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |

Table 3: BD-rates for block sizes belonging to Set 3 (4×4, 8×8, 16×16 and 32×32)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.9% | -0.8% | -0.8% | -0.8% | -0.7% | -0.7% | -0.6% | -0.6% | -0.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.4% | -2.0% | -2.3% | -2.3% | -1.9% | -2.3% | -2.2% | -1.8% | -2.2% |
| SC YUV 444 | -1.8% | -1.9% | -2.2% | -1.6% | -1.8% | -2.0% | -1.5% | -1.8% | -1.8% |
| RangeExt | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 130% |  |  | 132% |  |  | 132% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 97% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -0.7% | -0.7% | -0.7% | -0.6% | -0.5% |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -2.2% | -2.0% | -2.2% | -2.2% | -2.0% | -2.3% |  |  |  |
| SC YUV 444 | -1.3% | -1.4% | -1.5% | -1.2% | -1.3% | -1.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 114% |  |  | 116% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.1% | -0.4% | -0.4% | -0.1% | -0.3% |  |  |  |
| Class B | 0.0% | 0.3% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.8% | -1.8% | -1.8% | -2.2% | -2.2% | -2.1% |  |  |  |
| SC YUV 444 | -1.3% | -1.2% | -1.2% | -1.2% | -1.1% | -1.1% |  |  |  |
| RangeExt | -0.1% | 0.1% | 0.2% | -0.1% | 0.1% | 0.5% |  |  |  |
| Enc Time[%] | 112% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Source code inspection

The proponent implemented the proposed changes in the HM-10.0-rext-3.0 reference software. In particular, the following macro has been defined:

TS_LARGE_SIZE

The macro is used to change the value of the following two macros in TComRom.h:

MAX_TS_WIDTH

MAX_TS_HEIGHT

In particular the values for these two macros are set as follows:

- Set 1: MAX_TS_WIDTH = 8, MAX_TS_HEIGHT = 16

- Set 2: MAX_TS_WIDTH = 16, MAX_TS_HEIGHT = 32

- Set 3: MAX_TS_WIDTH = 32, MAX_TS_HEIGHT = 64

Moreover, after the setting of the aforementioned macros, also the function “xRecurIntraCodingQT” is modified to allow the transform skip check for all sizes regardless the use of the transform skip fast encoder parameter. It is reported that the source code changes made by the proponent reflect the algorithmic description presented in JCTVC-N0288.