| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July–2 Aug. 2013 | Document: JCTVC-N0338 WG11 Doc: m30427 |
| --- | --- |

| Title: | Cross-check report for JCTVC-N0189 : AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-858-845-1795 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution reports the results of a cross-check performed by Sharp on part of JCTVC-N0189 entropy coding compression efficiency for high bit depths. To increase efficiency at higher bit depths, modified residual coding structure were introduced.  All the results were confirmed from the cross-check.

# Description

## Direct Coding of Fixed Bits

The changes to the current HEVC process (see above) are as follows:

- The last significant scanning position in TU unit.

For each sub-block (4x4 block)

- Significant group flag (depends on block type)

- Significant map coding.

- If the flag is 1, then each significant coefficient C is split into a most-significant part CMSB and a least-significant part CLSB, where

CMSB = (abs(C) – 1) >> BF    CLSB = (abs(C) – 1) – (CMSB << BF)

and the following syntax elements pertaining to the group are coded immediately following it:

  - Significance map:
For each coefficient in the group, a flag is coded indicating whether or not the coefficient C is significant (has a non-zero value). No flag is necessary for the coefficient indicated by the last-significant position.

  - Greater-than-one map:
For up to eight coefficients with significance map value 1 (counted backwards from the end of the group), this indicates whether CMSB is greater than or equal to 1.

  - Greater-than-two flag:
For up to one coefficient with greater-than-one map value 1 (the one nearest the end of the group), this indicates whether CMSB is greater than or equal to 2.

  - Sign bits:
For all non-zero coefficients, sign bits are coded as equiprobable CABAC bins, with the last sign bit (in reverse scan order) possibly being instead inferred from parity when sign bit hiding is used.

  - For each non-zero coefficient:

    - Escape codes:
If the magnitude of CMSB was not completely described by an earlier syntax element, the remainder is coded as an escape code.

    - Fixed bits:
If BF is greater than 0, the value of CLSB is coded.

The value of BF would be signalled by the encoder in the slice header (with the sequence parameter set (SPS) indicating that the feature is enabled), since the best value is source dependent and some encoders may wish to adaptively control this function. However, a simple derivation for BF based on QP is as follows:

BF = max (0, int (QP / -6))

The results using this simple derivation for the fixed-bit scheme are shown in Error: Reference source not found.

## Golomb-Rice Parameter Modification

The fixed bits in 1.1 are split from the coefficient and coded separately. This could instead be combined into the existing Golomb-Rice/exponential-Golomb escape coding mechanism. By applying an offset to cRiceParam equal to BF, the fixed bits were extracted from the coefficient magnitude as part of the normal escape coding operation. Automatic Adaptation of BF for Golomb-Rice Parameter Modification has been also considered

# Results

Experiments have been performed as specified in JCTVC-N0188):

Table 1. BD-rate changes using green channel PSNR for OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = max(15, (bitDepth + 6)) and ENTROPY_CODING_DYNAMIC_ RANGE = max(15, (bitDepth + 6))  (RDOQ disabled, transform-skip enabled, lossless coding enabled).

| Bit Depth | Target PSNR | DCFB with auto adaption | G-R parameter modification with auto adaption |
| --- | --- | --- | --- |
| 8 | 48 dB | 1.14% | 0.02% |
| 10 | 60 dB | -0.78% | -0.72% |
| 12 | 72 dB | -8,33% | -8.25% |
| 14 | 84 dB | -20.69% | -20.57% |
| 16 | 96 dB | -24.81% | -24.64% |

# Conclusions

Entropy coding compression methods for high bit depths are cross-checked. Our verification test results confirm the reported coding efficiency gain shown in JCTVC-N0189.