| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0340 |
| --- | --- |

| Title: | Non-RCE3: Cross-check of N0256 (Intra Motion Compensation with 2-D MVs) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghye Min Elena Alshina | Email: | jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results for JCTVC-N0256 (Intra Motion Compensation with 2-D MVs). The source code provided by the proponent was reviewed and it was confirmed that the implementation was consistent with the algorithm described in the contribution document. The coding performance was evaluated by Samsung and the results match those provided by the proponent.

# Introduction

The JCTVC-N0256 proposes an Intra Motion Compensation with 2-D MVs which is consistent with inter motion compensation. As in inter case, MV prediction using neighboring MVs is exploited to predict 2D MVs. Exponential-Golomb of 3rd order codes are used to signal MVDs.  Search area for intra motion estimation is restricted with two restriction conditions, 1) current CTU and the left CTU and 2) current CTU and the rightmost 4 columns of the left CTU to support pipeline friendly features.

Samsung reviewed the source code and confirmed that the implementation was consistent with the algorithm described in the contribution document. Simulations were performed on a linux cluster under the common test condition [1].

# Test results

The cross-verification results of JCTVC-N0256 are summarized in several tables in this section.  Among test cases presented in the JCTVC-N0256, following two test cases were verified in this report.

- Search area is the current CTU and the left CTU, Fast search, MV coding method 1

- Search area is the current CTU and the rightmost 4 columns of the left CTU., Fast search, MV coding method 1

Test results match those provided by the proponent.

Table 1.  Lossless results (1)

Table 2.  Lossy results (1)

Table 3.  Lossless results (2)

Table 4.  Lossy results (2)

# Conclusion

This contribution reports the cross-verification results for JCTVC- N0256 (Intra Motion Compensation with 2-D MVs). It was confirmed that the implementation was consistent with the algorithm description and the test results matched those provided by the proponent.

# References

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, KR, April 2013.