| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0346 |
| --- | --- |

| Title: | Proposed editorial changes for maximum number of slices and tiles per picture limit |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Broadcom Corporation, USA Brian Heng Broadcom Corporation, USA Wade Wan Broadcom Corporation, USA | Email: Email: Email: | minhua@broadcom.com bheng@broadcom.com wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

It is asserted that, based on the calculations specified in the current HEVC version 1 text, the maximum number of slices per picture could be 0 for high levels with small enough pictures. A similar issue exists in the maximum number of tiles per picture limit. The following editorial changes are proposed to fix this issue. (Changes are marked in yellow).

# Proposed editorial changes

A.4.2 Profile-specific level limits for the Main and Main 10 profiles

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

Bitstreams conforming to the Main or Main 10 profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ) for the value of PicSizeInSamplesY of picture n − 1, where MaxLumaSr is the value specified in Error: Reference source not found that applies to picture n − 1.

- The difference between consecutive output times of pictures from the DPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ) for the value of PicSizeInSamplesY of picture n, where MaxLumaSr is the value specified in Error: Reference source not found for picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- The removal time of access unit 0 shall satisfy the constraint that the number of slice segments in picture 0 is less than or equal to Min( Max(1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * PicSizeInSamplesY / MaxLumaPs ), MaxSliceSegmentsPerPicture ), for the value of PicSizeInSamplesY of picture 0, where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Error: Reference source not found and Error: Reference source not found, respectively, that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in picture n is less than or equal to Min(Max(1,  MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Error: Reference source not found and Error: Reference source not found, respectively, that apply to picture n.

- For the VCL HRD parameters, BitRate[ i ] shall be less than or equal to CpbBrVclFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Error: Reference source not found in units of CpbBrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[ i ] shall be less than or equal to CpbBrNalFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Error: Reference source not found in units of CpbBrNalFactor bits/s.

- The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to 1.5 * ( Max( PicSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of PicSizeInSamplesY of picture 0, where MaxLumaSr and MinCr are the values specified in Error: Reference source not found that apply to picture 0.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to 1.5 * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and MinCr are the values specified in Error: Reference source not found that apply to picture n.

- The removal time of access unit 0 shall satisfy the constraint that the number of tiles in picture 0 is less than or equal to Min(Max(1,  MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs ), MaxTileCols * MaxTileRows ), for the value of PicSizeInSamplesY of picture 0, where MaxTileCols and MaxTileRows are the values specified in Error: Reference source not found that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in picture n is less than or equal to Min( Max(1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Error: Reference source not found that apply to picture n.

# References

[1] B. Bross, G. Sullivan and Y.K. Wang “Editors' proposed corrections to HEVC version 1,” JCTVC-N0041_v1, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013

# Patent rights declaration(s)

Broadcom Corporation does not have current or pending patent rights relating to the technology described in this contribution.