| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0348 M30454 |
| --- | --- |

| Title: | Non-RCE3: Cross-check of Intra Motion Compensation in JCTVC-N0206 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Technologies, Inc. |  |  |

_____________________________

# Abstract

This document provides the cross-check results for the proposal related to the intra motion compensation for HEVC Range Extensions proposed by Texas Instruments in JCTVC-N0206. Source codes and test results provided by the proponents are verified in this document. It has been confirmed that source codes are modified on top of HEVC_HM10.1_RExt3.0 as described in JCTVC-N0206 and test results exactly match to those provided by the proponents.

# Introduction

In JCT-VC M0350 [1] the intra motion compensation scheme was proposed for High Efficiency Video Coding (HEVC) Range Extension (RExt) [2]. In JCTVC-N0206 [3] the coding efficiency is improved by use of 3rd order Exp-Golomb binarization for the motion vector coding.

Source codes and test results provided by the proponents are verified in this document. It has been confirmed that the source codes are modified on top of HEVC_HM10.1_RExt3.0 as described in JCTVC-N0206 and test results exactly match to those provided by the proponents.

# Experimental Results

The simulation was performed using various screen contents under the RCE3 common test condition [4]. Both lossless and lossy coding cases were tested. HEVC_HM10.1_RExt3.0 is used as the anchor.

Below are the experimental results. It is confirmed that the test results exactly match to those provided by the proponent.

Table 1. Experimental results for lossless case.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -3.1% | 31.7 | 33.1 | -1.9% | 49.8 | 50.9 | -0.8% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 14.7 | -24.0% | 100.4 | 148.9 | -18.9% | 381.6 | 590.3 | -16.7% |
| SC YUV 444 | 11.4 | 17.6 | -21.6% | 128.9 | 199.4 | -19.6% | 325.6 | 555.6 | -19.2% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 133% |  |  | 112% |  |  | 106% |  |  |
| Dec Time[%] | 98% |  |  | 110% |  |  | 107% |  |  |

Table 2. Experimental results for lossy case.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -9.8% | -9.6% | -9.5% | -8.5% | -8.5% | -8.5% | -7.4% | -7.4% | -7.4% |
| Class B | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -30.9% | -30.8% | -31.2% | -30.3% | -30.3% | -30.4% | -29.7% | -29.6% | -29.6% |
| SC YUV 444 | -28.1% | -27.9% | -28.2% | -27.4% | -27.5% | -27.6% | -26.8% | -26.8% | -26.7% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -19.3% | -19.2% | -19.4% | -18.7% | -18.8% | -18.8% | -18.2% | -18.2% | -18.2% |
|  | -19.3% | -19.2% | -19.4% | -18.7% | -18.8% | -18.8% | -18.2% | -18.1% | -18.1% |
| Enc Time[%] | 134% |  |  | 133% |  |  | 133% |  |  |
| Dec Time[%] | 87% |  |  | 87% |  |  | 87% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -7.1% | -7.5% | -7.3% | -6.3% | -6.7% | -6.6% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -24.9% | -24.7% | -24.9% | -24.1% | -24.0% | -24.1% |  |  |  |
| SC YUV 444 | -23.9% | -24.1% | -24.3% | -23.5% | -23.5% | -23.7% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | -15.8% | -15.9% | -16.0% | -15.3% | -15.3% | -15.4% |  |  |  |
|  | -15.8% | -15.9% | -16.0% | -15.3% | -15.4% | -15.4% |  |  |  |
| Enc Time[%] | 113% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.4% | -5.0% | -4.4% | -4.0% | -4.2% | -4.2% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -20.2% | -19.2% | -19.4% | -19.7% | -18.6% | -18.6% |  |  |  |
| SC YUV 444 | -19.1% | -18.6% | -18.9% | -19.0% | -18.4% | -18.6% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Overall | -12.5% | -12.2% | -12.2% | -12.2% | -11.8% | -11.8% |  |  |  |
|  | -12.5% | -12.2% | -12.2% | -12.2% | -11.8% | -11.8% |  |  |  |
| Enc Time[%] | 112% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 108% |  |  |  |  |  |

# Conclusion

In this contribution the intra motion compensation related proposal for HEVC Range Extensions proposed in JCTVC-N0206 is cross-checked. It is confirmed that the test results exactly match to those provided by the proponent.

# Reference

- M. Budagavi and D.-K. Kwon, “AHG8: Video coding using intra motion compensation,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M0350, Apr. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 3,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M1005, Apr. 2013.

- D.-K. Kwon and M. Budagavi, “Non-RCE3: Intra motion compensation with variable length intra MV coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0206, Jul. 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra coding methods for screen content,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M1123, Apr. 2013.