| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0349 M30455 |
| --- | --- |

| Title: | Cross-check of Mode Dependent Intra Smoothing in JCTVC-N0143 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Technologies, Inc. |  |  |

_____________________________

# Abstract

This document provides the cross-check results for the proposal related to the mode dependent intra smoothing (MDIS) scheme for HEVC Range Extensions proposed by Canon in JCTVC-N0143. Source codes and test results provided by the proponents are verified in this document. It has been confirmed that source codes are modified on top of HEVC_HM10.1_RExt3.0 as described in JCTVC-N0143 and test results exactly match to those provided by the proponents.

# Introduction

In High Efficiency Video Coding (HEVC) Range Extension (RExt) [1], the mode dependent intra smoothing (MDIS) is performed depending on the TU size except for DC/HOR/VER modes. In JCTVC-N0143 [2] it is proposed to apply MDIS for all TU size except for 4×4 except for DC/HOR/VER modes.

Source codes and test results provided by the proponents are verified in this document. It has been confirmed that the source codes are modified on top of HEVC_HM10.1_RExt3.0 as described in JCTVC-N0143 and test results exactly match to those provided by the proponents.

# Experimental Results

The simulation was performed under the common test condition [3]. HEVC_HM10.1_RExt3.0 is used as the anchor. Below are the experimental results. It is confirmed that the test results exactly match to those provided by the proponent.

Table 1. Experimental results.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.5% | -0.3% | -0.3% | -0.4% | -0.2% | -0.3% | -0.3% | -0.1% | -0.2% |
| YCbCr 4:4:4 | -0.2% | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% | -0.3% | 0.0% | -0.1% |
| Overall | -0.4% | -0.2% | -0.3% | -0.4% | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% |
|  | -0.4% | -0.2% | -0.3% | -0.3% | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| Overall | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
|  | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| Enc Time[%] | 104% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |

# Conclusion

In this contribution the mode dependent intra smoothing proposal for HEVC Range Extensions proposed in JCTVC-N0143 is cross-checked. It is confirmed that the test results exactly match to those provided by the proponent.

# Reference

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 3,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M1005, Apr. 2013.

- E. François, C. Gisquet, G. Laroche, and P. Onno, “AHG5: on chroma QP for HEVC RExt,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0143, Jul. 2013.

- D. Flynn and K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1006, Jan. 2013.