| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0352 |
| --- | --- |

| Title: | Cross-check of JCTVC-N0161 on using decoded pictures from higher layer as references in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu, Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification results for JCTVC-N0161. Cross-check simulation confirms that BD-rate results match with those from proponents. Study of software reports that implementation matches the description in the contribution JCTVC-N0161.

# Introduction

JCTVC-N0161 proposed to use higher layer decoded pictures as reference pictures for lower layer pictures. This proposed approach only applies to SNR scalability.  The implementation is based on SHM2.0. The observed changes of software are listed as follows

- A flag for each layer is added to VPS syntax for controlling of proposed backwards cross-layer prediction.

- Modifications to support temporal scalability are defined by macro TEMP_SCALABILITY_FIX.

- Modifications to support reference layer from higher layer decoded pictures are defined by macro JCTVC_N0161.

- Configuration files are also modified to support temporal scalability by changing temporal_id and reference pictures in “coding structure” section, compared to CTC.

# Simulation and results

The code is compiled using MS Visual Studio 2010. The released source code is compiled under macro settings as follows.

#define TEMP_SCALABILITY_FIX             1      ///< software Fix for temporal scalability

#define JCTVC_N0161                      1      ///< enable use of higher layer pictures as reference for reference for lower layers

## Simulation settings

Since configure files are changed, a new anchor is generated with new configuration files with temporal scalability. QP settings and test video sequences are the same as CTC.

## Results

Three results are cross-checked and all results match with results in JCTVC-N0161. The terms are defined as

- Anchor1: SHM2.0 CTC

- Anchor2: SHM2.0 CTC + temporal scalability

- Proposed: proposed method + SHM2.0 CTC+ temporal scalability

### Anchor2 vs Proposed

The results are summarized in Table 1. Detailed results are available in accompanying excel file. Encoding and decoding time is not accurate due to  a heterogeneous cluster.

Table 1 Results of Anchor2 vs Proposed

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -2.8% | -9.2% | -9.4% |
| Class B | -2.4% | -7.8% | -9.3% |
| Overall (Test vs Ref) | -2.5% | -8.2% | -9.4% |
| Overall (Test vs single layer) | 11.8% | 21.1% | 21.3% |
| EL only (Test vs Ref) | -5.7% | -11.6% | -12.9% |
| Enc Time[%] | 97.6% |  |  |
| Dec Time[%] | 96.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  |
| BL Match | Not matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -3.5% | -7.8% | -7.7% |
| Class B | -3.0% | -7.3% | -8.3% |
| Overall (Test vs Ref) | -3.1% | -7.5% | -8.1% |
| Overall (Test vs single layer) | 23.8% | 27.5% | 29.7% |
| EL only (Test vs Ref) | -5.4% | -9.9% | -10.5% |
| Enc Time[%] | 100.1% |  |  |
| Dec Time[%] | 95.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  |
| BL Match | Not matched |  |  |
|  | LD-B HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -3.4% | -7.8% | -7.6% |
| Class B | -3.1% | -6.7% | -7.7% |
| Overall (Test vs Ref) | -3.2% | -7.0% | -7.7% |
| Overall (Test vs single layer) | 26.2% | 29.4% | 31.7% |
| EL only (Test vs Ref) | -5.4% | -9.4% | -10.1% |
| Enc Time[%] | 97.2% |  |  |
| Dec Time[%] | 91.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  |
| BL Match | Not matched |  |  |

### Anchor1 vs Proposed

The results are summarized in Table 2. Detailed results are available in accompanying excel file.

Table 2 Results of Anchor1 vs Proposed

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -2.7% | -9.1% | -9.3% |
| Class B | -2.2% | -7.7% | -9.3% |
| Overall (Test vs Ref) | -2.3% | -8.1% | -9.3% |
| Overall (Test vs single layer) | 11.8% | 21.1% | 21.3% |
| EL only (Test vs Ref) | -5.5% | -11.6% | -12.8% |
| Enc Time[%] | 91.9% |  |  |
| Dec Time[%] | 109.4% |  |  |
| Enc Mem[%] | #VALUE! |  |  |
| BL Match | Not matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -0.3% | -4.4% | -5.0% |
| Class B | 0.3% | -5.1% | -7.2% |
| Overall (Test vs Ref) | 0.2% | -4.9% | -6.5% |
| Overall (Test vs single layer) | 23.8% | 27.5% | 29.7% |
| EL only (Test vs Ref) | -5.1% | -10.2% | -11.9% |
| Enc Time[%] | 76.6% |  |  |
| Dec Time[%] | 92.2% |  |  |
| Enc Mem[%] | #VALUE! |  |  |
| BL Match | Not matched |  |  |
|  | LD-B HEVC SNR |  |  |
|  | Y | U | V |
| Class A | 1.1% | -3.4% | -4.3% |
| Class B | 1.5% | -3.8% | -5.7% |
| Overall (Test vs Ref) | 1.4% | -3.7% | -5.3% |
| Overall (Test vs single layer) | 26.2% | 29.4% | 31.7% |
| EL only (Test vs Ref) | -4.1% | -9.1% | -10.8% |
| Enc Time[%] | 77.1% |  |  |
| Dec Time[%] | 90.3% |  |  |
| Enc Mem[%] | #VALUE! |  |  |
| BL Match | Not matched |  |  |

### Anchor1 vs Anchor2

The results are summarized in Table 3. Detailed results are available in accompanying excel file.

Table 3 Results of Anchor1 vs Anchor2

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | 0.1% | 0.1% |
| Class B | 0.3% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.2% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 14.8% | 32.6% | 34.7% |
| EL only (Test vs Ref) | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 94.2% |  |  |
| Dec Time[%] | 113.1% |  |  |
| Enc Mem[%] | #VALUE! |  |  |
| BL Match | Not matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | 3.3% | 3.8% | 2.9% |
| Class B | 3.5% | 2.3% | 1.0% |
| Overall (Test vs Ref) | 3.4% | 2.7% | 1.6% |
| Overall (Test vs single layer) | 27.9% | 38.3% | 41.6% |
| EL only (Test vs Ref) | 0.3% | -0.4% | -1.6% |
| Enc Time[%] | 76.6% |  |  |
| Dec Time[%] | 96.4% |  |  |
| Enc Mem[%] | #VALUE! |  |  |
| BL Match | Not matched |  |  |
|  | LD-B HEVC SNR |  |  |
|  | Y | U | V |
| Class A | 4.7% | 4.8% | 3.6% |
| Class B | 4.7% | 3.0% | 2.0% |
| Overall (Test vs Ref) | 4.7% | 3.5% | 2.5% |
| Overall (Test vs single layer) | 30.3% | 39.5% | 43.1% |
| EL only (Test vs Ref) | 1.4% | 0.2% | -0.9% |
| Enc Time[%] | 79.4% |  |  |
| Dec Time[%] | 98.3% |  |  |
| Enc Mem[%] | #VALUE! |  |  |
| BL Match | Not matched |  |  |