| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0353 |
| --- | --- |

| Title: | Cross-check for JCTVC-N0222, Non-RCE2: Results for combination of methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research and Development |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for Document JCTVC-N0222 proposed by Qualcomm Inc. All the results released by the proponent are tested and it is reported that all the test points are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

Document JCTVC-N0222 presents a combination of several tools currently tested either in the context of RCE2 or in some Non-RCE2 contributions. In particular the tools tested in this proposal are as follows:

- Extension of transform skip to block size up to 8×8

- Prediction residuals rotation and constant context for significance map coding

- Rice parameter update and initialisation

- Residual DPCM extended to lossy coding for both intra and inter coding

- Replacement of intra DC mode

The tools are tested in combination using the fast transform skip mode proposed by the proponent. The results for the combination of the tested coding tools are presented in the following section.

# Test results

The combination is tested for both lossless and lossy coding using the RCE2 test conditions described in Document JCTVC-M1122. This cross-check document reports the same results in the following tables. Notice that the BD-rates for RGB content presented in this document are ordered differently from the ones released by the proponent. In particular the cross checker generated the anchors with the encoder parameter --SNRInternalColourSpace set to zero while the proponent set it to one. The overall effect of this parameter is a different ordering of the PSNR values according to the used input colour space conversion.

Table 1: Compression efficiency for the tested combinations and lossless coding.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.6 | -5.1% | 31.7 | 34.8 | -6.1% | 49.8 | 55.6 | -7.0% |
| Class B | 2.2 | 2.3 | -2.0% | 2.6 | 2.6 | -0.7% | 2.6 | 2.6 | -0.7% |
| SC RGB 444 | 10.1 | 12.2 | -15.6% | 100.4 | 124.0 | -15.6% | 381.6 | 493.1 | -16.2% |
| SC YUV 444 | 11.4 | 13.4 | -11.5% | 128.9 | 155.0 | -11.9% | 325.6 | 414.9 | -13.1% |
| RangeExt | 2.4 | 2.4 | -4.1% | 2.5 | 2.6 | -3.0% | 2.5 | 2.6 | -3.0% |
| Enc Time[%] | 93% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 94% |  |  | 100% |  |  | 101% |  |  |

Table 2: BD-rate for the tested combinations and lossy encoding.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -5.9% | -6.2% | -6.5% | -6.2% | -6.6% | -6.8% | -6.4% | -6.7% | -6.9% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| SC RGB 444 | -12.6% | -12.8% | -12.7% | -14.8% | -15.0% | -15.0% | -16.7% | -16.8% | -16.9% |
| SC YUV 444 | -8.4% | -8.5% | -8.6% | -9.2% | -9.8% | -9.8% | -10.3% | -10.9% | -10.9% |
| RangeExt | -0.3% | -0.1% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 114% |  |  | 114% |  |  | 114% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.2% | -4.3% | -4.5% | -4.2% | -4.2% | -4.4% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -10.3% | -10.3% | -10.3% | -12.1% | -12.2% | -12.2% |  |  |  |
| SC YUV 444 | -6.5% | -6.8% | -6.7% | -7.1% | -7.7% | -7.6% |  |  |  |
| RangeExt | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 111% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.9% | -3.0% | -2.1% | -2.9% | -3.0% | -2.7% |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% |  |  |  |
| SC RGB 444 | -10.8% | -10.6% | -10.6% | -12.6% | -12.6% | -12.6% |  |  |  |
| SC YUV 444 | -5.5% | -5.3% | -5.5% | -6.0% | -6.2% | -6.3% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.2% | 0.1% | 0.3% | 0.4% |  |  |  |
| Enc Time[%] | 110% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |

# Source code inspection

The proponent implemented the proposed changes in the HM-10.0-rext-3.0 reference software. Two source code versions have been released: one for lossless and one for lossy coding. In the following the source inspection for each version is discussed.

## Source code inspection for lossless coding

To combine all the aforementioned coding tools, the following macros have been defined:

- LAST_COEFF_AVRG

- MAX_RICE_PARAM

- FAST_UPDATE_RICE_PARAM

- RESIDUAL_ROTATION_RESTRICTION

- RESIDUAL_ROTATION_RESTRICTION_MAX_SIZE

- CONSTANT_CTX_SIGMAP

- LOSSLESS_INTRA_DC_REPLACEMENT

- RDPCM_INTER_TU

The first three macros refer to the Rice parameter update. The macros RESIDUAL_ROTATION_RESTRICTION and RESIDUAL_ROTATION_RESTRICTION_MAX_SIZE refer to the rotation of residuals for both intra and inter coded blocks. In particular, the second macro in this category limits the residual rotation to particular block sizes (4×4 in this proposal). The macro CONSTANT_CTX_SIGMAP enables the use of a constant context to encode the significance map. The macro LOSSLESS_INTRA_DC_REPLACEMENT replaces the intra DC prediction mode with a different predictor derived from the top, left and top-left samples. Finally the macro RDPCM_INTER_TU enables the application of residual DPCM to motion compensated residuals along the horizontal or vertical directions. In this proposal, the TU-based implementation for inter RDPCM is used. Source code inspection reveals perfect matching with the tested combinations.

## Source code inspection for lossy coding

To combine all the aforementioned coding tools for lossy coding, the following macros have been defined:

- LAST_COEFF_AVRG

- MAX_RICE_PARAM

- FAST_UPDATE_RICE_PARAM

- RESIDUAL_ROTATION_RESTRICTION

- RESIDUAL_ROTATION_RESTRICTION_MAX_SIZE

- CONSTANT_CTX_SIGMAP

- LOSSLESS_INTRA_DC_REPLACEMENT

- RDPCM_LOSSY

- TR_SKIP_8x8

- TRANSKIP_8X8_FAST

The first seven macros are the same as per lossless coding. The macro RDPCM_LOSSY enables the extension to RDPCM for lossy coding as described in Test D.1 for RCE2. The macro TR_SKIP_8x8 enables the use of transform skip also on 8×8 TUs while TRANSKIP_8X8_FAST allows to speed-up the computation when transform skip is also applied to 8×8 blocks. In particular, when this macro is set to 1, RDOQ is not run over 8×8 block with transform skip flag equal to 1 and transform skip is only tested on the root of RQT (i.e. TransformIdx equal to zero) for PUs with size N×N. Source code inspection reveals perfect matching with the tested combinations.