| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0354 |
| --- | --- |

| Title: | Non-RCE2: Results for combination of mirroring of non-transformed residual blocks (RCE2 Test B.3) with other methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Rajitha Weerakkody, Marta Mrak and Matteo Naccari 56 Wood Lane, London, W12 7SB, UK | Email: | rajitha.weerakkody@bbc.co.uk,marta.mrak@bbc.co.uk matteo.naccari@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This proposal presents the combination of the mirroring of coefficients for non-transformed residuals (RCE2 Test B.3) with a number of other tools proposed in relation to RCE2, as in JCTVC N0222, including: constant context for the significance map, Rice parameter initialization and update, prediction of residual for inter-coded blocks and for lossy coding, DC mode replacement and transform skip 8×8. Mirroring of non-transformed residuals is tested for maximal block size restrictions of 4×4 and 8×8.

# Introduction

This contribution presents the results of combining the method of mirroring the residual for non-transformed coefficients (RCE2 Test B.3) [1] with several tools proposed in relation to the RCE2 [1], as presented in JCTVC-N0222. The specific tools combined in this submission include:

- Mirroring of non-transformed residual blocks

  - For non-transformed blocks of size 4×4 only

  - For non-transformed blocks of size up to 8×8

- Replacement of Intra DC mode (JCTVC-N0053)

- Extension of RDPCM to inter and lossy (JCTVC-N0083)

- Rice Parameter update and initialization (JCTVC-N0222)

  - Setting: Applied to transform and transform skip & max Rice equal to 5 with fast update

- Extension of transform skip to 8×8 (JCTVC-N0288)

  - Setting: Fast encoder method

# Results

The proposed combination is implemented in HM10.1_Rext3.0 and tested under the RCE2 common test conditions [2]. The performance is compared to the anchor for lossless and lossy configurations.

Table 1. Performance for lossy coding: mirroring on 4x4 and 8×8 blocks

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -6.2% | -6.6% | -6.9% | -6.6% | -7.0% | -7.2% | -6.8% | -7.2% | -7.3% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| SC RGB 444 | -13.4% | -13.3% | -13.2% | -15.6% | -15.5% | -15.4% | -17.3% | -17.4% | -17.2% |
| SC YUV 444 | -8.7% | -8.8% | -8.9% | -9.5% | -10.1% | -10.1% | -10.7% | -11.3% | -11.3% |
| RangeExt | -0.3% | -0.1% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 115% |  |  | 115% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.3% | -4.5% | -4.7% | -4.3% | -4.4% | -4.6% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -10.5% | -10.6% | -10.5% | -12.4% | -12.5% | -12.4% |  |  |  |
| SC YUV 444 | -6.7% | -6.9% | -6.9% | -7.2% | -7.7% | -7.7% |  |  |  |
| RangeExt | -0.2% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 111% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.0% | -2.7% | -3.1% | -2.9% | -2.8% | -3.2% |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -10.9% | -11.0% | -11.0% | -12.9% | -12.8% | -12.7% |  |  |  |
| SC YUV 444 | -5.6% | -5.2% | -5.6% | -5.9% | -6.0% | -6.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.4% |  |  |  |
| Enc Time[%] | 109% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |

Table 2. Performance for lossy coding: mirroring only 4×4 blocks

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -6.1% | -6.5% | -6.8% | -6.5% | -6.9% | -7.1% | -6.7% | -7.1% | -7.2% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| SC RGB 444 | -13.3% | -13.2% | -13.0% | -15.5% | -15.4% | -15.2% | -17.3% | -17.3% | -17.1% |
| SC YUV 444 | -8.5% | -8.6% | -8.7% | -9.4% | -9.9% | -9.9% | -10.6% | -11.1% | -11.2% |
| RangeExt | -0.3% | -0.1% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 115% |  |  | 115% |  |  | 114% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.2% | -4.3% | -4.7% | -4.3% | -4.3% | -4.6% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -10.5% | -10.5% | -10.4% | -12.5% | -12.4% | -12.3% |  |  |  |
| SC YUV 444 | -6.8% | -7.0% | -7.0% | -7.4% | -7.9% | -7.8% |  |  |  |
| RangeExt | -0.2% | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 111% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.9% | -2.3% | -2.6% | -2.9% | -2.7% | -2.9% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% |  |  |  |
| SC RGB 444 | -11.1% | -11.1% | -11.1% | -13.0% | -12.8% | -12.8% |  |  |  |
| SC YUV 444 | -5.4% | -5.3% | -5.5% | -6.1% | -6.3% | -6.3% |  |  |  |
| RangeExt | -0.1% | 0.1% | 0.3% | 0.1% | 0.3% | 0.5% |  |  |  |
| Enc Time[%] | 109% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |

# References

- R. Weerakkody and M. Mrak, “Mirroring of Coefficients for Transform Skipping”, JCTVC-K0294, Shanghai, CN, October 2012.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.

# Patent rights declaration

British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).