| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0092 |
| --- | --- |

| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0355 |
| --- | --- |

| Title: | 3D/MV-HEVC HLS: Extending the supported number of layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Gerhard Tech Robert Skupin Thomas Schierl Byeongdoo Choi | Email: | karsten.suehring@hhi.fraunhofer.de gerhard.tech@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de b.d.choi@samsung.com |
| Source: | Fraunhofer HHI, Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes an extension mechanism for layer identifiers to increase the number of supported layers in MV-HEVC and 3D-HEVC. The range of nuh_layer_id is extended by an additional syntax element within the NAL units. The concept of so-called layer clusters allows using the existing extraction processes to select groups of related layers as proposed during the 4th meeting. The syntax has been modified slightly to ensure a backward compatible base layer and to be aligned with MV-HEVC Draft 4.

# Introduction

HEVC allows 64 values of nuh_layer_id to be used for scalable layers, views and depth. If no scalability is used, this implies a constraint of max. 64 views or 32 view plus depth combinations.

For the most common application this is totally sufficient. On the other hand we see huge camera arrays with more the 100 views showing up for future applications like light field imaging [1], [2] or holographic displays [3].

In this contribution we propose an extension mechanism for nuh_layer_id that does not hurt coding performance in the common cases (i.e. which does not change the NAL unit header) but allows increasing the number of views.

In the targeted special applications, accessing the NAL units that belong to a certain camera view (i.e. layer) requires parsing the beginning of the slice header, but a rough pre-selection can still be achieved using the traditional bitstream extraction process based on nuh_layer_id.

# Proposal

A new variable LayerId is used to identify layers. If the layer extension is disabled, LayerId is equal to nuh_layer_id. If the extension is enabled, nuh_layer_id is used as MSB of LayerId, while a new syntax element (layer_id_ext) forms the LSB. The new syntax element is coded inside the NAL units (e.g. parameter set extensions, slice header extension).

When the layer extension is enabled nuh_layer_id can be used to pre-select a set of related layers (a "layer cluster"), which can e.g. exist of neighboring views or a texture and a depth layer related to the same position.

We believe that composing LayerId from these two syntax elements provides sufficient flexibility for special case applications that require a huge number of layers, without hurting the efficiency of the common case with less than 65 layers.

# Syntax and semantics

In this section major syntax and semantic changes are discussed. A complete specification, showing all modifications can be found in document JCT3V-E0092_spec attached to this proposal. The additional changes in JCT3V-E0092_spec are the change of the nuh_layer_id syntax element to LayerId and the renaming of the syntax element layer_id_in_nuh to layer_id_in_nalu in various places.

## VPS extension

Following changes are suggested:

- Change number of bits for dimesion_id_len_minus1 from 3 to 4, to align the number of maximal possible DimensionId values to the new layer range.

- Add a new syntax element layer_id_ext_len syntax element, specifying the length in bits of the layer_id_ext syntax element and the max_layer_id_ext syntax element. .

- Add a new syntax element max_layer_id_ext specifying the maximum number of layers associated with each nuh_layer_id value greater than 0.

- Change layer_id_in_nuh to layer_id_nalu and increase bit length, since it is related to the LayerId value.

- Signal layer_id_in_nalu, dimension id, direct_dependency_flags, max_tid_il_ref_pics_plus1 and direct_dependency_type for the range of LayerId values, instead of range of nuh_layer_id values.

- Layer identifier lists associated to layer sets are not modified, hence they include nuh_layer_id values and not LayerId values. Therefore bitstream extraction can be performed based on nuh_layer_id values. When a nuh_layer_id is an element of an layer identifier list related to an output layer set, all layers associated with the nuh_layer_id are output layers.

Please note that in document JCT3V-E0223 an alternative method for dependency signalling is proposed, that can be utilized instead of the variant proposed here. The alternative method enables an hierarchical indication of dependencies, where nuh_layer_id values are utilized at the first level.

Moreover JCT3V-E0224 proposes an alternative method for LayerId construction. The alternative method allows that multiple nuh_layer_id values are associated with only one LayerId value.

Syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| layer_id_ext_len | u(4) |
| if( layer_id_ext_len > 0 ) |  |
| max_layer_id_ext | u(v) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(4) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 0; i <= VpsMaxNumLayers − 1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag && i > 0 ) [Ed. (JB): syntax is not compatible with SHVC, or use of splitting_flag.] |  |
| layer_id_in_nalu[ i ] | u(v) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= VpsMaxNumLayers − 1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i< VpsMaxNumLayers − 1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= VpsMaxNumLayers − 1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| } |  |

Semantics

layer_id_ext_len specifies the length, in bits, of the layer_id_ext and the max_layer_id_ext syntax element.

max_layer_id_ext specifies the maximum value of the layer_id_ext_syntax element. The length, in bits, of max_layer_id_ext syntax element is equal to layer_id_ext_len. When not present max_layer_id_ext is inferred to be equal to 0.

The variable MaxNumExtLayers is set equal to max_layer_id_ext + 1.

The variable VpsMaxNumLayers specifying the maximum number of layers in the extended layer range is set equal to (  vps_max_layers_minus1  * MaxNumExtLayers + 1).

The function ExtendedLayerId( nuhLayerId, extensionId ,extLen ) is defined as follows:

ExtendedLayerId( nuhLayerId, extId ,extLen ) = ( nuhLayerId <<  extLen ) + extId

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to ( NumScalabilityTypes − splitting_flag ), inclusive, dimBitOffset[ j ] is derived as follows.

(F-1)

When dimension_id_len_minus1[ NumScalabilityTypes − 1] is not present, the following applies:

–	The value of dimension_id_len_minus1[ NumScalabilityTypes − 1 ] is inferred to be equal to 5 + layer_id_ext_len− dimBitOffset[ NumScalabilityTypes − 1 ].

–	The value of dimBitOffset[ NumScalabilityTypes ] is set equal to 6 + layer_id_ext_len.

layer_id_in_nalu[ i ] specifies the value of the LayerId syntax element in VCL NAL units of the i-th layer. For i in a range from 0 to ,VpsMaxNumLayers − 1 inclusive, when not present, the value of layer_id_in_nalu[ i ] is inferred to be equal to i. The length, in bits, of the layer_id_in_nalu[ i ] syntax element is equal to ( 6 + layer_id_ext_len ).

default_one_target_output_layer_flag equal to 1 specifies that only the layers with nuh_layer_id value equal to the highest entry in the layer identifier lists of each of the default output layer sets are target output layers. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers.

output_layer_flag[ i ][ j ] equal to 1 specifies that the layers with nuh_layer_id value equal to the j-th entry in the layer identifier list of the i-th output layer set are target output layers. output_layer_flag[ i ][ j ] equal to 0 specifies that the layers with nuh_layer_id value equal to the j-th entry in the layer identifier list of i-th output layer set are not target output layers.

NOTE 1 – Only when layer_id_ext is greater than 0, multiple layers can have the same nuh_layer_id_value.

## SPS/PPS

Following changes are suggested:

- Signal layer_id_ext using ue(v) to avoid a parsing dependency to the VPS

Syntax

| pps_extension( ) / sps_extension( ){ | Descriptor |
| --- | --- |
| ... |  |
| layer_id_ext | ue(v) |
| … |  |
| } |  |

Semantics

layer_id_ext is used in combination with nuh_layer_id to identify the layer. The value of layer_id_ext shall be less than or equal to max_layer_id_ext in the active VPS.

The variable LayerId is derived as specified in the following:

LayerId = ExtendedLayerId( nuh_layer_id, layer_id_ext, layer_id_ext_len)

### SEI

Following changes are suggested:

- SEI messages can generically be extended by using the payload extension mechanism:

Syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if ( nuh_layer_id > 0 ) |  |
| sei_layer_id_ext | ue(v) |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Semantics

sei_layer_id_ext is used in combination with nuh_layer_id to identify the layer. The value of sei_layer_id_ext shall be less than or equal to max_layer_id_ext in the active VPS.

The variable LayerId is derived as specified in the following:

LayerId = ExtendedLayerId( nuh_layer_id, sei_layer_id_ext , layer_id_ext_len)

## Slice segment header

Following changes are suggested:

- Add an additional syntax element layer_id_ext in the slice header. layer_id_est is only signalled in slice headers that are contained in a NAL unit with nuh_layer_id greater than 0, i.e. enhancement layer NAL units to ensure backward compatibility. Alternatively the syntax element could be moved to the slice header extension.

Syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| if ( nuh_layer_id > 0 ) |  |
| layer_id_ext | u(v) |
| for( i = 0; i < (num_extra_slice_header_bits); i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| … |  |
| } |  |

Semantics

layer_id_ext is used in combination with nuh_layer_id to identify the layer. The syntax element layer_id_ext is coded using layer_id_ext_len bits. When not present layer_id_ext is inferred to be equal to 0. The value of layer_id_ext shall be less than or equal to max_layer_id_ext in the active VPS.

The variable LayerId is derived as specified in the following:

LayerId = ExtendedLayerId( nuh_layer_id, layer_id_ext, layer_id_ext_len)

## Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

Following changes are suggested:

- Since TargetDecLayerIdList includes nuh_layer_id values, but LayerId values are required in the subclause, a list targetDecLayerIdList including LayerId values is constructed from TargetDecLayerIdList.

# Conclusion

An extension mechanism for layer identification to increase the number of supported layers in MV-HEVC and 3D-HEVC has been proposed in this document. The proposed design has in particular following advantages:

- Due to the view clustering and unmodified layer identifier lists, nuh_layer_id values can still be used for sub stream extraction.

- The modifications compared to the current MV-HEVC design are very small. Only two new syntax elements are introduced.

- When the extension mechanism is deactivated, there is practically no processing overhead.

# References

- WILBURN, Bennett, et al. High performance imaging using large camera arrays. ACM Transactions on Graphics, 2005, 24. Jg., Nr. 3, S. 765-776.

- WILBURN, Bennett S., et al. Light field video camera. In: Electronic Imaging 2002. International Society for Optics and Photonics, 2001. S. 29-36.

- HORIMAI, Hideyoshi, et al. Full-color 3D display system with 360 degree horizontal viewing angle. In: Proc. Int. Symposium of 3D and Contents. 2010. S. 7-10.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).